# 3GPP TSG-RAN WG4 Meeting # 112-bis                                                         R4-2416228

# Hefei, China, 14 – 18 October 2024

# Title:	DC_(n)8AA MSD and MSD test point clean-up

# Agenda Item:	5.2.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our evaluation of the MSD requirements for DC_(n)8AA based on the agreed-upon test points of [1]. We also share our views on resolving MSD test points for intra-band contiguous EN-DC that do not comply with the core requirement rules on cell group placement.

# Discussion

The following agreements were approved at the last meeting [1]:

- For DC_(n)8AA, only single-switched uplink operation is supported.

- Interested companies are invited to evaluate the DC_(n)8AA MSD for the following test points – see Table 1.

For this discussion paper, we have labelled each test point (TP) as “TP1 and TP2” with an additional column to the proposed WF Table 1.

Table 1: MSD test points for DC_(n)8AA.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode | Test Point # |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | FFS | FDD | TP1 |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | FFS |  |  |
|  | 8 | 910 | 10 | 25 (RBstart = 25) | 955 | FFS |  | TP2 |
|  | n8 | N/A | 20 | N/A | 940 | FFS |  |  |
| NOTE 2: The transmitter powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. |  |  |  |  |  |  |  |  |

This document presents our MSD evaluation based on power amplifier (PA) measurement results.

## On MCG/SCG placement for combinations with single-switched uplink

We recall in Figure 1 (blue highlighted text), that the core requirements on the MCG/SCG placement for intra-band contiguous EN-DC combinations were restricted to single-switched uplink-only operations. As shown in Figure 2, we reproduce the IMD landscape and MCG/SCG placement that correspond to TP1 of Table 1.

Figure 1: Rel-18 EN-DC core requirements on MCG/SCG placement for intra-band contiguous MSD test points.

Figure 2: MCG/SCG placement for DC_(n)8AA first MSD test point “TP1” of Table 1.

We observe from Figure 2 that the first DC_(n)8AA MSD test point does not comply with the core requirements. This is because the downlink carrier from the cell group, with uplink, is not configured closer to the uplink operating band compared to the downlink carriers from the cell group, without uplink. To the contrary, in this example, the NR downlink cell is configured furthest away from the uplink band. This is also true for the 2nd MSD test point of DC_(n)8AA (TP2).

We observe that several of the other test points previously agreed upon in Table 7.3B.2.1-1 do not comply with this rule. Table 2 shows the example of one DC_(n)71AA test point that has a similar MCG/SCG placement relative to its uplink band, compared to DC_(n)8AA TP1.

Table 2: Example of previously agreed EN-DC test point that does not comply to the MCG/SCG placement rule.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode | Test Point # |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | [21.3] | FDD | TP1 |
|  | N71 | 673.0 | 20 | [5 (RBstart = 0)] | 627.0 | [0] |  |  |
| NOTE 2: The transmitter powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. |  |  |  |  |  |  |  |  |

Observation 1: The agreed-upon 1UL MSD test points for DC_(n)8AA [1] do not comply with the core requirement rule on MCG/SCG placement. Additionally, several other legacy MSD test points also violate this requirement. This includes the 1UL MSD test points for DC_(n)71AA.

To our understanding, the purpose for introducing this rule for EN-DC were:

- To align the intra-band contiguous EN-DC MCG/SCG placement rule with the NR-CA PCC/SCC placement rule.

- To save MSD test points, i.e. time saving and TS complexity savings were key motivations.

This rule ensures that the PCC/MCG MSD/self-desense is already covered by the single band REFSENS requirements. Thus, no MSD analysis is needed for the PCC/MCG cells. The only MSD that may need to be studied are for cases where the SCC/SCG may be affected by a low order IMD product. Until now, such test points were not needed for NR-CA. However, the key concerns for EN-DC have always been to focus on the LTE anchor cell group performance. This may explain why several of these single-switched uplink MSD test points were introduced to capture the LTE MSD, at the expense of not complying with the MCG/SCG placement rule.

As a way forward, three options are considered for intra-band contiguous MSD test points:

- Option 1: Maintain consistency between NR-CA and EN-DC, by retaining the EN-DC MCG/SCG placement rule. This means that several EN-DC MSD test points may need to be cleaned-up.

- Option 2: Maintain consistency between NR-CA and EN-DC, by removing the EN-DC rule on MCG/SCG placement and reflect these changes in the NR-CA TS. This means that new 1UL NR-CA MSD test points may need to be added.

- Option 3: Break the consistency between NR-CA and EN-DC by:

  - Retaining the NR-CA PCC/SCC placement rule.

  - Changing the EN-DC MCG/SCG placement rule so that 1UL test points verify that the LTE SCG MSD is due to NR transmissions.

Option 3 offers the benefit of reducing workload and minimizing the impact on legacy agreed-upon test points. RAN4 may need to review if by removing such rule some test points need to be added.

Proposal 1: Three options may be considered to address EN-DC MSD test points for intra-band contiguous test points that do not comply with the core requirement MCG/SCG placement:

- Option 1: Maintain consistency between NR-CA and EN-DC by retaining the EN-DC MCG/SCG placement rule. RAN4 may need to verify if some 1UL EN-DC test points need to be removed.

- Option 2: Maintain consistency between NR-CA and EN-DC by removing the EN-DC MCG/SCG placement rule, and reflect this change into the NR-CA core requirements. RAN4 may need to verify if new 1UL test points are needed for intra-band contiguous NR-CA.

- Option 3: Break the consistency between NR-CA and EN-DC by:

  - Retaining the current NR-CA PCC/SCC placement rule.

  - Changing the EN-DC MCG/SCG placement rule to verify only the LTE SCG MSD due to 1UL transmission in the NR band, by adopting the following text:

“For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements are verified withthe downlink carrier(s) from the cell group without uplink  configured closer to the uplink operating band than any of the downlink carriers from the cell group with uplink.”

  - Retaining all 1UL test points that specify the LTE SCG MSD (see for example Table 2 test point for DC_(n)71AA). The NR band MSD shall be specified as “N/A” rather than “0” since the NR performance is covered by the band’s REFSENS requirements.

  - Removing all 1UL intra-band EN-DC MSD test points which specify the NR band SCG MSD.

We propose to adopt Option 3. An example of the resulting clean-up by adopting Option 3 is provided in Table 3.

Table 3: Example EN-DC MSD test point clean-up based on Proposal 1, Option 3.

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | [10 (RBstart = 150)] | 1865.0 | N/A |  |
| DC_(n)5AA | 5 | 826.5 | 5 | N/A | 871.5 | 5.2 | FDD |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | N/A |  |
| DC_(n)5AA | 5 | 829 | 10 | N/A | 874 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| DC_(n)5AA | 5 | 831.5 | 5 | N/A | 876.5 | 5.2 |  |
|  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| DC_(n)5AA | 5 | 834 | 10 | N/A | 879 | 1.5 |  |
|  | n5 | 844 | 10 | 25 (RBend = 51) | 889 | N/A |  |
| DC_(n)12AA | 12 | 703.5 | 5 | N/A | 733.5 | 4.5 | FDD |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | N/A |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 | FDD |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | [21.3] | FDD |
|  | n71 | 673.0 | 20 | [5 (RBstart = 0)] | 627.0 | N/A |  |
| DC_(n)71AA3 | 71 | N/A | 15 | N/A | 639.5 | [5.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0)] | 624.5 | N/A |  |
| DC_(n)71AA4 | 71 | 680.5 | 5 | N/A | 639.5 | [6.8] |  |
|  | n71 | 670.5 | 151 | [5 (RBstart = 2)] | 627.0 | N/A |  |
| DC_(n)71AA5 | 71 | 680.5 | 5 | N/A | 634.5 | [6.4] |  |
|  | n71 | 670.5 | 15 | [5 (RBstart = 0) | 624.5 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

## Power Amplifier Calibration

- Local Oscillator (LO) leakage: -28 dBc, IQ Image rejection: -28 dB.

- C-IM3: -60 dBc, C-IM5: -70 dBc.

- Power Class 3 (PC3) operation and 4 dB post PA insertion losses.

- PA calibration point is 20 MHz, 15 kHz, QPSK, DFT-S-OFDM, 100 RB at lower channel edge with 1 dB MPR- PC3 ACLR -30 dBc.

## Measurement results and MSD proposal

Based on Proposal 1, we recommend evaluating only the Band 8 MSD using TP1 and the following parameters:

- Band 8/n8 Tx-Rx isolation: 50 dB, Tx-to-Antenna rejection: 45 dB.

- Post PA loss: 4 dB.

- Primary antenna to diversity antenna isolation: 10 dB.

- MRC combining and -1 dB Signal-to-Noise Ratio (SNR).

- Power Class 3 operation.

- The Band 8/n8 PA emissions are measured at 23 dBm.

Figure 3 shows the measured 1UL NR PA emissions affecting the LTE band 8 SCG for UL configurations of TP1.

Figure 3: Measured Band n8 20 MHz CBW LCRB=20(RBSTART=86) PA emissions affecting LTE B8.

The measured noise levels and corresponding MSD are captured in Table 4.

Table 4: Measured noise interference levels and B8 1UL MSD for DC_(n)8AA.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | PA noise (dBm/MBW) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | -20.5 | 19 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | -41.9 | N/A |  |

Proposal 2: Based on Proposal 1, adopt the following 1UL MSD requirements for B8 in DC_(n)8AA.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | 19 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | N/A |  |

# Conclusion

In this contribution, we recognize that the agreed upon MSD test points in [1] do not comply with the REFSENS core requirements of EN-DC intra-band contiguous band combinations that are restricted to single-switched uplink operation. Additionally, several other legacy 1UL MSD test points do not comply with these requirements.

Observation 1: The agreed-upon 1UL MSD test points for DC_(n)8AA [1] do not comply with the core requirement rule on MCG/SCG placement. Several other legacy MSD test points also violate this requirement. This includes the 1UL MSD test points for DC_(n)71AA.

We submit Proposal 1 to address the issues captured in Observation 1. An example of MSD test point clean-up is provided in Table 3 above.

Proposal 1: Three options may be considered to address EN-DC MSD test points for intra-band contiguous test points that do not comply with the core requirement MCG/SCG placement:

- Option 1: Maintain consistency between NR-CA and EN-DC by retaining the EN-DC MCG/SCG placement rule. RAN4 may need to verify if some 1UL EN-DC test points need to be removed.

- Option 2: Maintain consistency between NR-CA and EN-DC by removing the EN-DC MCG/SCG placement rule, and reflect this change into the NR-CA core requirements. RAN4 may need to verify if new 1UL test points are needed for intra-band contiguous NR-CA.

- Option 3: Break the consistency between NR-CA and EN-DC by:

  - Retaining the current NR-CA PCC/SCC placement rule.

  - Changing the EN-DC MCG/SCG placement rule to verify only the LTE SCG MSD is due to 1UL transmission in the NR band by adopting the following text:

“For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements are verified withthe downlink carrier(s) from the cell group without uplink  configured closer to the uplink operating band than any of the downlink carriers from the cell group with uplink.”

  - Retaining all 1UL test points that specify the LTE SCG MSD (see for example Table 2 test point for DC_(n)71AA). The NR band MSD shall be specified as “N/A” rather than “0” since the NR performance is covered by the band’s REFSENS requirements.

  - Removing all 1UL intra-band EN-DC MSD test points which specify the NR band SCG MSD.

We propose to adopt Option 3. An example of the resulting clean-up by adopting Option 3 is provided in Table 3.

Based on Proposal 1, we recommend capturing only the LTE anchor point MSD, using the requirements captured in Proposal 2.

Proposal 2: Based on Proposal 1, adopt the following 1UL MSD requirements for B8 in DC_(n)8AA.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | 19 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | N/A |  |

# References

[1]	R4-2414349 WF on DC_(n)8AA, 3GPP TSG-RAN WG4 Meeting # 112, Skyworks, Murata, China Unicom, CHTTL.