3GPP TSG-RAN WG4 Meeting#112bis                             R4-2416238

Hefei, China 14th – 18th Oct, 2024

Agenda item:	6.11.3

Source: 	ZTE Corporation, Sanechips

Title: 	Further discussion on OTA test enhancement

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we have some initial discussions on the necessity of improvement of co-location issues. In this contribution, we would like to share further views on the OTA co-location requirements.

Agreement reached at RAN4#110bis meeting: - Issue 3-1: Co-location reference antenna - Agreement: - Step 1: further investigate the improvement of the co-location reference antenna if necessary; - Step 2: If this is agreeable, then further discuss e.g. the relevant scenarios, applicable antenna types or define requirements on OTA parameters such as field-strength or power density Agreement reached at RAN4#111 meeting: Issue 3-1: Co-location reference antenna - Recommended for further discussion - Further discuss the wideband antennas and check its possibility of translation between reference co-location antenna and wideband antenna; - Further discuss co-location scenario and coupling loss between two AAS BS especially for FR1 high bands; - Further discuss whether the side-by-side deployment is typical scenario for co-location deployment, e.g., in SBFD deployment TX and RX panels could not be in the same height - Other issues are not precluded. Agreement: For bands lower than band [n41/3.5GHz], to reuse the existing co-location requirement e.g. co-location reference antenna definition; ü How to provide the alternative solution for the colocation testing antenna e.g. wide band horn antenna is FFS For bands above band [n41/3.5GHz], ü How to provide the alternative solution for the colocation testing antenna e.g. wide band horn antenna is FFS For FR1 high bands, companies are encouraged to identify the difference with the following two options: 1) AAS BS with colocation reference antenna; 2) AAS BS vs AAS BS;

Discussion

As we discussed in the last RAN4 meeting, the following issues could be further discussed to identify the necessity for co-location reference antenna improvement.

  - .Coupling loss between two AAS BS in FR1 high bands

First of all, NR BS coexisting with non-AAS BS in FR1 high bands are not very common. Instead, two AAS BS coexisting with each other are more ubiquitous which are one candidate issue captured in the agreed WF [1]. Based on the above consideration from the practical deployment perspective, we conducted some coupling loss measurement shown as following:

- (left figure) Two AAS BS are separated in the vertical domain (10cm separation as worst assumption).

- (Right figure) Two AAS BS are separated in the horizontal domain (10cm separation as worst assumption).

Figure 2.1-1. coupling loss measurement for FR1 high bands with separation distance as 10cm from edge to edge

Figure 2.1-2a. antenna ports for coupling loss measurement with vertical separation [expected worst coupling loss]

Figure 2.1-2b. antenna ports for coupling loss measurement with horizontal separation [expected worst coupling loss]

According to the above testing conditions/assumptions, some initial measurement results for FR1 high bands are provided in the following Table 2.1-1. As shown in the following table, if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB under the vertical isolation (closet antenna elements as shown in Figure 2.1-2a). However if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc (closet antenna elements as shown in Figure 2.1-2b). It also depends on the BS’s implementation, for different BS RF architecture, coupling loss among different antenna ports are varying from 31dB to 46dBc.

Table 2.1-1. summary of measured coupling loss different frequency ranges for FR1 WA BS

| Antenna type | 2.6G-BS1 | 2.6G-BS2 | 3.5G-BS1 | 3.5G-BS2 | 4.9G-BS1 |
| --- | --- | --- | --- | --- | --- |
| Isolation in vertical domain(dB) | <-54 | <-57 | <-52 | <-53 | <-56 |
| Isolation in horizontal domain | <-31 | <-40 | <-34 | <-35 | <-46 |

Observation 1:if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB with10cm separation distance.

Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc.

  - .The typical co-location deployment

During the last RAN4 meeting, there were some discussion whether the side by side is still typical scenario for co-location deployment especially considering that Tx and Rx of SBFD BS might be not at the same height.

First of all, the side by side co-location deployment should be still typical for the roof based installation even though Tx and Rx of SBFD BS might be somehow separated deployed to ensure its Tx/Rx isolation as shown in Figure 2.2-1. The minimum horizontal separation distance among DUT and CLTA as 10cm from edge to edge in the specification is still the worst assumption to ensure the largest interfering signal coming into the DUT. Indeed, according to the AAU/RRU installation guidance, the minimum horizontal separation distance should be 0.6m and minimum vertical separation distance should be 0.2m which are both larger than the specified 0.1m separation distance. In other words, the co-location requirement itself in the specification is still very stringent requirement compared with the practical deployment.

Figure 2.2-1. antenna installed on top of the roof

In addition, as shown in Figure 2.2-2 and Figure 2.2-3, there are two other co-location deployment. The 1st one is different operators sharing the same mast and different operators as shown in Figure 2.2-2 utilize the different platform or heights to generate the 3 sector or more sectors to provide the coverage. The vertical separation distance would vary from 0.35m to 0.6m which is highly dependent on the installation restrictions from the mast.

Figure 2.2-2. antenna installed on the mast

The 2nd one as shown in Figure 2.2-3 is the different operators sharing the same BS tower and different operators utilize the different platform to generate the 3 sector or more sectors to provide the coverage as shown in Figure 2.2-3, the vertical separation distance could be in meters level which should be most relaxed co-location requirement compared with the previous two deployments.

Figure 2.2-3. antenna installed on top of the BS tower

Observation 3: the side by side co-location deployment should be still typical for the roof based installation

Observation 4: according to the AAU/RRU installation guidance, the minimum horizontal separation distance should be 0.6m and minimum vertical separation distance should be 0.2m which are both larger than the specified 0.1m separation distance. The co-location requirement itself in the specification is very stringent requirement compared with the practical deployment.

Observation 5: it might be very difficult to have Tx and Rx installed in the different heights of BS mast or tower according to the legacy deployment practice;

  - .Difference between AAS BS vs AAS BS and AAS vs co-location reference antenna

In the previous discussions, one of the critical issue is to identify the difference between AAS vs AAS BS and AAS vs co-location reference antenna. In the following section,we would like to provide the further analysis from both transmitter and receiver requirements perspective.

| Clause number | Requirement | Co-location reference antenna operation | Type |
| --- | --- | --- | --- |
| 6.5 | OTA transmit ON/OFF power for FR1 | Measure emission | Mandatory |
| 6.7.5.3 6.7.5.5 | OTA spurious emission: Protection of the BS receiver of own or different BS | Measure emission | Optional based on declaration |
|  | OTA spurious emission: Co-location with other base stations |  |  |
| 6.8 | OTA transmitter intermodulation | Inject the interferer signal | Mandatory |
| 7.6.3 | OTA out-of-band blocking: Co-location with other base stations | Inject the interferer signal | Optional based on declaration |

Figure 2.3-1. the legacy co-location reference antenna for OFF power and spurious emission requirement

For the legacy co-location reference antenna, received power at co-location reference antenna output should be Ptx+G_Tx-Coupling_loss+G_Rx_CLTA.

Figure 2.3-2. the proposed new co-location reference antenna for OFF power and spurious emission requirement

For the proposed new co-location reference antenna for FR1 high bands, received power at co-location reference antenna output should be Ptx+G_Tx-Coupling_loss+G_Rx_sub-array.

The main difference for OFF power and spurious emission measurement would be G_Rx_CLTA and G_Rx_sub-array pattern. These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Observation 6: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Figure 2.3-3 the proposed new co-location reference antenna for Tx intermodulation and co-location OOBB requirement

For the legacy co-location reference antenna, received power at sub-array of AAS BS should be Ptx+G_Tx_CLTA -Coupling_loss+G_Rx_sub_array

Figure 2.3-4. the legacy co-location reference antenna for Tx intermodulation and co-location OOBB requirement

For the proposed new co-location reference antenna for FR1 high bands, received power at sub-array of AAS BS should be Ptx+G_Tx_AAS -Coupling_loss+G_Rx_sub_array.

The main difference for Tx intermodulation and co-location reference antenna would be G_Tx_CLTA and G_Tx_AAS antenna pattern. These two passive antenna and AAS BS pattern would be quite different. G_Tx_CLTA should provide much more stringent requirement for it since it provides the much wider beam in azimuth plane towards the victim AAS BS with narrow beam. In other words, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

Observation 7: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

  - .Wideband antennas and potential translation between reference co-location antenna and wideband antenna

Potential translation between reference co-location antenna and wideband antenna would be discussed later on considering the practical wideband antenna.

Conclusions

In this contribution, we shared some further views on co-location reference antenna concept in RAN4 and proposals are made as following:

Observation 1:if two NR AAS BS are placed with vertical separation which is quite typical for the mast installation, then the coupling loss would be larger than 50dB with10cm separation distance.

Observation 2: if two NR BS are placed with horizontal separation which is typical for roof based installation, the coupling loss is still larger than 30dBc and the coupling loss among different antenna ports are varying from 31dB to 46dBc.

Observation 3: the side by side co-location deployment should be still typical for the roof based installation

Observation 4: according to the AAU/RRU installation guidance, the minimum horizontal separation distance should be 0.6m and minimum vertical separation distance should be 0.2m which are both larger than the specified 0.1m separation distance. The co-location requirement itself in the specification is very stringent requirement compared with the practical deployment.

Observation 5: it might be very difficult to have Tx and Rx installed in the different heights of BS mast or tower according to the legacy deployment practice;

Observation 6: These two received gain for passive antenna of CLTA and AAS sub-array would be quite different. In other words, the measured spurious emission or transmitter OFF power from CLTA cannot be reused or applicable for AAS BS co-locating with AAS BS at sub-array level.

Observation 7: These two passive antenna and AAS BS pattern would be quite different, from Tx intermodulation and co-location OOBB requirement, it seems that the legacy CLTA requirement would provide more stringent conformance testing for it.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- R4-2319398, Background and technical challenges with BS RF co-location requirements in relation to development of NCR RF co-location requirements, Ericsson, Noted

- R4-1703389, “On BS to BS co-location/co-siting background”, Ericsson

- R4-1706766,Co-location isolation between different frequency bands, Ericsson, Noted

- R4-2406088,Way Forward for [110bis][311] NR_BS_RF, ZTE, approved.

- R4-2404259,OTA test enhancements - CLTA, Nokia

- R4-2409955, Way forward for [111][311] NR_BS_RF, Approved.