3GPP TSG-RAN WG4 Meeting#112bis                              R4-2416246

Hefei, China, 14th – 18th Oct, 2024

Agenda item:	6.8.2.2.1

Source: 	ZTE Corporation

Title: 	Discussion on Tx requirements for NTN UE

Document for:	Discussion

Introduction

In RAN#103 meeting, the work item [RP-240857] on Enhanced requirements and test methodology for NR and IoT NTN was approved as one of Rel-19 RAN4 package.The NTN HPUE is one of objective for Rel-19 NTN evolution. In this contribution, we want to share some initial views on Tx RF requirements for Rel-19 NTN HPUE.

High power UE (HPUE) for NTN

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes

  - Evaluate the impact on the uplink performance of TN (Terrestrial network)

    - Example bands include bands n256 and 256, n255 and 255

    - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS

    - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

Discussion on NTN HPUE Tx requirements

Table 2.1 UE RF requirements for Rel-19 NTN HPUE requirements

| NR-NTN UE Tx requirement | Proposals for Rel-19 NTN HPUE requirements |
| --- | --- |
| 6.2.1 UE maximum output power | No specification impact. |
| 6.2.2 MPR | MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation. In addition, regarding the SAR requirement for FDD NTN HPUE, we could follow the legacy approach to leave it up to p-MPR design instead of defining any uplink duty cycle to guarantee the SAR limits. |
| 6.2.3 A-MPR | - Regarding the A-MPR requirements, once HPUE e.g. PC2/PC1.5or PC1 are supported in any specific bands, then the A-MPR requirement should be revisited again to identify the appropriate the A-MPR requirements. |
| 6.2.4 Configured transmitted power | No specification impact. |
| 6.3.1 Minimum output power | No specification impact. |
| 6.3.2 Transmit OFF power | No specification impact. |
| 6.3.3 Transmit ON/OFF time mask | No specification impact. |
| 6.3.4 Power control | No specification impact. |
| 6.4.1 Frequency error | No specification impact. |
| 6.4.2 Transmit modulation quality | No specification impact. |
| 6.5.1 Occupied bandwidth | No specification impact. |
| 6.5.2.2 Spectrum emission mask | This depends on the outcome of coexistence study, we propose to reuse the TN UE SEM requirement except for IoT NTN HPUE with multiple tone case. |
| 6.5.2.3 Additional Spectrum emission mask | No specification impact. |
| 6.5.2.4.1 NR ACLR | This depends on the outcome of coexistence study, we propose to reuse the TN UE ACLR requirement except for IoT NTN HPUE with multiple tone case. |
| 6.5.3.1 General spurious emissions | No specification impact |
| 6.5.3.2 Spurious emissions for UE co-existence | No specification impact. |
| 6.5.3.3 Additional spurious emissions | Specification impact. |
| 6.5.4 Transmit intermodulation | No specification impact |

Proposal 1: To use the proposals in Table 2.1 for Tx requirements of Rel-19 NTN HPUE

Conclusions

In this contribution, we want to share some initial understandings on Rel-19 NTN HPUE Tx requirements and proposals are made in Table 2.1

References

- R4-2217473 WF on co-existence assumption for IoT over NTN, MTK, approved.

- RP-240857,New WID: Enhanced requirements and test methodology for NR and IoT NTN

- TR 38.863

- R4-2414275	WF on UE RF requirements for NTN HPUE, Samsung, Approved.