# 3GPP TSG-RAN WG4 Meeting # 112-bis                                                         R4-2416253

# Hefei, China, 14 – 18 October 2024

# Title:	Valid Uplink-CA Configurations

# Agenda Item:	6.1.1.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document captures the results of comparing the valid inter-band NR Carrier Aggregation (CA) Uplink (UL) configurations specified in the Bandwidth Combinations Set (BCS) tables against the specified Power Class (PC) captured in the numerous tables of clause 6. Several corrections are suggested for the next meeting. We also present our views on changes, with the objective of preventing such errors from propagating through Release 19.

# Discussion

The scope of the two, three, four, five and six band NR-CA configurations captured in the BCS tables of clause 5 has evolved throughout the evolution of the NR specifications from early Rel-15 to Rel-18:

- Rel-15 contains only one “two-band BCS” table that captures the valid channel bandwidth (CBW) configurations of each downlink (DL) component carrier (CC) / DL band, and when UL CA is supported, the valid UL CA configurations. The column entitled “Uplink CA configuration” contains two types of entries:

  - Entries with “-“ indicate that no UL CA is valid/supported, meaning that any DL constituent band can be configured as a valid single UL carrier band for operation at the default PC3.

  - Entries with “CA_nx-ny” indicate that the two-band UL CA in Band “x” and Band “y” is a valid PC3 UL CA configuration, for example CA_n3A-n78A.

Table 1: First two rows of the Rel-15 two-band BCS table 5.5A.3.1-1.

- Rel-16 captures additional inter-band BCS tables to specify two-band, three-band and four-band DL-CA configurations. The valid UL CA configurations contain the same information as in Release 15, i.e. either single UL carrier, or two-band UL CA and its valid CBW per CC/band. Table 2 illustrates the first two rows of the two-band BCS table.

Table 2: First two rows of the Rel-16 two-band BCS table 5.5A.3.1-1.

- In Rel-17, an additional BCS table is added to capture the valid five-band DL CA configurations, i.e. a total five BCS tables. The column “Uplink CA configuration” is changed to “Uplink CA configuration, or single uplink carrier” with three new footnotes, creating a total of three types of valid uplink configurations:

  - Entries with “-“ remain unchanged from Rel-15, i.e. any constituent DL band can be configured as a valid single UL carrier for operation at the default PC3.

  - Two types of single band uplink carrier entries, where:

    - Note 10 indicates that such entries are meant to capture only single band configurations with power classes other than PC3

    - Note 8 indicates that PC2 is supported. For example, Table 3 shows that band n78 PC2 is a valid UL configuration for DL CA_n1A-n78A,

    - Note 9 indicates that PC1.5 is supported.

Table 3: Example of Rel-17 two-band CA configurations from BCS table 5.5A.3.1-1.

  - Two types of UL CA entries, where the two-band BCS tables contain:

    - An entry without a footnote. This means that this UL CA configuration has PC3 suppor.

    - An entry with footnote 8 indicates that PC2 is supported. For example, Table 3 shows that UL CA_n1A-n78 PC2 is a valid UL configuration for DL CA_n1A-n78A.

- In Rel-18, an additional BCS table is added to capture the valid six-band DL CA configurations. New footnotes are added to specify the supported UL power classes and number of Tx per power classes per band or UL CA band combination, with three types of UL configuration entries This includes multiple possible PC implementations:

  - Entries with “-“ remain unchanged from Rel-15, i.e. any constituent DL band can be configured as a valid single UL carrier for operation at the default PC3.

  - Two types of valid single UL carrier configurations, with footnotes now capturing the number of Tx:

    - note 8 indicates PC2 support with up to 2Tx.

    - note 9 indicates PC1.5 support with up to 2Tx, even though PC1.5 can only be supported with 2Tx.

    - Table 4 shows the example of DL CA_n2A-n77A, where single UL band n77 operation is a valid configuration for operation at PC3, PC2, and PC1.5.

  - Four types of UL CA entries, wherein the two-band BCS tables include:

    - Entry without footnote, to indicate PC3 support.

    - Entry with Note 8, to indicate PC2 support with 1Tx in each band, i.e. 2Tx in total.

    - Entry with Note 13, to indicate PC2 support with 1Tx in one band, and 2Tx in the other, i.e. 3Tx in total

    - Entry with Note 14, to indicate PC1.5 support with 1Tx in one band, and 2Tx in the other, i.e. 3Tx in total.

    - Table 4 shows the example of DL CA_n2A-n77A, where UL CA_n2A-n77A is a valid configuration for operation at PC3, PC2, and PC1.5.

    - In the three, four, five band BCS tables, the footnotes to indicate PC2 or PC1.5 do not capture the number of Tx per band.

Table 4: Example of Rel-18 two-band CA configurations from BCS Table 5.5A.3.1-1a to Table 5.5A.3.1-1e

- In Rel-19, we anticipate the need to capture new methods to support a UL PC:

  - For single carrier operation, Rel-19 introduces a new 3Tx, which may be requested as a valid UL configuration for some DL CA band combinations. Currently, single carrier 4Tx for FWA remains only a single UL / single DL operation. However, in the future, it is possible that 4Tx becomes a valid 1UL configuration to support some DL CA deployments.

  - For two-band UL CA operations, it is expected that PC2 FDD may be requested in an FDD+TDD or FDD+FFD UL CA combination. Conversely, it is likely that new footnotes or amendments to the current footnotes will be needed since in Rel-18 FDD bands are restricted to PC3 for two-band UL CA.

  - Table 5 shows an excel file that captures the new Rel-19 requests. The table includes  the PC per band and the number of Tx per band in a clear manner.

Table 5: Example of Rel-19 two-band CA configuration requests.

Observation 1: The NR CA configurations and BCS defined for two, three, four, five, and six band inter-band CA combinations have considerably expanded their scope since Rel-15. The main role of these bands is to specify the supported CBW of the DL constituent bands.

In Rel-18 multiple dimensions have been added:

- For valid single uplink carrier and for UL CA configurations, HPUE support is captured using footnotes. This is reflected in the two-band BCS table, and also captures the number of Tx. However, the number of Tx is not captured in the three, four, and five band BCS tables. Footnotes include valuable supplemental information.

In Rel-19, it is expected that further complexity may be added to the current footnote approach. This accounts for new UL CA power classes enabled by PC2 in FDD bands, and also accounts for new single carrier operations with more than 2Tx.

## Parsing the valid NR UL-CA configurations

This section summarizes key findings of parsing only the valid inter-band UL CA configurations of the latest TS 38.101-1 Rel-18 version. Configurations for single UL carrier and for intra-band UL CA are outside the scope of this evaluation.

### Valid UL CA configurations not reflected in the UL CA power class tables.

The following observations result from comparing the entries in the two-band BCS Table 5.5A.3.1-1a to Table 5.5A.3.1-1e, with power class requirements captured in the tables of clause 6.

- UL CA_n25A-n41C is captured without footnote in the BCS table, meaning that PC3 is supported, but:

    - There is no entry for this combination in the power class Table 6.2A.1.3-1. PC3 needs to be added.

    - A PC2 triple-beat MSD test point is specified in Table 7.3A.5-1a.

    - Proponents are invited to clarify the intended PC support for all applicable DL CA configurations.

  - UL CA_n40A-n41C is captured without footnote in the BCS table, meaning that PC3 is supported, but:

    - There is no entry for this combination in the power class Table 6.2A.1.3-1. PC3 needs to be added.

    - The band n40 MSD resulting from dual UL CA_n40A-n41C needs to be further studied.

    - Proponents are invited to clarify the intended PC support for all applicable DL CA configurations.

  - UL CA_n41C-n66A, CA_n41C-n71A are specified without footnotes in the BCS table, meaning that PC3 is supported, but:

    - There is no entry for these combinations in the power class Table 6.2A.1.3-1. PC3 needs to be added.

  - UL CA_n41C-n77A is captured without footnote in the BCS table, meaning that PC3 is supported, but:

    - There is no entry for this combination in the power class Table 6.2A.1.3-1, and,

    - Two MSD test points are specified for band n77 due to dual UL band n41 IMD interference, one for PC3 and one for PC2, implying that this UL CA combination supports HPUE.

    - PC3 and PC2 should be specified for UL CA_n41C-n77A.

  - UL CA_n41A-n85A is captured with footnote 8 in the BCS table, meaning that PC2 with 1Tx in each band is supported, but:

    - Only PC3 is specified in the power class Table 6.2A.1.3-1. PC2 needs to be specified. No impact on MSD is expected for this two band UL CA combo.

### UL CA power classes not reflected in the BCS tables.

- UL CA_n41A-n71A is specified for PC3, PC2 and PC1.5 with Band n71 restricted to PC3 operation, and,

  - PC1.5 and PC2 are achieved using Band n41 PC2 with UL-MIMO (Table 6.2H.3.1-1).

  - PC1.5 and PC2 are achieved using Band n41 PC2 with TxD (Table 6.2L.3.1-1).

  - PC2 is achieved with PC3 or PC2 in Band n41 (Table 6.2A.1.3-1).

However, this valid UL CA configuration is captured only with footnote 8 in the two-band BCS tables, meaning that only PC2 with 1Tx in each band is supported.

Footnotes 13, and 14 should be attached to the two-band BCS tables and propagated to the applicable higher order DL CA band combinations. Proponents are invited to further evaluate the DL CA combination the HPUE operation with UL-MIMO and/or TxD applies.

### Additional Corrections.

- UL CA_n41A-n77A PC2 is specified with footnote 2 in Table 6.2L.3.1-1. However, note 2 is specific to an FDD-TDD band combination. We propose to introduce a new footnote 6 “The UE supports PC2 with TxD in either one of the TDD bands and PC2 in the other TDD band.”

- In Table 6.2A.1.3-1:

  - UL CA_n34A-n79A has a double entry. We believe RAN4 should keep only the entry which specified support for PC2 and PC3.

  - “n” is missing for Band n25 in entry for UL CA_n25A-n48A

- In the two-band BCS tables, the uplink configurations for DL CA_n25(2A)-n38A are erroneous.

Observation 2: With the current footnote approach, it is quite easy to introduce errors in the TS. We observe two categories of errors:

- HPUE captured using footnotes in the BCS tables, but not reflected in the clause 6 power class tables.

- HPUE specified in the clause 6 power class tables, but not reflected in the two, three, four, and five band BCS tables.

Considering the number of errors remaining at a late stage of the Rel-18 development, and considering the impact of the new power classes introduced in Rel-19, RAN4 may need to reconsider if the footnote approach is a future proof method for testing.

Proposal 1: We invite proponents of the hereabove flagged UL CA band combinations to consider the intended power class requirement. Proponents should also clarify to which higher order DL CA band combination the missing PC requirements apply.

Proposal 2: Based on the clarifications of proposal 1, submit a CR for the meeting #113 to:

- Introduce in Table 6.2A.1.3-1 the missing UL CA power class requirements for UL CA_n25A-n41C, CA_n40A-n41C, CA_n41C-n66A, CA_n41C-n71A, CA_n41C-n77A, CA_n41A-n85A.

- Move the triple-beat MSD test point for CA_n25A-n41C from the PC2 table to the PC3 table.

- [FFS] on MSD test point for CA_n40A-n41C, depending on the outcome of RAN4 discussions.

- Introduce missing footnotes 13 and 14 to the two-band BCS Tables 5.5A.3.1-1a to Table 5.5A.3.1-1e, and propagate to the higher-order three, four, and five band BCS tables.

- Introduce new footnote 6 in Table 6.2L.3.1-1 “Note 6: The UE supports PC2 with TxD in either one of the TDD bands and PC2 in the other TDD band.”

- In Table 6.2A.1.3-1:

  - Remove entry where only PC3 is specified for UL CA_n34A-n79A.

  - Change entry for CA_25A-n48A to “CA_n25A-n48A”

- In two-band BCS Tables 5.5A.3.1-1a to Table 5.5A.3.1-1e, correct the UL configurations for DL CA_n25(2A)-n38A.

## Alternative solutions to capture valid UL-CA configurations

The complexity of the Rel-18 footnote handling is summarized in the overview of Table 6. A quick parsing of the latest release has highlighted that, despite all the efforts of the RAN4 proponents, this footnote approach is prone to introducing multiple errors – see observation 2. This approach also produces a TS where it is difficult to identify which power class is supported for a given DL CA band combination. And with the additional power class options currently being studied in Rel-19, we believe RAN4 should decide if this footnote approach constitutes a future proof method to capture the valid UL power classes.

Table 6: Overview of Rel-18 power class footnotes in the 2,3,4,5,6 band BCS tables.

| UL CA PC / footnote / #Tx |  | two-band BCS Tables 5.5A.3.1-1a to 5.5A.3.1-1e | three-band BCS Table 5.5A.3.2-1a | four-band BCS Table 5.5A.3.3-1a | five-band BCS Table 5.5A.3.4-1a | six-band BCS Table 5.5A.3.5-1a |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 |  | default PC |  |  |  |  |
| PC2 2Tx | Footnote | Note 8 | Note 7 | Note 5 | Note 3 | no HPUE request |
|  | Number of Tx | 1Tx in each band | Not specified | Not specified | Not specified |  |
| PC2 3Tx | Footnote | Note 13 | n-a | n-a | n-a |  |
|  | Number of Tx | 1Tx+2Tx |  |  |  |  |
| PC1.5 | Footnote | Note 14 | Note 9 | Note 6 | Note 5 |  |
|  | Number of Tx | 1Tx+2Tx | Not specified | Not specified | Not specified |  |

We believe that:

- Even though the original intention of BCS tables was to capture only the valid CBW configuration of each band, it is essential that the supported UL power class for either single UL carrier or inter-band CA UL operation be captured, since this defines which type of MSD needs to be studied.

- However, capturing the valid UL power class could be simplified, and we consider two options:

  - Option 1:

    - Keep the Rel-18 footnotes only to capture the total composite UL CA power class.

    - Remove all considerations related to the number of Tx. The number of Tx is inherently captured in the UL CA power class tables in clause 6. Footnote 13 may be removed.

    - Remove the power class footnotes from the three-band and the four-band BCS tables.

    - Consider adding footnotes to accommodate future new power classes.

  - Option 2: remove all footnotes and adopt a format similar to that currently used for the Rel-19 WID excel requests, where both the power class and the number of Tx per UL Band is captured in dedicated columns. See the example shown in Table 5.

Table 7 highlights (in yellow) the footnotes that may be deleted. This is where  PC2 and the PC1.5 support is needed only for two-band and three-band BCS tables.

Table 7: Footnote simplification option 1. Deleted footnotes are highlighted in yellow.

| UL CA PC / footnote / #Tx |  | two-band BCS Tables 5.5A.3.1-1a to 5.5A.3.1-1e | three-band BCS Table 5.5A.3.2-1a | four-band BCS Table 5.5A.3.3-1a | five-band BCS Table 5.5A.3.4-1a | six-band BCS Table 5.5A.3.5-1a |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 |  | default PC |  |  |  |  |  |
| PC2 2Tx | Footnote | Note 8 | Note 7 | Remove note 5 | Remove note 3 | no HPUE request |
|  | Number of Tx | Remove the number of Tx | Not specified |  |  |  |
| PC2 3Tx | Footnote | Remove note 13 | n-a |  |  |  |
|  | Number of Tx | Remove the number of Tx |  |  |  |  |
| PC1.5 | Footnote | Note 14 | Note 9 | Remove note 6 | Remove note 5 |  |
|  | Number of Tx | Remove the number of Tx | Not specified |  |  |  |

Option 1 is only valid under the assumption that the HPUE dual-UL IMD test points have been specified for the worst-case HPUE UL number of Tx configurations. Since inter-band UL CA with PC1.5 can only be supported with dual-Tx in the TDD band, we assume that RAN4 has considered which of the PC2 2Tx or PC2 3Tx constitutes the worst-case front-end scenario for the affected DL band, or for the affected 3rd DL band, for the case of 3-band CA.

Table 8 shows that for the example of UL CA_n2A-n77A, there are possibly 6 different UL configurations that qualify for a composite UL PC2.

Table 8: UL CA_n2A-n77A specified Rel-18 power classes.

| UL CA_n2A-n77A PC2 configuration |  | Band n2A (FDD) | Band n77A (TDD) | UL-CA (FDD+TDD) | PC table |
| --- | --- | --- | --- | --- | --- |
| Conf. #1 | PC | PC3 | PC3 | PC2 | Table 6.2A.1.3-1 |
|  | #Tx | 1Tx | 1Tx | 2Tx |  |
| Conf. #2 | PC | PC3 | PC2 | PC2 | Table 6.2A.1.3-1 |
|  | #Tx | 1Tx | 1Tx | 2Tx |  |
| Conf. #3 | PC | PC3 | PC3 | PC2 | Table 6.2H.3.1-1 |
|  | #Tx | 1Tx | 2Tx with UL-MIMO | 3Tx |  |
| Conf. #4 | PC | PC3 | PC2 | PC2 | Table 6.2H.3.1-1 |
|  | #Tx | 1Tx | 2Tx with UL-MIMO | 3Tx |  |
| Conf. #5 | PC | PC3 | PC3 | PC2 | Table 6.2L.3.1-1 |
|  | #Tx | 1Tx | 2Tx with TxD | 3Tx |  |
| Conf. #6 | PC | PC3 | PC2 | PC2 | Table 6.2L.3.1-1 |
|  | #Tx | 1Tx | 2Tx with TxD | 3Tx |  |

Using the example of the three-band DL CA_n2A-n5A-n77A with UL CA_n2A-n77A PC2, we can extract the following four MSD test points that contain dual-UL CA_n2A-n77A IMD interference:

    - Band n2 MSD due to dual-UL IMD2 = 32.1dB from fall-back CA_n2A-n77A.

    - Band n2 MSD due to dual-UL IMD4 = 19.1dB from fall-back CA_n2A-n77A.

    - Band n2 MSD due to dual-UL IMD5 = 20dB from fall-back CA_n2A-n77A.

    - Band n5 MSD due to dual-UL IMD5 = 13.6dB.

Each of these PC2 test points are specified with a generic band n2/ band n77 UL configuration for which the number of Tx in band n77 is not specified. We therefore assume that the MSD has been specified, assuming the worst-case UL CA_n2-n77 configuration out of all six possible configurations captured in Table 8. Under this assumption, we believe that it is no longer necessary to keep both notes 8 and 13 in the two-band BCS tables to capture PC2 operation. Only note 8 is sufficient. Additionally, capturing the number of Tx is no longer needed.

Observation 3: It can be assumed that the PC2 dual-UL IMD test points for two and three band DL CA have been evaluated, with the worst-case PC2 UL configuration out of 2Tx or 3Tx with UL-MIMO/TxD. In which case:

- It is no longer necessary to capture PC2 with footnotes 8 and 13 in the two-band BCS tables. One footnote is sufficient.

- Capturing the number of Tx to achieve PC2 for inter-band UL-CA is no longer needed.

Proposal 3: To clarify the TS, and to better identify which UL CA power class is supported by a given DL CA band combination, interested companies are invited to evaluate the following two options for inter-band UL CA. We have a preference for Option 2 to trigger initial discussions.

  - Option 1: This option assumes that the dual-UL IMD test points are specified for the worst-case UL CA configuration (2Tx or 3Tx):

    - Keep the Rel-18 footnotes only to capture the total composite UL CA power class.

    - Remove all considerations related to the number of Tx. The number of Tx is inherently captured in the UL CA power class tables in clause 6. For example, footnote 13 of the two-band BCS table may be removed.

    - Remove the power class footnotes from the three-band and the four-band BCS tables.

    - Consider adding footnotes to accommodate future new power classes.

See the example of simplification shown in Table 7.

Table 7: Footnote simplification option 1. Deleted footnotes are highlighted in yellow.

| UL CA PC / footnote / #Tx |  | two-band BCS Tables 5.5A.3.1-1a to 5.5A.3.1-1e | three-band BCS Table 5.5A.3.2-1a | four-band BCS Table 5.5A.3.3-1a | five-band BCS Table 5.5A.3.4-1a | six-band BCS Table 5.5A.3.5-1a |
| --- | --- | --- | --- | --- | --- | --- |
| PC3 |  | default PC |  |  |  |  |
| PC2 2Tx | Footnote | Note 8 | Note 7 | Remove note 5 | Remove note 3 | no HPUE request |
|  | Number of Tx | Remove the number of Tx | Not specified |  |  |  |
| PC2 3Tx | Footnote | Remove note 13 | n-a |  |  |  |
|  | Number of Tx | Remove the number of Tx |  |  |  |  |
| PC1.5 | Footnote | Note 14 | Note 9 | Remove note 6 | Remove note 5 |  |
|  | Number of Tx | Remove the number of Tx | Not specified |  |  |  |

  - Option 2: remove all footnotes and capture more explicitly the power class and the number of Tx of each UL constituent band, as was done for the Rel-19 WID excel requests, where both the power class and the number of Tx per UL Band is captured in dedicated columns. See the example shown in Table 5.

Table 5: Example of Rel-19 two-band CA configuration requests.

# Conclusion

This contribution presents the results of parsing the UL CA power class captured in the latest Rel-18 NR CA configurations, and BCS defined for NR inter-band CA vs. the specified UL CA power class specified in clause 6. We observe several errors and omissions which we believe result from a footnote approach. Through each release, this condition has increased in complexity, making the TS difficult to read and difficult to cross-check the correctness of the specified UL power classes. We anticipate that with the new power class options being studied in Rel-19, that this footnote approach may become even more difficult to manage. We, therefore, submit Proposal 1, and Proposal 2, to correct the Rel-18 errors and ensure a clean Rel-19 start. Proposal 3 is an initial step to trigger further discussions on how to simplify the processes in which valid UL configurations and inter-band UL CA power classes are captured in Rel-19, and subsequent releases.

Observation 1: The NR CA configurations and BCS defined for two, three, four, five, and six band inter-band CA combinations have considerably expanded their scope since Rel-15, wherein their main role is to specify the supported CBW of the DL constituent bands.

In Rel-18 multiple dimensions have been added:

- For valid single uplink carrier and for UL CA configurations, HPUE support is captured using footnotes. This is reflected in the two-band BCS table, and also captures the number of Tx. However, the number of Tx is not captured in the three, four, and five band BCS tables. Footnotes include valuable supplemental information.

In Rel-19, it is expected that further complexity may be added to the current footnote approach. This accounts for new ULCA power classes enabled by PC2 in FDD bands, and also accounts for new single carrier operations with more than 2Tx.

Observation 2: With the current footnote approach, it is quite easy to introduce errors in the TS. We observe two categories of errors:

- HPUE captured using footnotes in the BCS tables, but not reflected in the clause 6 power class tables.

- HPUE specified in the clause 6 power class tables, but not reflected in the two, three, four, and 5 band BCS tables.

Observation 3: It can be assumed that the PC2 dual-UL IMD test points for two and three band DL CA have been evaluated, with the worst-case PC2 UL configuration out of 2Tx or 3Tx with UL-MIMO/TxD. In which case:

- It is no longer necessary to capture PC2 with footnotes 8 and 13 in the two-band BCS tables. One footnote is sufficient.

- Capturing the number of Tx to achieve PC2 for inter-band UL-CA is no longer needed.

Proposal 1: We invite proponents of the hereabove flagged UL CA band combinations to consider what was the intended power class requirement. Proponents should also clarify to which higher order DL CA band combination the missing PC requirements apply.

Proposal 2: Based on the clarifications of proposal 1, submit a CR for the meeting #113 to:

- Introduce in Table 6.2A.1.3-1 the missing UL CA power class requirements for UL CA_n25A-n41C, CA_n40A-n41C, CA_n41C-n66A, CA_n41C-n71A, CA_n41C-n77A, CA_n41A-n85A.

- Move the triple-beat MSD test point for CA_n25A-n41C from the PC2 table to the PC3 table.

- [FFS] on MSD test point for CA_n40A-n41C, depending on the outcome of RAN4 discussions.

- Introduce missing footnotes 13 and 14 to the two-band BCS Tables 5.5A.3.1-1a to Table 5.5A.3.1-1e, and propagate to the higher-order three, four-4, and five band BCS tables.

- Introduce new footnote 6 in Table 6.2L.3.1-1 “Note 6: The UE supports PC2 with TxD in either one of the TDD bands and PC2 in the other TDD band.”

- In Table 6.2A.1.3-1:

  - Remove the entry where only PC3 is specified for UL CA_n34A-n79A.

  - Change entry for CA_25A-n48A to “CA_n25A-n48A”

- In the two-band BCS Tables 5.5A.3.1-1a to Table 5.5A.3.1-1e, correct the UL configurations for DL CA_n25(2A)-n38A.

Proposal 3: To clarify the TS, and to better identify which UL CA power class is supported by a given DL CA band combination, interested companies are invited to evaluate the following two options for inter-band UL CA. We have a preference for Option 2 to trigger initial discussions.

  - Option 1: This option assumes that the dual-UL IMD test points are specified for the worst-case UL CA configuration (2Tx or 3Tx):

    - Keep the Rel-18 footnotes only to capture the total composite UL CA power class.

    - Remove all considerations related to the number of Tx. The number of Tx is inherently captured in the UL CA power class tables in clause 6. For example, footnote 13 of the two-band BCS table may be removed.

    - Remove the power class footnotes from the three-band and the four-band BCS tables.

    - Consider adding footnotes to accommodate future new power classes.

See the example of simplification shown in Table 7.

  - Option 2: Remove all footnotes and capture more explicitly the power class and the number of Tx of each UL constituent band, as was done for the Rel-19 WID excel requests, where both the power class and the number of Tx per UL Band is captured in dedicated columns. See the example shown inTable 5 below.

Table 5: Example of Rel-19 two-band CA configuration requests.