# 3GPP TSG-RAN WG4 Meeting #112bis                                                           R4-2416255

# Hefei, China, 14th – 20th October, 2024

Title: 	A-MPR evaluations of NS_03N, NS_N04N, NS_05N and NS_24 for 3MHz CBW for NTN communication.

Source: 	Huawei, HiSilicon

Agenda item:	6.8.3.2

Document for:	Discussion

# Introduction

RAN4 112 meeting was the first meeting about introduction of 3MHz CBW to NTN bands, n254, n255 and n256. Based on the WF [1], evaluation of A-MPR values for 3MHz CBW when NS_03N, NS_N04N, NS_05N and NS_24 is going to be done in the following sections

# Discussion

The PA was calibrated based on the following assumptions:

- PC3

- 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc, Image: 28dBc, CIM3: 60dBc, CIM5: 70dBc

- ASE and A-SEM, SEM and ACLR are considered as gating factors (EVM was excluded)

The Table 6.2.3.1-1A of TS 38.101-5 is reproduced below to show the mapping of the highlighted NS values to different bands

TS 38.101-5 Table 6.2.3.1-1A: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n256 | NS_01 | NS_24 | NS_100 |  |  |  |  |  |
| n255 | NS_01 | NS_02N |  |  |  |  |  |  |
| n254 | NS_01 | NS_03N | NS_04N | NS_05N |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. |  |  |  |  |  |  |  |  |

One can observe that NS_24 maps to n256 and NS_03N, NS_04N and NS_05N map to n254. The following sections each of the highlighted NS values will be evaluated.

## NS_24

NS_24 was evaluated during introduction of n65 in Rel-16 and a 5MHz offset from 2010MHz  was considered for its evaluations[2]. We also considered the same offset for our evaluations as shown in Figure 2.1-1.

Figure 2.1-1 5 MHz offset from 2010 MHz

Figure 2.1-2 shows A-MPR evaluations for 3MHz CBW, QPSK modulation at Fc=2003.5MHz (i.e. closest to protected region) and 0dB is assigned if A-MPR ≤ PC3 MPR. The simulations about other modulation orders and carrier frequencies can be found in Appendix NS_24.

Figure 2.1-2 A-MPR evaluations of a 3MH CBW, with QPSK modulation, centered at 2003.5 MHz

Based on our evaluation we propose the following A-MPR regions and values.

Proposal 1: Consider the following A-MPR regions and values for NS_24 with 3MHz CBW.

Table 2.1-1 A-MPR for NS_24 for 3MHz CBW

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 3MHz | 1997.5 < Fc ≤ 2000.5 |  | ≥2.16 | A3 |
| 3MHz | 2000.5 < Fc ≤ 2003.5 |  | ≥1.44 | A4 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |

Table 2.1-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A3 | A4 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 7 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 8 |
| DFT-s-OFDM 64 QAM | ≤ 3.5 | ≤ 8.5 |
| DFT-s-OFDM 256 QAM |  | ≤ 8 |
| CP-OFDM QPSK | ≤ 4.5 | ≤ 8.5 |
| CP-OFDM 16 QAM | ≤ 4.5 | ≤ 8.5 |
| CP-OFDM 64 QAM | ≤ 4.5 | ≤ 8.5 |
| CP-OFDM 256 QAM |  | ≤ 8.5 |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 2.1-1 |  |  |

## NS_03N

NS_03N is issued to apply the FCC regulatory conditions. When it is issued the additional spurious emissions requirements are applicable per TS 38.101-5 sections 6.5.2.3.1 and 6.5.3.3.4, respectively. Figure 2.2-1 shows A-MPR evaluations for 3MHz CBW, QPSK modulation at Fc= 1611.5MHz (i.e. closest to protected region) and 0dB is assigned if A-MPR ≤ PC3 MPR. The simulations about other carrier frequencies can be found in Appendix NS_05N. The simulations shows that A-MPR is only needed for Fc=1611.5

Proposal 2: Consider the following A-MPR regions and values for NS_03N with 3MHz CBW.

Table 2.2-1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 1611.5 <= fc < 1613 |  | >= 2.16 | A2 |

Table 2.2-2: A-MPR for NS_03N

|  | Modulation | A2 |
| --- | --- | --- |
| DFT-s-OFDM | QPSK | 3.5 |
|  | 16QAM | 4.0 |
|  | 64QAM | 4.5 |
|  | 256QAM | 5 |
| CP-OFDM | QPSK | 4.5 |
|  | 16QAM | 4.5 |
|  | 64QAM | 4.5 |
|  | 256QAM | 4.5 |

## NS_04N

NS_04N is issued to apply the ETSI 301 441 regulatory conditions. When it is issued the additional SEM and additional spurious emissions requirements are applicable per TS 38.101-5 sections 6.5.2.3.1 and 6.5.3.3.4, respectively. Figure 2.3-1 shows A-MPR evaluations for 3MHz CBW, QPSK modulation at Fc= 1611.5MHz (i.e. closest to protected region) and 0dB is assigned if A-MPR ≤ PC3 MPR. The simulations about other carrier frequencies can be found in Appendix NS_04N. The simulations shows that A-MPR is only needed for Fc=1611.5

Figure 2.3-2 A-MPR evaluations of a 3MH CBW, with QPSK modulation, centered at 1611.5 MHz

Based on our evaluation we propose the following A-MPR regions and values.

Proposal 3: Consider the following A-MPR regions and values for NS_04N with 3MHz CBW.

Table 2.3-1: A-MPR regions for NS_04N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 1611.5 <= fc < 1613 |  | >= 2.16 | A2 |

Table 2.3-2: A-MPR for NS_04N

|  | Modulation | A2 |
| --- | --- | --- |
| DFT-s-OFDM | QPSK | 3.5 |
|  | 16QAM | 4.0 |
|  | 64QAM | 4.5 |
|  | 256QAM | 5 |
| CP-OFDM | QPSK | 4.5 |
|  | 16QAM | 4.5 |
|  | 64QAM | 4.5 |
|  | 256QAM | 4.5 |

## NS_05N

NS_05N is issued to apply the ETSI 301 441 regulatory conditions. When it is issued the additional SEM and additional spurious emissions requirements are applicable per TS 38.101-5 sections 6.5.2.3.2 and 6.5.3.3.4, respectively. Our A-MPR evaluations, shown in the Appendix NS_05N, showed that MPR should be enough for 3MHz and no A-MPR is needed.

Proposal 4: No A-MPR is needed for 3MHz CBW when NS_05N is issued.

# Conclusions

Proposal 1: Consider the following A-MPR regions and values for NS_24 with 3MHz CBW.

Table 2.1-1 A-MPR for NS_24 for 3MHz CBW

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 3MHz | 1997.5 < Fc ≤ 2000.5 |  | ≥2.16 | A3 |
| 3MHz | 2000.5 < Fc ≤ 2003.5 |  | ≥1.44 | A4 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |

Table 2.1-2: A-MPR for modulation and waveform type

| Modulation/Waveform | A3 | A4 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 7 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 8 |
| DFT-s-OFDM 64 QAM | ≤ 3.5 | ≤ 8.5 |
| DFT-s-OFDM 256 QAM |  | ≤ 8 |
| CP-OFDM QPSK | ≤ 4.5 | ≤ 8.5 |
| CP-OFDM 16 QAM | ≤ 4.5 | ≤ 8.5 |
| CP-OFDM 64 QAM | ≤ 4.5 | ≤ 8.5 |
| CP-OFDM 256 QAM |  | ≤ 8.5 |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 2.1-1 |  |  |

Proposal 2: Consider the following A-MPR regions and values for NS_03N with 3MHz CBW.

Table 2.2-1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 1611.5 <= fc < 1613 |  | >= 2.16 | A2 |

Table 2.2-2: A-MPR for NS_03N

|  | Modulation | A2 |
| --- | --- | --- |
| DFT-s-OFDM | QPSK | 3.5 |
|  | 16QAM | 4.0 |
|  | 64QAM | 4.5 |
|  | 256QAM | 5 |
| CP-OFDM | QPSK | 4.5 |
|  | 16QAM | 4.5 |
|  | 64QAM | 4.5 |
|  | 256QAM | 4.5 |

Proposal 3: Consider the following A-MPR regions and values for NS_04N with 3MHz CBW.

Table 2.3-1: A-MPR regions for NS_04N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 1611.5 <= fc < 1613 |  | >= 2.16 | A2 |

Table 2.3-2: A-MPR for NS_04N

|  | Modulation | A2 |
| --- | --- | --- |
| DFT-s-OFDM | QPSK | 3.5 |
|  | 16QAM | 4.0 |
|  | 64QAM | 4.5 |
|  | 256QAM | 5 |
| CP-OFDM | QPSK | 4.5 |
|  | 16QAM | 4.5 |
|  | 64QAM | 4.5 |
|  | 256QAM | 4.5 |

Proposal 4: No A-MPR is needed for 3MHz CBW when NS_05N is issued.

# Reference

[1]	“R4-2413528, WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi.”

# Appendix NS_24

# Appendix NS_03N

# Appendix NS_04N

# Appendix NS_05N