# 3GPP TSG-RAN WG4 Meeting # 112bis					R4-2416257

# Hefei, China, 14th – 18th  October, 2024

Source:		Ericsson

Title:			RedCap UE RF impact

Agenda item:		6.26.2.1

Document for:		Approval

# Introduction

In this paper, we present our general view on the RedCap UE RF remaining issue.

# Discussion

In WF[1], the remaining issue is below:

## Issue 1-4: Simultaneous operation with GNSS

Agreement: Further study NR NTN UE simultaneous operation with GNSS considering e.g.

- Operating bands with UL close to GNSS frequencies (n254, n255)

- Emissions on GNSS frequencies

- Whether transmission gaps enabling successful GNSS reception are present

- Potential impact on field performance

- Potential need for specification updates, e.g. frequency error

In our understanding, the simultaneous operation is assumed for NR NTN, which is different with IoT NTN. In TR36.763 for IoT NTN, the simultaneous GNSS and NTN is not assumed.

NR NTN assume the simultaneous operation

IoT NTN does not assume the simultaneous operation

## 5.2	IoT NTN UE capabilities

GNSS capability in the UE is taken as a working assumption in this study for both NB-IoT and eMTC devices.

Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed.

In TR 38.741, the FCC requirement 25.216 is captured to protect the aeronautical radionavigation-satellite service and such requirement is reflected in TS38.101-5 with NS values.

GNSS protection is in place in TS 38.101-5

Though the issue for Issue 1-4 from WF[1] is for in-device coexisting,  RAN4 needs understand better if the GNSS receiver desensitization will not occur when the NS values for GPS protection are enabled. If additional transmitter power backoff would be allowed on top the allowed A-MPR, it may endanger the uplink link budget.

Check if the issue 1-4 does not apply to region 2.

# Conclusions

In this contribution, we present our view on the remaining issue for  Redcap UE RF:

Observation 1 NR NTN assume the simultaneous operation

Observation 2 IoT NTN does not assume the simultaneous operation

Observation 3 GNSS protection is in place in TS 38.101-5

Proposal-1: Check if the issue 1-4 does not apply to region 2.

# References

- R4-2413527, Way Forward for [112][311] NR_NTN_Ph3_UE_RF, Qualcomm Inc