3GPP TSG-RAN WG4#112-bis	R4-2416258

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.5.2.2

Source: 	Qualcomm France

Title: 	Non-collocated Intra-band NR CA/EN-DC UE requirements

Document for:	Approval

- Introduction

Considerations on open aspects for Non-collocated NR CA/EN-DC UE are provided in this contribution.

- WF in RAN4#112

WF [1]:

## Sub-topic 2-2: UE RF requirements for Type 4 EN-DC/NR-CA

< Issue 2-2-1:  How to capture the power imbalance and REFSENS requirements >

Way forward:

- No rush to make decision on CR. Discuss it as big CR.

## Sub-topic 2-3: UE Capability(s)/UE behavior and BS signalling for Type 4 EN-DC/NR-CA

### < Issue 2-3-1:  New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA >

Way forward:

- Define one capability for NR CA and one UE capability for EN-DC

  - FFS on how to differentiate type 4a and 4b.

### < Issue 2-3-2:  Whether to support Type 2 capability by UE having Type 4 capabilities >

Way forward:

- Modify last meeting’s agreements as following with yellow highlighted part.

RAN4#111 < Issue 2-2-2: Whether to support Type 2 capability by UE having Type 4 capabilities > Agreement: - If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default. UE still needs to report Type 2 Capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS). regardless of whether UE indicates Type-2 capability or not. - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities. - If the issue is identified for these bullets, RAN4 will revisit them.

### < Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)>

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

### < Issue 2-3-5:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

### < Issue 2-3-6:  Switch between Type 4a/4b and single CC operation >

Way Forward:

- No new BS signaling needed from Type-4 to single carrier fallback.

### < Issue 2-3-7:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4 and BS signaling.

- And then, continue further discussion on UE behavior in the next meeting.

### <Issue 2-3-9: When to inform RAN2 the demand on new UE capability(s) and new BS signaling>

Way Forward:

- Continue further discussion in the next meeting.

## Sub-topic 2-5: LS to RAN5

< Issue 2-5-1:  LS to RAN5 >

Way Forward:

- LS for type 4 frequency separation to RAN5 is needed. R4-2411419 is Postponed to the end of this WI.

- Discussion

< Issue 2-3-1:  New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA >

Way forward:

- Define one capability for NR CA and one UE capability for EN-DC

  - FFS on how to differentiate type 4a and 4b.

This way forward should be sufficient for RAN4. RAN2 will define UE capabilities in similar manner as for type 2, one for NR CA and one for EN-DC. There is no particular reason why RAN4 should consider how to differentiate type 4a and type 4b; that can be done in RAN2.

Proposal 1: RAN2 will define one UE capability for NR CA and one UE capability for EN-DC. There is no need for RAN4 to discuss how to differentiate type 4a and type 4b as that can be done in RAN2

< Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)>

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signaling in the next meeting.

It would be beneficial to define new BS signalling for this case, to maintain consistency with Type 2 signaling. An issue when the NW is Rel-18 and UE is Rel-19 was discussed by interested companies in previous meeting. In that case when UE indicates support of Type 2 and Type 4 including 4 MIMO layers, it was commented that Rel-18 NW does not understand Rel-19 capabilities and hence would only understand 4 MIMO layers. In our view, NW should manage this aspect because UE has indicated both Rel-18 UE capabilities and Rel-19 UE capabilities. So even NW sees maxMIMO-Layers =4 it should we able to deduce that UE supports Type 2, because if not it would not have indicated Type 2 support at all.

Proposal 2: Specify BS signalling for NR CA and EN-DC, e.g. “nonCollocatedTypeNR-CA-r19” and

“nonCollocatedTypeMRDC-r19” using same logic as in Type 2. No impacts to specifications to manage case when Rel-19 UE operates in Rel-18 NW

< Issue 2-3-5:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

UE supporting Type 4a/4b could optionally support 6L/8L per CC in collocated mode (Type 1). The default is anyhow 4L/CC for Type 1. There is no new UE capability/signaling required for that as UE can already in current specifications indicate maxMIMO-Layers and can reconfigure between different number of MIMO layers when in Type 1.

Proposal 3: No need for BS signalling to switch between Type 4a/4b and Type 1 with 6L/8L per CC

< Issue 2-3-7:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4 and BS signaling.

- And then, continue further discussion on UE behavior in the next meeting.

The following is observed:

Proposal 4: UE behaviour during initial connection is assumed as follows for a UE which supports intraBandNR-CA-non-collocated-r19 and/or interBandMRDC-WithOverlapDL-Bands-r19

-If maxMIMO-layers≥6

-UE is Type 1

-If maxMIMO-layers=4

- UE is Type 1 or Type 4 depending on if Rel-19 BS signalling field(s) is/are present/absent and NW is Rel-19

-UE is Type 2 if Rel-18 BS signalling field(s) is/are absent and NW is Rel-18

-If maxMIMO-layers=2

-UE is type 1 if intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16 are present

-UE is type 2 if intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16 are absent

<Issue 2-3-9: When to inform RAN2 the demand on new UE capability(s) and new BS signaling>

Way Forward:

- Continue further discussion in the next meeting.

RAN4 should conclude all the work and inform RAN2 in a single LS after everything is ready, including mid-term check in December plenary and possible respective actions.

Proposal 5: RAN4 should first conclude everything in Type 4a/4b and see if Type 3a/3b is included later into the WID scope, and after that send all UE capability and NW signalling decisions related into this WID into RAN2 on one go.

- Conclusion

Remaining aspects for Non-collocated Rel-19 Intra-band Non-contiguous CA/EN-DC were considered with the following observations and proposals.

Proposal 1: RAN2 will define one UE capability for NR CA “intraBandNR-CA-non-collocated-r19” and one UE capability for EN-DC “interBandMRDC-WithOverlapDL-Bands-r19”. There is no need for RAN4 to discuss how to differentiate type 4a and type 4b as that can be done in RAN2

Proposal 2: Specify BS signalling for NR CA and EN-DC, e.g. “nonCollocatedTypeNR-CA-r19” and

“nonCollocatedTypeMRDC-r19” using same logic as in Type 2. No impacts to specifications to manage case when Rel-19 UE operates in rel-18 NW

Proposal 3: No need for BS signalling to switch between Type 4a/4b and Type 1 with 6L/8L per CC

Proposal 4: UE behaviour during initial connection is assumed as follows for a UE which supports intraBandNR-CA-non-collocated-r19 and/or interBandMRDC-WithOverlapDL-Bands-r19

-If maxMIMO-layers≥6

-UE is Type 1

-If maxMIMO-layers=4

- UE is Type 1 or Type 4 depending on if Rel-19 BS signalling field(s) is/are present/absent and NW is Rel-19

-UE is Type 2 if Rel-18 BS signalling field(s) is/are absent and NW is Rel-18

-If maxMIMO-layers=2

-UE is type 1 if intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16 are present

-UE is type 2 if intraBandNR-CA-non-collocated-r18 and interBandMRDC-WithOverlapDL-Bands-r16 are absent

Proposal 5: RAN4 should first conclude everything in Type 4a/4b and see if Type 3a/3b is included later into the WID scope, and after that send all UE capability and NW signalling decisions related into this WID into RAN2 on one go.

- Reference

[1] R4-2414453, “WF on Rel-19 non-collocated scenario”, KDDI