# 3GPP TSG-RAN WG4 Meeting #112bis                                                           R4-2416260

# Hefei, China, 14th – 20th October, 2024

Title: 	How to introduce UE types into the specifications and rectifying errors UE architecture types

Source: 	Huawei, HiSilicon

Agenda item:	6.5.2.3

Document for:	Discussion

# Introduction

RAN4 112th meeting the WF was agreed to introduce type 1 and type 2 UE architecture definitions to the specifications starting Rel-18[1]. As in this meeting no Rel-18 CR is permitted, per meeting agenda, within  section 2.1 of this document, we will outline proposed text revisions intended for use in forthcoming Change Request. Besides type 4, which is defined in Rel-19, can be defined using the following methods. On the section 2.2 we will address a mistake that was made during Rel-18 in UE architecture definitions that needs to be rectified for the continuity of Rel-19.

# Discussion

## How to introduce type UE type 1,2 and 4

### How to introduce type1, type 2 and type 4 definitions into the 38.101-1 for NR CA

The initial step is to determine the most appropriate section for specifying the definitions of the various types. We recommend considering two potential sections: 7.1A and 7.10A.

Section 7.1A could give a foundational overview, allowing readers to grasp the different types of UE receiver architectures from the outset. This early introduction facilitates a comprehensive understanding of the subject matter as the document progresses.

On the other hand, section 7.10A also presents a viable option, particularly since it will include specifications for type 2 (and type 4). By placing the definitions here, readers can quickly appreciate the relevant use cases associated with these specific types, streamlining their understanding of how these definitions apply in context.

Ultimately, the choice between these sections should consider the balance between providing foundational knowledge and enhancing the reader's ability to immediately apply the definitions to practical scenarios

Proposal 1: Use either TS 38.101-1 7.1A or 7.10A to include the definitions of UE type 1, 2 and 4.

The second step would be how to define the different UE architecture types. One way would be to define them as a table which includes all the necessary information such as Table 1. It should be mentioned that in the type description column, one can remove the MTDD, and MRTD descriptions as they are related to RRM specifications. However preserving them would be helpful in proper clarifications of each UE types.

Table 1 Intra-band non-contiguous CA UE types

| intra-band non-contiguous CA UE types | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | the UE supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | [TBD] |

The other way would be to use regular text to define different types such as:

“An intra-band non-contiguous CA type 1 UE supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133.”

“An intra-band non-contiguous CA type 2 or 4 UE supports inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133. Such a UE shall report intraBandNR-CA-non-collocated-r18 capability”

Using the table could be a better solution because, when updating other parts of the specification, we can refer to a given type without using extensive phrases. For example NOTE 6 of Table 5.5A.2-1 in TS38.101-1 can be updated as follows:

NOTE 6: If a UE does not indicate intraBandNR-CA-non-collocated-r18, the minimum requirements for intra-band non-contiguous CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB(type 1 in Table 7A-XXXX). If a UE indicates intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2, the power imbalance requirement defined in subclause 7.10A apply(type 2 in Table 7A-XXXX).. If a UE indicates intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, the minimum requirements for intra-band non-contiguous CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB(type 1 in Table 7A-XXXX).. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable intra-band non-contiguous NR CA configuration is a subset of a higher order NR CA configuration.

Proposal 2: Use Table 1 for defining different intra-band non-contiguous CA UE types UE types. If needed the RRM parameters can be removed, however preserving them would be helpful in proper clarifications of each UE types

### How to introduce type1, type 2 and type 4 definitions into the 38.101-3 for EN-DC

Similar to NR CA case we recommend considering two potential sections for EN-DC: 7.1 and 7.10B.

Section 7.1 could give a foundational overview, allowing readers to grasp the different types of UE receiver architectures from the outset. This early introduction facilitates a comprehensive understanding of the subject matter as the document progresses.

On the other hand, section 7.10B also presents a viable option, particularly since it will include specifications for type 2 (and type 4). By placing the definitions here, readers can quickly appreciate the relevant use cases associated with these specific types, streamlining their understanding of how these definitions apply in context.

Proposal 3: Use either TS 38.101-3 7.1 or 7.10B to include the definitions of type 1,2 and 4.

Regarding how to capture the UE type definitions, we suggest the same method as NR-CA and use Table 2 which would help to better referencing, too. It should be mentioned that in type description, one can remove the MTDD, and MRTD descriptions as they are related to RRM specifications, however preserving them would be helpful in proper clarifications of each UE types.

Table 2: Inter-band EN-DC operation with overlapping or partially overlapping DL bands UE types

| For inter-band EN-DC operation with overlapping or partially overlapping DL bands UE types | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | The UE supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in clause 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in clause 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | [TBD] |

Proposal 4: Use Table 2 for defining different UE types for inter-band EN-DC operation with overlapping or partially overlapping DL bands. If needed the RRM parameters can be removed, however preserving them would be helpful in proper clarifications of each UE types

### RRM specification update

In our opinion it would be better to have the RRM CRs after the RF CRs in order to ensure a harmonized approach.

Proposal 5: It would be better to have the RRM CRs after the RF CRs in order to ensure a harmonized approach.

## Correction on architecture of UE type 1 and type 3

During Rel-18 architecture of different UE types were gathered on a table in the WF R4-2217734. At the time, type 1 UE architecture was defined with shared mixers, AGC, LNA and BB among all the CCs. Type 1 was considered as the legacy UE Rx architecture. Moreover, type 3 was considered as the architecture with partially shared components (Shared LNA and AGC among all CCs. Separated mixers and analog BBs for each CC). On the other hand in TR 36.823 section 6.2.3.1 (Rel-15) the reference architecture for intra-band non-contiguous CA was considered with partially shared components and not fully shared ones. Therefore that table needs to be updated as follows:

Table 2.2-1 UE Rx architecture types

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6dB full range | => common AGC on LNA, separate Mixer and LPFs |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 1b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6dB full range | => common AGC on LNA, separate Mixer and LPFs |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | 2Rx/band but need 2LO frequencies: each CC has its specific AGC on LNA |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

It should be mentioned that by considering type 1 with separated mixers and low pass filters (LPF) for each CC, the in-gap interferences, which is due to the intra-band non-contiguous CA configuration, will have no more impact. The LPF of each CC will block in-gap interferences, which is the behavior of the UEs prior to Rel-18.

Fortunately the specifications are written in an architecture agnostic manner, hence no error was introduced into the 
specifications when we specified the Rel-18 requirements for NonCol_intraB_ENDC_NR_CA WI.

Proposal 6: Update the Table in WF R4-2217734, which is mentioned in the Rel-19 WID, to Table 2.2-1.

# Conclusions

Proposal 1: Use either TS 38.101-1 7.1A or 7.10A to include the definitions of type 1 and type 2

Proposal 2: Use Table 1 for defining different intra-band non-contiguous CA UE types UE types. If needed the RRM parameters can be removed, however preserving them would be helpful in proper clarifications of each UE types

Table 1 Intra-band non-contiguous CA UE types

| intra-band non-contiguous CA UE types | Type description | dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and MRTD requirements according to clause 7.6.4-1 in 38.133 | N/A |
| Type 2 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | intraBandNR-CA-non-collocated-r18 |
| Type 4 | The UE supports the inter-band RF requirements (with DL power imbalance as specified in clause 7.10A.2) and MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 | [TBD] |

Proposal 3: Use either TS 38.101-3 7.1 or 7.10B to include the definitions of type 1 and type 2.

Proposal 4: Use Table 2 for defining different UE types for inter-band EN-DC operation with overlapping or partially overlapping DL bands. If needed the RRM parameters can be removed, however preserving them would be helpful in proper clarifications of each UE types

Table 2: Inter-band EN-DC operation with overlapping or partially overlapping DL bands UE types

| For inter-band EN-DC operation with overlapping or partially overlapping DL bands UE types | Type description | Dedicated UE capability |
| --- | --- | --- |
| Type 1 | It supports intra-band RF requirements (with a maximum power spectral density imbalance between downlink carriers, within 6 dB) and an (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133. | N/A |
| Type 2 | The UE supports inter-band RF requirements (with DL power imbalance as specified in clause 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | interBandMRDC-WithOverlapDL-Bands-r16, requirementTypeIndication-r18 |
| Type 4 | The UE supports inter-band RF requirements (with DL power imbalance as specified in clause 7.10A in TS 38.101-3) and an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 | [TBD] |

Proposal 5: It would be better to have the RRM CRs after the RF CRs in order to ensure a harmonized approach.

Proposal 6: Update the Table in WF R4-2217734, which is mentioned in the Rel-19 WID, to Table 2.2-1.

Table 2.2-1 UE Rx architecture types

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6dB full range | => common AGC on LNA, separate Mixer and LPFs |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 1b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6dB full range | => common AGC on LNA, separate Mixer and LPFs |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | 2Rx/band but need 2LO frequencies: each CC has its specific AGC on LNA |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

# Reference

[1]	“R4-2414454, WF on Rel-18 clarifications of RF requirements for different UE types, Huawei.”