# 3GPP TSG-RAN WG4 Meeting # 112-bis                                                         R4-2416262

# Hefei, China, 14th – 18th October 2024

# Title:	CA_n40-n41 Simultaneous RxTx

# Agenda Item:	5.11.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document presents our views on the challenges for CA_n40-n41 simultaneous RxTx. This contribution serves as a follow up to the latest agreed upon Way Forward (WF) [1] on that band combination.

# Discussion

The following summarizes the options that were agreed upon at the last meeting, and itemizes our discussion on CA_n40-n41 with simultaneous RxTx [1]:

Option 1: Keep the requirements with n41 using 4Rx in simultaneous Rx-Tx for CA_n40-n41.

Option 2: Analyse the UE architecture with n41 using 2Rx in simultaneous Rx-Tx for CA_n40-n41 and try to define the separate requirement including MSD optionally from that in Rel-18, taking the implementation difficulties into consideration.

- However, the UE should support 4Rx for n41 when the UE does not indicate simultaneous Rx-Tx for CA_n40-n41, i.e single n41 or other CA combo.

Option 3:  Study whether new UE signalling(s) could be introduced for a certain band combination to indicate that:

- Concurrent support of 4x4 MIMO and simultaneous Rx/Tx is not supported.

- Concurrent support of UL MIMO, and/or TxD and/or SRS antenna switching, and simultaneous Rx/Tx is not supported.

Challenges to support CA_n40-n41 with simultaneous RxTx have been extensively discussed at RAN4 for more than two years, with initial proposals to account for handset antenna challenges reported in meeting #105 [2]. Since then, this band combination has been the subject of several agreed upon WFs [3, 4, and 5], to include more than 20 discussion papers. However, no agreement was reached in Rel-18 on the subject of relaxations related to simultaneous RxTx. More recently, the request to support UL CA_n41C has further added complexity to the discussions, since RAN4 was not able to agree upon MSD requirements, despite previous agreement on an MSD test point [5].

  - MSD issues for CA_n40-n41

    - One uplink CA_n40A-n41A MSD due to cross-band isolation interference

Downlink (DL) CA_n40A-n41A with 1 uplink (UL) operation may be affected by high cross-band isolation interference where:

- Band n41 may be affected by ACLR2 interference due to 1UL Band n40 transmission (see Figure 1, left side). The PC3 Band n41 10 MHz channel bandwidth (CBW) is 28.1 dB due to ~-23dBm interference level referred to the PA output pin. For PC2, the band n41 10MHz MSD is 31.1dB.

- Band n40 may be affected by ACLR2 interference due to 1 UL Band n41 transmission (see Figure 1, right side) with a 5 MHz CBW MSD of 31.4 dB. Note that the upper 5 MHz DL channel is in very close proximity to the Band n41 ACLR1 interference.

Figure 1:  CA_n40A-n41A cross-band isolation MSD due to 1UL transmissions.

    - CA_n40A-n41C MSD due dual-UL IMD interference

Support for intra-band contiguous UL CA_n41C has recently been requested for DL CA_n40A-n41C. Figure 2 shows that under the regional spectrum allocation of Band n41, the Band n40 5 MHz uppermost DL may be affected by a dual-UL IMD3 interference condition. This MSD has been extensively studied [6..12] and RAN4 is still considering which CA_n41C UL RB allocation should be specified for the MSD test point:

- Configuration #1:

  - Adopt the legacy 1RB allocation on each UL CC, i.e. “1RB+1RB”.

  - Band n40 5 MHz CBW MSD ranges from 42.5 dB to 55 dB [6, 7, 8, 9, and 10].

  - To our knowledge, only DC_3_n3 was able to reach MSD > 40 dB. None of the NR-CA band combinations studied at RAN4 have ever exceeded more than 50 dB MSD.

- Configuration #2:

  - Adopt full RB allocation on each UL CC.

  - Band n40 5 MHz CBW MSD: from 35 dB to 45 dB, with proposal at 31.4 dB [12].

Figure 2:  Band n40 MSD due to UL CA_n41C for DL CA_n40A-n41C with consideration for regional deployment.

Observation 1: With MSD ranging from 42.5 dB to 55 dB for the UL configuration “1RB+1RB”, the Band n40 MSD for CA_n40A-n41C with UL CA_n41C is the highest NR MSD that RAN4 has ever studied. Even if we consider fully allocated UL component carriers, the Band n40 MSD remains very high (31 ~ 45 dB). The Band n41 MSD due to Band n40 cross-band isolation is also very high (28.1 dB).

Currently, and until the debate of UL RB allocation is closed, we consider that the Rel-18 conventions on UL RB allocations for DL CA MSD (due to dual-UL interference from an intra-band UL configuration) are still valid. We therefore present below our MSD proposal, based on [10] where we have modified “RBSTART” to reflect the IMD landscape of Figure 2. We reiterate that this proposal assumes that the UE applies the MPR allowance to meet the -13 dBm/MHz emission requirements.

Proposal 1: For CA_n40A-n41C, consider adopting the Power Class 3 (PC3) Band n40 MSD test point shown in Table 1.

Table 1: PC3 Band n40 MSD/REFSENS for CA_n40A-n41C.

| Band / Channel Bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Band combination | NR Band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | TDD | IMD3 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 38) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 238) | 2625 |  |  |  |

The excessively high MSD levels for 1UL cross-band isolation and for dual-UL are summarized in Table 2.

Table 2: Option 1/2 NR-CA HPUE MSD table simplification. HPUE MSD tables are highlighted in yellow.

| UL Band | DL Band | UL Fc | UL BW | SCS of UL Band | UL RB Allocation | DL Fc | DL BW | MSD | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n40 | n41 | 2350 | 100 | 30 | 270 (RBstart=3) | 2501 | 10 | 28.1 | 1UL ACLR2 |
| n41 | n40 | 2546 | 100 | 30 | 270 (RBstart=0) | 2397.5 | 5 | 31.4 | 1UL ACLR2 |
|  | n40 | N/A |  |  |  | 2397.5 | 5 | [42.5] | 2UL IMD3 |
| n41 |  | 2545 | 60 | 30 | 1 (RBSTART= 38) | N/A |  |  |  |
|  |  | 2625 | 100 | 30 | 1 (RBSTART= 238) | N/A |  |  |  |

  - Additional issues due to Simultaneous RxTx

Table 3 reproduces the agreed upon valid configurations for CA_n40A-n41A, where the valid UL configurations are:

- Single UL carrier in Band n40 or in Band n41, and support for PC3and PC2 with up to 2Tx and PC1.5, which by definition is 2Tx.

- Dual UL CA_n40A-n41A with support for PC3 and PC2, with 1Tx in each band.

- UL CA with UL-MIMO or with TxD is not supported.

Table 3: Valid UL configurations for DL CA_n40A-n41A.

Supporting a complex band combination, such as CA_n40-n41 with simultaneous RxTx, and without consideration for multiplexing or carrier aggregation with any other band(s), can be accomplished with a variety of architectures. This includes adding filters, adding LNAs, adding PAs, or adding antenna ports. Ultimately, the issue becomes a matter of tradeoff between cost and complexity under the constraints of the UE form factor. The architecture captured as background information in WF [1] contains four antenna ports, six LNAs and two PAs, and with its switching capability, would enable the following configurations:

- Single UL carrier in Band n40 or Band n41, with 1Tx in each band, and PC3 or PC2 1Tx.

- UL CA_n40A-n41A for PC3 or PC2 operation, with 1Tx in each band.

- 4Rx in Band n41 and 2Rx in Band n40.

On the subject of 4Rx support in Band n41, the proponents of this proposal [13] consider that due to the high Tx ACLR interference, the Band n41 4Rx requirement may be relaxed to 2Rx, only when the UE is near to the cell edge. While we acknowledge that this is true for the cell edge, we also believe that the area of the cell where Tx interference may require relaxation for Band n41 4Rx to 2Rx may be much greater than just the cell edge:

- From a DL RSRP perspective, we have reported in [14] measured Band n40 PA ACLR interference levels of ~-23dBm and ~-20dBm at the PA output pin. These high interference levels may impact a significantly larger area than just cell-edge,

From an UL perspective:

- It is not uncommon to observe TDD UEs transmitting at maximum output power in areas other than the cell edge. This is due to a) commercial TDD networks which are configured with a much smaller number of UL slots than the number of DL slots, b) because of this UL “time bottleneck”, some TDD schedulers may strive to minimize the transmission time. One method is to allocate the highest MCS, which in turn requires transmission at a high output power to meet the higher SNR requirements of high MCS, c) in rare, UL-heavy use cases (such as with a large file upload to a cloud server), a TDD UE may transmit at maximum output power, even in areas very close to the base station.

- The ACLR performance of an envelope-tracked PA does not decrease suddenly with output power back-off. To the contrary, the ACLR performance is often constant over several dBs below Pcmax, and in some UEs, the ACLR only starts to improve when the PA control scheme is switched from ET mode to the APT mode.

Observation 2:  Even with the six LNA, two PAs, four antenna architecture of WF [1], the Band n41 4Rx relaxation to 2Rx may be required in many areas of the cell, not just the cell edge.

We consider that the discussions on simultaneous TxRx should account for an architecture that may be qualified as “low-cost” with regards to the number of components. For this approach, our objective is to support CA_n40A-n41A at no extra cost. This architecture contains four antennas, two PAs and four LNAs to support 4Rx in Band n41, 2 or 4 Rx in Band n40, either for single band operation, or for CA of Band n40, and Band n41 with bands other than Band n40/n41.When this architecture is configured to support CA_n40A-n41A:

- The four LNAs are shared to support both bands, meaning that only 2Rx can be supported in each band, i.e. Band n41 4Rx needs to be downgraded to 2Rx.

- The two PAs can be shared between both bands, i.e. enabling either UL-MIMO/TxD in either Band n41 or Band n40, or UL CA_n40A-n41A with 1Tx in each band. However, UL CA with UL-MIMO cannot be supported.

  - For SRS antenna switching (SRS-AS): When the UL is configured for single UL carrier in either Band n40 or Band n41, is it expected that 1T4R will create interruptions for both the UL and the DL of the other band.

Observation 3:  For low-cost architectures that support only four LNAs, the Band n41 4Rx relaxation to 2Rx is required.

Regarding the WF [1] options 1, 2, and 3:

- Based on Observation 1, and on the subject of the Band n41 MSD, it is evident that Option 1 cannot become the baseline assumption of RAN4.

- Based on Observations 2 and 3:

  - we support Option 2. Note that Option 2 does not preclude a UE implementation from supporting all current features of CA_n40A-n41A. Additional RF-FE components can be added at the expense of cost. For example, we consider an architecture that supports eight LNAs and four PAs.

  - The Band n41 4Rx requirement must relaxed to 2Rx for CA_n40-n41, with simultaneous RxTx.

  - On the subject of Option 3, we will use our meeting time to evaluate existing signalling capabilities, and address whether these capabilities do not already address methods to signal restricted support of features such as UL-MIMO, TxD, or SRS-AS.

Proposal 2: Consider WF [1] Option 2 to address CA_n40-n41 simultaneous TxRx challenges.

# Conclusion

This paper captures our views on supporting CA_n40-n41 with simultaneous RxTx.

Observation 1: With MSD ranging from 42.5 dB to 55 dB for the UL configuration “1RB+1RB”, the Band n40 MSD for CA_n40A-n41C with UL CA_n41C is the highest NR MSD that RAN4 has ever studied. Even if we consider fully allocated UL component carriers, the Band n40 MSD remains very high (31 ~ 45 dB). The Band n41 MSD due to Band n40 cross-band isolation is also very high (28.1 dB).

Proposal 1: For CA_n40A-n41C, consider adopting the PC3 Band n40 MSD test point of Table 1.

Table 1: PC3 Band n40 MSD/REFSENS for CA_n40A-n41C.

| Band / Channel Bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Band combination | NR Band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | TDD | IMD3 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 38) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 238) | 2625 |  |  |  |

Observation 2:  Even with the six LNA, two PA, four antenna architecture of WF [1], the Band n41 4Rx relaxation to 2Rx may be required in many areas of the cell, not just the cell edge.

Observation 3:  For low-cost architectures that support only four LNAs, the Band n41 4Rx relaxation to 2Rx is required.

Proposal 2: Consider WF [1] Option 2 to address CA_n40-n41 simultaneous TxRx challenges.

# References

[1]	R4-2412606 WF on simultaneous Rx-Tx for CA_n40-n41, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, , Netherlands, Huawei.

[2]	R4-2218925 Discussion simultaneous Rx/Tx inter-band combinations for CA_n40-n41, 3GPP TSG-RAN WG4 Meeting #105, Toulouse, France, Mediatek.

[3]	R4-2303691 WF on simultaneous Rx/Tx transmission, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, Huawei, HiSilicon, Skyworks Solutions,Inc, Mediatek Inc.

[4] 	R4-2306547 WF on simultaneous Rx-Tx inter-band combinations, 3GPP TSG-RAN WG4 Meeting #106-bis-e, Electronic Meeting, Huawei, HiSilicon, Skyworks Solutions Inc.

[5]	R4-2403716 WF on CA_n40A-n41C, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, ZTE Corporation, Skyworks

[6]	R4-2404172 MSD Analysis for PC3 CA_n40A-n41C with UL CA_n41, 3GPP TSG-RAN WG4 Meeting #110b, Changsha, China, Apple.

[7] 	R4-2405444 Considerations on CA_n40A-n41C, 3GPP TSG-RAN WG4 Meeting #110b, Changsha, China, Qualcomm France.

[8]	R4-2405955 CA_n40A-n41C MSD, 3GPP TSG-RAN WG4 Meeting #110b, Changsha, China, Skyworks Solutions, Inc.

[9]	R4-2407072 MSD Analysis for CA_n40A-n41C, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, Apple.

[10]	R4-2407155 CA_n40A-n41C MSD, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, Skyworks Solutions, Inc.

[11]	R4-2409316 Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, 3GPP TSG-RAN WG4 Meeting #111, Fukuoka, Japan, Huawei, HiSilicon.

[12]	R4-2412934 Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Huawei, HiSilicon.

[13]	R4-2411256 Discussion on simultaneous Rx-Tx of CA_n40-n41, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Murata Manufacturing Co Ltd.

[14] R4-2300916 CA_n40-n41 non-simultaneous RxTx PC3 PC2 MSD, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, Skyworks Solutions, Inc..