3GPP TSG-RAN WG4#112-bis	R4-2416263

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.1.1.1.1

Source: 	Qualcomm France

Title: 	MSD for HPUE

Document for:	Approval

- Introduction

Considerations on MSD for HPUE is provided in this contribution.

- WF in RAN4#112

As part HPUE of WF [1], the following was agreed for MSD:

Way forward:

- RAN4 can discuss the MSD general rules, and strive to down-select among the following options

- PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission

- Using some equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD

- Other solutions are not precluded

NOTE: RAN4 continues normal procedure specifying MSD for all PC’s band combination supports until down-selection is done

- Discussion

In this contribution we concentrate in second sub-bullet, that is to use equations or look-up tables to derive HPUE MSD from respective PC3 MSD. This contribution is written to account the possibility that RAN4 does not agree to stop specifying PC2/PC1.5 MSD’s. We remain open for that discussion; however we believe it is important to consider how HPUE MSD is specified in an ultimately convenient way if it continues to be specified as the current way of working is not efficient.

3.1 Boundary conditions for using equations/look-up tables:

We begin by describing the boundary conditions which should be met if equations/look-up tables were used.

- Enables extremely simple and low effort MSD derivation process

- Enables to specify limited amount of requirements for each UE power class

- Resolves “PC3 MSD=0dB” issue with regards to deriving PC2/PC1.5 MSD

- Maintains companies’ full freedom to provide PC3 MSD analysis based on each companies’ calculation methodology and other assumptions

The current practice for HPUE MSD’s is somewhat vague, meaning that for some combinations the proponents and/or companies working with band combinations provide rigorous analysis and then based on those the requirements are agreed. For some combinations there are multiple analysis provided by different companies, and for some other combinations there is just analysis from one company. Unfortunately, for large number of combinations there are no analysis at all meaning these HPUE combinations keep on dragging in HPUE basket without getting ready.

If HPUE MSD continues to be specified for HPUE combinations, the most important aspect is that the process to specify PC2/PC1.5 is extremely simple and low effort one. If the process is too complicated and requires challenging RF analysis/calculations, it does not serve its purpose.

We have developed an ultimately simple Excel tool for this purpose so that anyone in 3GPP can calculate PC2/PC1.5 MSD from existing PC3 MSD. As such, there is no “Magic” in the tool, instead just normal MRC combining formula is manipulated to extract PC3 interferers which result in certain MSD and the re-fed into same formula with increased interferer levels. The said “User Interface” is very simple as can be seen below. For cross-band/Harmonic Mixing/UL harmonics it uses just PC3 MSD as an input for cases where PC3 MSD is specified, and Band frequency information for cases when PC3 MSD is not specified. For IMD’s, it uses six parameters as an input; PC3 MSD, UE power class, Max power class for each band, and IMD coefficients for each band. Additionally, RAN4 needs to agree just two parameter values; PRX/DRX interference imbalance, and the additional noise into PRX in case of 2TX.

Secondly, limited number of requirements should be specified. For instance, having two set of requirements for PC2 FDD Inter-band CA is somewhat cumbersome. In PC2 TDD, the TX diversity came into specs in Rel-16 leading into situation that MSD requirements were not changed i.e. the PC2 MSD with TDD band as an aggressor continued to be specified assuming interferer goes up proportional to power increase etc. In Rel-17 the first FDD PC2 Inter-band MSD cases where specified and there two set of requirements were specified: for 1TX 26dBm and for 2TX 2*23dBm. Hence currently there is different treatment for TDD HPUE aggressor and FDD HPUE aggressor which obviously is not ideal. If HPUE MSD continues to be specified for HPUE combinations, there should be only single set of requirements for each PC which support both 1TX and 2TX implementations.

One of the challenging issues in HPUE combinations are cases when PC3 MSD is not specified i.e. there is no PC3 MSD (0dB). In our proposal, we resolve the “PC3 MSD=0dB” issue be using certain rules based on which an artificial PC3 MSD is assumed to derive PC2/PC1.5 MSD. We note that for any combination, if single band in non-CA would just meet REFSENS, there would be some small MSD present even that is no specified in 3GPP. We took a pragmatic approach here by choosing equations for PA noise and for TX leakage which set the conditions when PC2/PC1.5 MSD is specified in case PC3 is not specified i.e. is 0dB. We used several band combinations which have ~1dB or lower MSD specified as a reference when deriving the rules. Please note that these rules apply under the assumption that PC3 MSD is “correct” i.e. if PC3 MSD was omitted and is hence 0dB even it should be a lot more that the rules are not used.

3GPP RAN4 has had the very old principle of allowing companies to provide their MSD analysis based on companies’ calculation methods and other assumptions, based on which the MSD is the chosen using averaging or other means. This is still working well and should be maintained. In our proposal, freedom for any company to provide MSD evaluations using companies’ calculation methods and other assumptions fully remains for PC3 MSD.

For IMD’s, the practice has been to increase the amount or interferer proportional to power increase and IMD order per band. The consequentially, the MSD level goes very high especially for IMD4 and IMD5, which we are planning to further address in November meeting by possibly limiting the increase of interferer power to certain percentage lower than current 100% to specify more meaningful HPUE IMD4/IMD5 MSD. We invite interested companies to investigate this as well.

3.2 About PRX/DRX imbalance and additional noise into PRX due to 2TX:

PRX/DRX imbalance plays an important role in 2TX MSD value. The larger the imbalance is, the larger the 2TX MSD is compared to respective 1TX MSD. We are trying to make the process fair for all stakeholders, so we suggest using 6dB PRX/DRX imbalance. We believe that is a good compromise allowing 2TX implementations without extra burden yet still providing 2TX MSD number which is not too much larger that respective 1TX MSD hopefully allowing to just specify single MSD for each HPUE PC which is agnostic to number of TX’es. We fully acknowledge that e.g. 9dB has been used sometimes for FDD 2TX CA especially, but we believe to find a suitable compromise a smaller number is required for CA/EN-DC in order to achieve a single PC2 MSD value which is not too large compared to respective 1TX MSD. We propose to add 1dB into PRX noise in case of 2TX MSD, which is to account the coupling between TX antennas.

3.3 Explanation of the HPUE MSD tool:

- The interferences for PRX/DRX which result in the specified PC3 MSD are calculated assuming X dB difference between PRX/DRX interference. In this interference calculation the noise is, purely from simplicity perspective, to be non-correlated.

- Please note that with PRX/DRX imbalance parameter, PC3 MSD, and the assumption that Thermal noise per branch is REFSENS+1+10*LOG(2), the value of PC3 REFSENS becomes obsolete in equation.

- For 1TX PC2 cross-band/Harmonic Mixing/UL harmonics the interference is increased proportionally to the power increase, i.e. for PC2 MSD 3dB is added into PC3 PRX/DRX interferences

- For 2 TX PC2 cases, the PC3 PRX interference is increased by 1dB due to noise coupling, and same interference is used for DRX

- For PC1.5 cases, 3dB is added into respective PC2 PRX/DRX interferences

- For IMD’s the interferer level is increase proportionally to the power increase per band and the IMD coefficients. This is pending further evaluations until November

3.4 Illustration of the tool:

Below one can see how the tool looks like. It has two separate calculators, one for Cross-Band, Harmonic Mixing and UL harmonics and one for IMD’s The only things that must be used as input by the proponent are highlighted yellow color.

The Cross-Band, Harmonic Mixing and UL harmonics calculator shows both PC2 and PC1.5 MSD, for PC2 it shows both 1TX and 2TX MSD. As indicated earlier we suggest specifying only the worst case which is 2TX. The UL IMD calculator shows MSD for the power class which is used as an input parameter including the UE power per band.

In appendix, we provide several examples of the resulting PC2/PC1.5 MSD’s so that interested companies can have a look.

3.5 Next steps:

We propose the next steps in case RAN4 decides to continue specifying PC2/PC1.5 MSD. If RAN4 decides to stop specifying PC2/PC1.5 MSD then these proposals become obsolete.

Proposal 1: Agree to use the tool proposed above for deriving PC2/PC1.5 MSD.

Proposal 2: Companies to evaluate if the interferer level for PC2/PC1.5 can be scaled by less than current 100% to achieve more meaningful PC2/PC1.5 IMD4 and IMD5.

Proposal 3: Agree to capture just single worst case MSD value in TS for each PC which is agnostic to number of TX.

Proposal 4: Agree to use the following rules in deriving PC2/PC1.5 MSD for cases where PC3 MSD is 0dB:

- If one of the rules above is met, PC2 1TX MSD=0.4dB, PC2 2TX MSD=0.5dB, PC1.5 2TX MSD=0.8dB.

- If both rules above are met, PC2 1TX MSD=0.8dB, PC2 2TX MSD=0.9dB, PC1.5 2TX MSD=1.5dB.

- In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment.

Proposal 5: Agree that the assumed PRX/DRX difference is 6dB, and additional noise into PRX is 1dB due to 2 TX case for PC2/PC1.5 NR CA and EN-DC.

Proposal 6: The tool is published in 3GPP in Excel format if all proposals are agreeable.

- Conclusion

HPUE MSD was thoroughly considered in this contribution with the following proposals which are valid in case RAN4 continues to specify PC2/PC1.5 MSD for NR CA and EN-DC.

Proposal 1: Agree to use the tool proposed above for deriving PC2/PC1.5 MSD.

Proposal 2: Companies to evaluate if the interferer level for PC2/PC1.5 can be scaled by less than current 100% to achieve more meaningful PC2/PC1.5 IMD4 and IMD5.

Proposal 3: Agree to capture just single worst case MSD value in TS for each PC which is agnostic to number of TX.

Proposal 4: Agree to use the following rules in deriving PC2/PC1.5 MSD for cases where PC3 MSD is 0dB:

- If one of the rules above is met, PC2 1TX MSD=0.4dB, PC2 2TX MSD=0.5dB, PC1.5 2TX MSD=0.8dB.

- If both rules above are met, PC2 1TX MSD=0.8dB, PC2 2TX MSD=0.9dB, PC1.5 2TX MSD=1.5dB.

- In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment.

Proposal 5: Agree that the assumed PRX/DRX difference is 6dB, and additional noise into PRX is 1dB due to 2 TX case for PC2/PC1.5 NR CA and EN-DC.

Proposal 6: The tool is published in 3GPP in Excel format if all proposals are agreeable.

- Reference

[1] R4-2410605, “WF on HPUE for CA in TN”, Samsung

[2] R4-2412619, “MSD for HPUE”, Qualcomm France

- Appendix

Please find multiple different MSD cases evaluated using the tool

MSD for Cross-Band, Harmonic Mixing and UL harmonics:

MSD for IMD’s: