# 3GPP TSG-RAN WG4 Meeting #112bis                                                           R4-2416265

# Hefei, China, 14th – 20th October, 2024

Title: 	UE capability discussions for type 4 UEs

Source: 	Huawei, HiSilicon

Agenda item:	6.5.2.2

Document for:	Discussion

# Introduction

RAN4 112th meeting the WF[1] was agreed, in which some open points are left without a conclusion. In this document we are going to address them.

# Discussion

### < Issue 2-3-1: New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA > Way forward: - Define one capability for NR CA and one UE capability for EN-DC - FFS on how to differentiate type 4a and 4b.

In order to distinguish between type 4a and type 4b for EN-DC, the legacy UE capabilities that are available to indicate the number of MIMO layers per CC in E-URTRA, as explained in [2]. It should be mentioned that a distinction of type 4a and 4b for NR CA is not needed because for the bands n77, n78 the UE is required to be equipped with a minimum of four Rx antenna ports, as specified in TS 38.101-1 section 7.2. Therefore only a type 4 with 4Rx per CC needs to be considered.

Proposal 1: Use the legacy UE capabilities to distinguish between type 4a and 4b in EN-DC configuration. No type 4a/b distinction is needed for NR-CA and a type 4 with 4Rx per CC for NR CA needs to be considered

### < Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)> Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting.

As a type 4 UE is capable of operating in collocated scenarios, there is no technical advantage to move from a type 4 (with 4Rx/CC) to a type 1 with 4Rx/CC.

Proposal 2: As type 4 UE is capable of performing in collocated scenarios, there is no need to switch from type 4 to type 1 4Rx.

### < Issue 2-3-5: New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting.

Such a switching would be interesting, because the operators can gain 2 or 4 more MIMO layers/CC when they switch from type 4 to type 1 with 6 or 8 Rx in a collocated scenario, respectively. For such a switching, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too. Moreover another way would be to use the legacy NW IE maxMIMO-Layers=6 or 8 to switch from type 4 to type 1 with 6 or 8 Rx. The risk of using this legacy IE would be that if one day RAN4 decides to introduce a new type with 6 or 8Rx/CC for non-collocated deployment, using maxMIMO-Layers =6 or 8 would cause confusion between the new UE type and type 1 with 6 or 8Rx/CC.

Proposal 3:

Option1: To allow a switching from type 4a/b (default type) to type 1 with 6Rx/8Rx, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too.

Option 2: If there is a certainty that in future releases, no new non-collocated UE type (such as type 5 and ...) with 6 or 8 Rx/CC is added,  the legacy NW IE maxMIMO-Layers=6 or 8 to switch from type 4 to type 1 with 6 or 8 Rx.

### < Issue 2-3-7: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4 and BS signaling. - And then, continue further discussion on UE behavior in the next meeting.

# Conclusions

Proposal 1: Use the legacy UE capabilities to distinguish between type 4a and 4b in EN-DC configuration. No type 4a/b distinction is needed for NR-CA and a type 4 with 4Rx per CC for NR CA needs to be considered

Proposal 2: As type 4 UE is capable of performing in collocated scenarios, there is no need to switch from type 4 to type 1 4Rx.

Proposal 3:

Option1: To allow a switching from type 4a/b (default type) to type 1 with 6Rx/8Rx, RAN2 should, either introduce a new NW IE or extend nonCollocatedTypeNR-CA/ nonCollocatedTypeMRDC to make it applicable for UEs configured with maxMIMO-Layers ≤ 4, too.

Option 2: If there is a certainty that in future releases, no new non-collocated UE type (such as type 5 and ...) with 6 or 8 Rx/CC is added,  the legacy NW IE maxMIMO-Layers=6 or 8 to switch from type 4 to type 1 with 6 or 8 Rx.

# Reference

[1]	“R4-2414453, WF on Rel-19 non-collocated scenario, KDDI.”

[2]	“R4-2412439, Remaining open UE capability discussions for type 4 UEs, Huawei, HiSilicon.”