3GPP TSG-RAN Working Group 4 (Radio) Meeting #112-bis	R4-2416269

# Hefei, China, 14th – 18th October 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | <CR#> | rev | <Rev#> | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2: Addition of spatial emission requirement to protect FSS UL within band n104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In this draft CR additions to TS 38.104 included to capture the new requirement for band n104 to protect co-channel FSS UL service. The intention with this draft CR is to further stimulate the discussion in the meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In sub-clause 9.9, the BS conformance requirement and test method is added in a separated sub-clause in the transmitter section. - In Annex N, the description of EEIRP post-processing procedures are captured. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the new requirement will not be captured in BS RF core specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.9, Annex N |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.104 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See R4-2416266 för detailed discussion. There are several other clauses that need modifications in case this CR is endorsed. E.g. 4.8 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.9	OTA spatial emission

### 6.9.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

### 6.9.2	Minimum requirement

The minimum requirement for BS type 1-O and BS type 1-H is in TS 38.104 [2], subclause 9.9.2.

### 6.9.3	Test purpose

The test purpose is to verify that radiated power above the horizon does not exceed specified EEIRP levels.

### 6.9.4	Method of test

#### 6.9.4.1	Initial conditions

The test environment: Normal, see TS 38.141-2, Annex B.2.

RF channels to be tested for single carrier: B and T; see TS 38.141-2, subclause 4.9.1. It is sufficient to test only one carrier, the one characterised by all of requirements below:

- widest supported carrier bandwidth

- largest supported Power Spectral Density (PSD) level.

Beams to be tested: For each declared CAR test specified test beams, as described in TS 38.141-2, Annex N.

The test object shall be placed on the positioner with mechanical tilt set to 0 degrees.

Since this requirement is targeting the power transmitted above the horizon for a specific site installation, it is required that the BS is mounted on the positioner such that blocking effects due to the positioner are  minimized.

#### 6.9.4.2	Procedure

- Calibrate the test range, according to calibration described in TR 37.941, subclause 8.3.

- Place the test object on the positioner, such that blocking effect in the vertical domain shall be minimized.

- Align the manufacturer declared coordinate system orientation (D.2) of the test object with the coordinate system used by the test system.

- Configure the test object according to applicable test configuration in TS 38.141-2, subclause 4.8 using the corresponding test model NR-FR1-TM1.1 described in TS 38.141-1, subclause 4.9.2.

- Set the positioner at location for measurement point (θn,φm) for m=1..M and n=1..N.

- Enable test beam  k for k=1..K.

- Measure EIRPkθn,φm=EIRPp1+EIRPp2 where p1 and p2 denotes two orthogonal polarizations.

- Repeat step 6 and 7 for all K test beams.

- Repeat step 5, 6, 7 and 8 for all M and N positioner locations.

- Calculate EEIRPi profile from measured EIRPkθn,φm patterns for i=1..7 as described in TS 38.141-2, Annex N.

### 6.9.5	Test requirement

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6 425 to 7 075 MHz, shall not exceed the limits specified in table 6.9.5-1 for any declared coverage angular ranges (CAR).

Table 6.9.5-1: EEIRP limits as function of elevation above horizon

| Elevation angular range θL ≤ θ < θH (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- |
| 0<<5 | 27 |
| 5<<10 | 23 |
| 10<<15 | 19 |
| 15<<20 | 18 |
| 20<<30 | 16 |
| 30<<60 | 15 |
| 60<<90 | 15 |
| NOTE: The requirement shall apply to all supported mechanical tilts. |  |

Annex N (normative): Expected EIRP

## N.1	General

The EEIRP profile is calculated from an integration over measured EIRP patterns.

The definition of the test beams is described in N.2

The EEIRP profile can be extracted from measured EIRP spectral density radiation patterns described in clause N.2

## N.2	Definition of test beams

[TBD]

## N.3	EEIRP post-processing procedures

### N.3.1 Average EIRP pattern calculation

In the first stage, the average EIRP radiation pattern over K test beams are calculated as:

EIRPθn,φm=k=1KwkEIRPkθn,φm

, where EIRPkθn,θm  is the EIRP spectral density radiation pattern measured for the k-th test beam and wk refers to the weight for the k-th beamforming direction, i.e., the fraction of the steering range represented by the k-th beamforming direction.

### N.3.2	EEIRP bin calculation

In the second stage, average the EEIRP level for the i-th elevation range is calculated as:

EEIRPi=θHi-θLisinθHi-sinθLiMN∙n=nLinHim=1MEIRPθn,φmcosθn

where N  is the number of sampling points within the θLi and θHibounding range over upper hemisphere in the elevation range, nLiand nHi are the lowest and highest elevation sampling angles within the (θLi, θHi) bounding range, with  N= nHi-nLi; M is the number of sampling points in the azimuth range.