# 3GPP TSG-RAN WG4 Meeting #112bis                                                           R4-2416270

# Hefei, China, 14th – 20th October, 2024

Title: 	MPR evaluations for PC1.5 with TxD for intra-band contiguous CA with contiguous allocations

Source: 	Huawei, HiSilicon

Agenda item:	6.1.1.1.2

Document for:	Discussion

# Introduction

In RAN4 112 the WF[1] was approved, in which guidelines were given regarding MPR evaluations for PC1.5 in an intra-band contiguous CA configuration .

## 1.1 MPR evaluation methodology and assumption Online agreement: - MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19 - MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19

In the last meeting in [2] some MPR for contiguous CA evaluations were given. In another contribution[3], it was proposed to apply the ∆MPR of between PC2 2Tx and PC2 1Tx to the PC1.5 1Tx MPR values to obtain the corresponding MPR values for PC1.5 2Tx for intra-band contiguous and non-contiguous CA. In another document, it was proposed to study the effect of PSD imbalance in defining the MRP values for intra-band contiguous and non-contiguous CA configurations

In this document we are going to evaluate the following cases:

- PC1.5 TxD in an intra-band contiguous CA configuration:

- Contiguous allocation

- The effect of PSD imbalance between CCs on the MPR evaluations with contiguous allocations

# Discussion

The PA was calibrated based on the following assumptions:

- PC1.5 (2Tx: 26dBm+26dBm, 1LO)

- 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc, Image: 28dBc, CIM3: 60dBc, CIM5: 70dBc

- ACLR, SEM, EVM are considered as gating factors. EVM is considered for 64QAM and 256QAM modulation orders when only 1CC is being allocated

## MPR Evaluations for PC1.5 2Tx with an intra-band contiguous CA configuration: Contiguous allocations

We did MPR evaluations for the cases of 20MHz+ 20MHz and 100MHz+100MHz. The details of MPR evaluations can be found in Annex 1. In order to take into account reverse intermodulation (RIMD) and preliminary consideration of implementation margin. Therefore we propose the following MPR values which could be subjected to further modifications with further evaluations.

Table 1: Contiguous RB allocation for Power Class 1.5 with dual Tx

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer | inner | outer |
| DFT-s-OFDM | QPSK | 2.5 | 4 | 3 | 4.5 |
|  | 16QAM | 2.5 | 4.5 | 3 | 4.5 |
|  | 64QAM | 3.5 | 5 | 4 | 5.5 |
|  | 256QAM | 5.5 | 6 | 5.5 | 6 |
| CP-OFDM | QPSK | 3 | 6 | 3.5 | 6.5 |
|  | 16QAM | 3 | 6 | 3.5 | 6.5 |
|  | 64QAM | 5.5 | 6 | 6 | 6.5 |
|  | 256QAM | 7 | 7 | 7 | 7 |

It should be mentioned that due to lack of time for 100MHz+100MHz cases, except QPSK modulation, the evaluations were based on a limited set of configurations. We will try to provide exhaustive results for these cases during the meeting.

Proposal 1: Consider MPR evaluations of Table 1 for intra-band contiguous NR CA with contiguous RB allocation for PC 1.5 with dual Tx.

Observation 1: For contiguous allocations, only the case of PC1.5 with 2Tx is considered (Not considering 2 PA+2LOs). Therefore, it would be meaningful to have a common set of MPR requirements as Max(MPRclassC, MPRclassB)

## The effect of PSD imbalance between CCs on the MPR evaluations with contiguous allocations

In the last meeting it was proposed to study the effect of PSD imbalance on the intra-band contiguous NR CA with a PC1.5. MPR evaluations were performed for the following cases:

- PSD imbalance ( ∆PSD=PSDCCh- PSDCCl ):  ∆PSD = 0dB, 3dB, 6dB

- CBWCCh, CBWCCl: (5MHz,5MHz), (20MHz, 20MHz)

- Modulations: QPSK, 256QAM

- Waveform: CP-OFDM, DFT-S-OFDM

As shown in Annex2, the power imbalances of 3dB and 6dB has some effects for different allocations, however such PSD differences do not change the maximum MPR for the inner and outer regions. Therefore in contiguous allocations the evaluations can be performed with equal PSD between the two CCs, as before. .

Proposal 2: Equal PSD can be considered for intra-band contiguous NR CA with contiguous allocations, as a power imbalance ≤6dB does not change the max MPR for inner and outer regions.

# Conclusions

Proposal 1: Consider MPR evaluations of Table 1 for intra-band contiguous NR CA with contiguous RB allocation for PC 1.5 with dual Tx.

Observation 1: For contiguous allocations, only the case of PC1.5 with 2Tx is considered (Not considering 2 PA+2LOs). Therefore, it would be meaningful to have a common set of MPR requirements as Max(MPRclassC, MPRclassB)

Proposal 2: Equal PSD can be considered for intra-band contiguous NR CA with contiguous allocations, as a power imbalance ≤6dB does not change the max MPR for inner and outer regions.

# Reference

[1]	“R4-2414277, WF on HPUE for CA in TN, Samsung.”

[2]	“R4-2412349, R19 MPR for PC1.5 contiguous CA, OPPO.”

[3]	“R4-2411168, MPR for PC1.5 intra-band contiguous UL CA, Apple.”

# Annex1

# Annex2