3GPP TSG-RAN WG4 Meeting #112bis	R4-2416273

Hefei, China, 14th – 18th October, 2024

Agenda Item:	6.16.2

Source:	Ericsson

Title:	On MMSE-IRC receiver for interference mitigation with 8Rx

Document for:	Discussion

# 1	Introduction

In RAN#104 meeting, the revised WID on NR demodulation performance: Phase 5 was approved [1]. According to the WID, following are the objectives for UE part:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

Following the objectives of the WID, companies first discussed the test scope and made following agreements:

Test scope - RAN4 to introduce UE demodulation requirements for both TDD and FDD - Not to cover PDSCH absolute physical layer throughput requirements for 8Rx - Candidate options on whether to cover CQI requirements for ICI scenario: - Option 1: CQI reporting requirements under ICI should be cover for 8Rx - Option 2: No need to cover such requirements

By default, PDSCH requirements will account for both inter-cell interference and intra-cell inter-user interference, unless any technical issues are identified. To establish parameter assumptions for the test cases, it was encouraged to share initial evaluations during this meeting to assess the performance gain of the IRC receiver compared to the MMSE receiver. In this contribution, we present our perspectives along with initial evaluations to facilitate better determination of the parameter assumptions.

# 2	Inter-cell interference

In this section, we discuss the scenario of inter-cell interference cancellation with 8Rx reception.

## 2.1	PDSCH requirements with inter-cell interference

The first and most important task is to discuss and decide on the interference model and its corresponding power level.

Interference model and power level

As for the inter-cell interference model, we propose to reuse the model used for Rel-17 NR performance enhancement that contains two interfering cells for initial evaluation purpose.

In Rel-17, the interference model that RAN4 agreed to use was defined in B.6 in TS38.101-4 [2], where the interference to noise ratio (INR) is defined as follows:

Each interfering cell involved in PDSCH performance requirements with intercell interference is characterized by its associated interferer to noise ratio (INR) value: INRi=j=1NRXIor(i)(j)j=1NRXNoc(j) where Ior(i)(j) is the average received power spectral density from the i-th strongest interfering cell involved in the requirement scenario on the j-th antenna connector and Noc is the average power spectral density of a white noise source consistent with definition provided in section [4.4.3/4.5.3].

Reuse the Dominant Interferer Proportion (DIP) method that considered back in Rel-17 to determine the interference profile. Following exact INR values for two different interference cells can be reused for performing initial evaluation:

| Parameter | Unit | Interference Cell 1 | Interference Cell 2 |
| --- | --- | --- | --- |
| INR | dB | 7.77 | 2.29 |

Specifically, as described in B.6.2 in TS38.101-4 [2], the interference model can be assumed as that in each subframe, each interfering cell shall transmit 16 QAM based randomly modulated data over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel. Transmitted physical channels shall include SSB and TRS/CSI-RS as specified in requirements section.

It is always our preference to consider both HomNet and HetNet scenarios. Similar to what RAN4 demod has agreed in Rel-17 performance evolution phase 3 WI, we propose to define different test cases for HomNet and HetNet scenarios so that the test scope of RAN4 demod can for example include both 1 interference cell and 2 interference cells in the parameter assumption.

Below with respect to the open issue outlined in WF [1], we support Option 1A.

Interference modelling and power levels - Candidate options: - Option 1: Use the Intercell Interference model, specified in Clause B.6.2 of TS 38.101-4 - Option 1A: Reuse the same interference profiles (i.e., cover HomNet and Hetnet scenarios) and same power levels (i.e., INR values) defined for Rel-17 2/4Rx - Option 1B: Only consider HomNet scenario with 2 interference cells and reuse the same power levels (i.e., INR values) defined for Rel-17 2/4Rx - Option 2: RAN4 to discuss if it is still reasonable to use 16QAM based randomly modulated data for each interfering cell

Proposal 1: Consider both HomNet and HetNet scenarios and reuse the same interference profiles and same power level (i.e., INR value) defined for Rel-17 2/4Rx.

Regarding the transmission rank for the interference cells, legacy configurations are shown as follows:

| Parameter | Unit | interference Cell 1 | Interference Cell 2 |
| --- | --- | --- | --- |
| Transmission rank | % | 80% and 20% probability for rank 1 and rank 2 respectively | 80% and 20% probability for rank 1 and rank 2 respectively |

In Rel-19 with 8Rx reception, we propose to also consider rank 1 and rank 2 mixed with probabilities as specified above for the transmission rank of the interfering cells since higher rank interference tend to appear as white noise.

As for the precoding of interfering PDSCH, we propose to reuse the method that is to assume random precoding with single panel type I codebook per slot and per PRB bundling granularity, with PRB bundling size of 2.

We also propose to use 16QAM randomly modulated symbols for the modulation order of interfering PDSCH.

Proposal 2: Assume the following as the interference cell model:

- Number of interference cell: 1&2

- Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2

- Interference model: Reuse the assumption of the Rel-17 inter-cell interference requirement (as specified in B.6.2)

- SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

  - Since PDSCH is not scheduled in slot#0, no impacts to PDSCH demodulation performance even if SSB is overlapped between serving cell and interference cell(s).

- Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells

Our evaluation results in the following chapters are all based on the interference model mentioned above.

Detailed parameter assumptions

In line with the objectives, we propose using the legacy Rel-17 inter-cell interference parameter configurations as the baseline for evaluation. Adjustments can then be made, if necessary, to refine the assumptions and define the PDSCH requirements under inter-cell interference for 8Rx reception. In Rel-18, RAN4 defined UE demodulation requirements for 8Rx in both FR1 FDD and TDD. We propose extending these requirements to cover both FDD with SCS=15kHz and TDD with SCS=30kHz.

Below with respect to the open issue outlined in WF [1], we support Option 1.

Network Type and SCS/CBW - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U) - Other options are not precluded

Specifically, we suggest the following assumptions for evaluation:

Proposal 3: Consider following parameter assumptions for the SCS/CBW for inter-cell interference scenario

- Duplex CBW/SCS

  - FDD 10MHz/15kHz

  - TDD 40MHz/30kHz, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

Regarding the propagation condition, rank, MCS and antenna configuration, basically we’ve evaluated different combinations with candidates selected from Rel-17 demodulation evolution phase 3 and extended to 8Rx reception. Followings are some selected test cases that comparing the gain based on different parameter combinations.

Table 2.1-1 Initial evaluations

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain (dB) |
| 1-1 | 8 | 10 / 15 | MCS13, 16QAM, 0.48 | TDA30-10 | 8x8, ULA Low | 22.4 | 24.8 | 2.4 |
| 1-2 | 2 | 10 / 15 | MCS19 | TDLC300-100 | 2x8, ULA medium B | 28.77 | N/A | Inf |
| 1-3 | 4 | 10 / 15 | MCS17 | TDA30-10 | 4x8, ULA low | 12.8 | 18.1 | 5.3 |
| 1-4 | 8 | 10 / 15 | MCS17 | TDA30-10 | 8x8, ULA low | N/A | N/A | N/A |
| 2-1 | 2 | 10 / 15 | MCS13 | TDLA30-10 | 2x8, ULA Low | 2.8 | 7.8 | 5.0 |
| 2-2 | 8 | 10 / 15 | MCS13 | TDLA30-10 | 8x8, ULA Low | 19.6 | 23.4 | 3.8 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |

From the simulation results above, we can observe that it’s difficult to see the gain configured with MCS17 and rank 8 (Test 1-4). Results with MCS13 (Tests 1-1 and 2-2) demonstrate highly stable gain, and the SNR point required to achieve 70% of maximum throughput is lower compared to MCS19 and MCS17.

Observation 1: It is difficult to see the gain configured with MCS17 and rank 8

Observation 2: Results with MCS13 demonstrate highly stable gain, and the SNR point required to achieve 70% of maximum throughput is lower compared to MCS19 and MCS17

Below with respect to the open issues outlined in WF [1], we support Option 1 for the propagation condition, option 1C for the rank, option 3 for the MCS and option 3 for the Antenna.

Propagation condition - Candidate options: - Option 1: Reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet) - Option 2: Consider TDLA30-10 as the starting point Rank - Candidate options: - Option 1: Consider higher rank for 8Rx - Option 1A: RAN4 to investigate rank 1, 2, 4 and choose a reasonable requirement value - Option 1B: Rank 2 and Rank 4 as the starting point - Option 1C: Rank 2 and Rank 4 and Rank 8 - Option 1D: Rank 4 - Option 1E: Consider 2 layers, 4 layers and 6 layers - Option 2: Same as Rel-17 2/4Rx, i.e., Rank 1 MCS - Candidate options: - Option 1: Reuse the same configuration for Rel-17 2/4Rx, i.e., MCS 13 - Option 2: MCS 16 - Option 3: MCS 19 for rank 2, MCS 17 for rank 4 and 8 - Option 4: Define different parameters for FWA, Vehicular and Industrial deployments respectively Antenna and MIMO correlation - Candidate options: - Option 1: 2T8R ULA low - Option 2: 4T8R - Option 3: - 2T8R ULA medium B - 4T8R ULA low - 8T8R ULA low

We proposed the following parameter assumptions:

Proposal 4:

- Antenna configuration

ULA low

- Rank

  - Rank 2

  - Rank 4

  - Rank 8

- Propagation condition

  - TDLA30-10

- MCS

  - MCS13

More evaluation results can be found in our companion simulation result paper [2].

Test applicability can be discussed after the test cases have been decided. Our preference is to apply the new requirements for 8Rx from Rel-18.

## 2.3	CQI reporting

As indicated in the objective of WID, we think it’s also necessary to define CQI reporting requirement for inter-cell interfering cancellation using 8Rx MMSE-IRC receiver. With this new requirement, the UE’s ability of tracking the channel variations and selecting the largest transport format possible using 8Rx reception under the inter-cell interference condition can be tested.

In Rel-17, RAN4 defined CQI reporting test in fading condition for inter-cell interference scenario. We therefore propose to define CQI reporting test in fading condition for 8Rx UE as well. Regarding the interference model, we propose to reuse following assumptions as a starting point:

- One interfering cell

- CSI-IM overlaps with PDSCH from interference

- NZP CSI-RS on target cell overlaps with NZP CSI-RS from interference

We suggest reusing the test metric as well:

- The ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified DIP and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ γ;

- when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

We propose to consider the assumption assumed in Rel-17 CQI reporting fading tests with inter-cell interference as baseline, e.g., 6.2.3.1.2.3 (4Rx) in TS38.101-4 [2]

Proposal 5: Consider the assumption assumed for Rel-17 CQI reporting fading tests with inter-cell interference as baseline

# 3	Intra-cell inter-user interference

In this section, we discuss the scenario of intra-cell inter-user interference cancellation with 8Rx reception.

## 3.1	PDSCH requirements with intra-cell inter-user interference

As for the intra-cell inter-user interference model, we propose to reconsider the model used for Rel-17 NR performance enhancement. Following aspects are within the pool of priority that need to be discussed so that to determine the overall interference model.

Number of paired UEs

Reuse the legacy pairing UE model to set up one target UE and one co-scheduled (interfering) UE. Since we are taking MMSE-IRC receiver as the baseline receiver, the target UE does not need to know the number of co-scheduled UEs and their ranks when it is processing in demodulation. Based on this, we propose to consider only 1 co-scheduled UE but with different layers in different test cases.

Proposal 6: Consider only 1 co-schedule UE in the interference model

Rank for target and interference PDSCH

Previously in Rel-17 intra-cell interference discussion, RAN4 agreed to consider rank combination 1+1 for 2Rx and 2+2 for 4Rx cases. As we are considering 8Rx reception, it’s possible to have additional rank 4 and rank 8 configurations on top of rank 2 configuration of Rel-17. Considering the real network deployment scenario, it’s also likely to configure a hybrid rank combination for target and interfering UE(s) if the Network can hardly find two paired UEs with the same rank configuration. Thus, we propose to evaluate following two possible rank combinations and further check the results before making any down-selection for the purpose of introducing requirements.

Proposal 7: Consider following rank configuration for initial evaluations

- For 2 layers

  - Target UE + interference UE:

    - 1+1

- For 4 layers

  - Target UE + interference UE:

    - 2+2

- For 8 layers

  - Target UE + interference UE:

    - 2+6

    - 4+4

Antenna configuration

As indicated in the objectives of the WID, new PDSCH requirement will be introduced for 8Rx UEs. Similar to the antenna configuration used in Rel-18 UE RF enh WI, we propose to consider 2Tx, 4Tx and 8Tx so that the test case design is suggested to be based on 2T8R, 4T8R and 8T8R.

Proposal 8: Consider following antenna configurations:

- 2T8R

  - For rank 1+1

- 4T8R

  - For rank 2+2

- 8T8R

  - For rank 2+6 and/or 4+4

Codebook type

As for the codebook type, we prefer to reuse the assumption of Rel-17 intra-cell interference cancellation that is Type I single panel codebook so that the precoding configuration can be assumed as randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [3].

Proposal 9: Consider Type I Single Panel only. For 8Tx scenario, choose one configuration, e.g., (N1, N2, O1, O2) = (4, 1, 4, 1)

PMI selection and precoding matrix generation

We think the precoding method used for Rel-18 NR demodulation performance evolution WI can be reconsidered, which is stated as follows:

- Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH

Proposal 10: Reuse the PDSCH & PDSCH DMRS Precoding configuration of Rel-18 NR demodulation performance evolution WI

PRB bundling size and precoding granularity

As for the PRB bundling size and precoding granularity, we prefer to 2PRBs for frequency domain and per slot for time domain, which is the same assumption as the existing UE demodulation requirements.

Proposal 11: 2PRBs for PRB bundling size and precoding granularity

MCS for interfering PDSCH

We propose to reuse the assumption for Rel-17 NR demodulation performance evolution WI which is the following:

- Random 16QAM symbols

Proposal 12: Assume Random 16QAM symbols for co-scheduled UE

Detailed parameter assumptions

Following the objective, we propose to take the legacy Rel-17 intra-cell inter-user interference parameter as the baseline and do the evaluation, then update it with proper parameter assumptions if needed for introducing PDSCH requirement with inter-cell interference with 8Rx reception. We propose to consider following parameter assumptions as a starting point:

Proposal 13: Consider following parameter assumptions for intra-cell inter-user interference scenario

- Duplex CBW/SCS

  - For FDD 15kHz SCS: Cover 10MHz CBW

  - For TDD 30kHz SCS: Cover 40MHz CBW, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- Antenna configuration

  - 2T8R ULA low

  - 4T8R ULA low

  - 8T8R ULA low

- MCS for target UE for evaluation

  - MCS17, MCS13

- Channel model

  - TDLA30-10

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

In the meantime, we’ve evaluated some cases that mostly based on our proposed parameter assumptions above. Simulation results have been shared below:

Table 3.1-1 Initial evaluations

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 1-1 | 1+1 | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLC300-100 | 2x8, ULA Low | 2.6 | 3.7 | 1.1 |
| 1-2 | 2+2 | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLC300-100 | 4x8, ULA Low | 5.6 | 13.0 | 7.4 |
| 1-3 | 4+4 | 10 / 15 | 16QAM, 0.48 | Random 16QAM symbols | TDLC300-100 | 8x8, ULA Low | 13.9 | N/A | Inf |
| 1-4 | 1+1 | 10 / 15 | MCS19 | Random 16QAM symbols | TDLC300-100 | 2x8, ULA Medium B | 13.4 | N/A | Inf |
| 1-5 | 2+2 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 9.2 | N/A | Inf |
| 1-6 | 4+4 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | 19.2 | N/A | Inf |

From the results above, we observe that MCS13 under TDLC300-100 propagation conditions achieves significant gain, particularly for rank combinations 2+2 and 4+4. Similarly, MCS19 and MCS17 also demonstrate notable gains over the reference receiver, while maintaining relatively low SNR levels at 70% of maximum throughput.

Observation 3: MCS13 under TDLC300-100 propagation conditions demonstrates significant performance gains, particularly in rank combinations 2+2 and 4+4

Observation 4: Both MCS19 and MCS17 achieve substantial improvements over the reference receiver, with relatively low SNR requirements at 70% of maximum throughput, indicating its feasibility

More evaluation results can be found in our companion simulation result paper [2].

# 4	Summary

In this contribution, we shared our views on how to introduce PDSCH and CQI reporting requirement for two scenarios. Our proposals are summarized as follows:

Proposal 1: Consider both HomNet and HetNet scenarios and reuse the same interference profiles and same power level (i.e., INR value) defined for Rel-17 2/4Rx.

Proposal 2: Assume the following as the interference cell model:

- Number of interference cell: 1&2

- Frequency shift to the target cell: 300Hz for interference cell 1, -100Hz for interference cell 2

- Interference model: Reuse the assumption of the Rel-17 inter-cell interference requirement (as specified in B.6.2)

- SSB configuration: First SSB in Slot #0 for all interference cells (Same as serving cell)

  - Since PDSCH is not scheduled in slot#0, no impacts to PDSCH demodulation performance even if SSB is overlapped between serving cell and interference cell(s).

- Transmission rank: Random rank with 70% and 30% probability for rank 1 and rank 2 for all interference cells

Proposal 3: Consider following parameter assumptions for the SCS/CBW for inter-cell interference scenario

- Duplex CBW/SCS

  - FDD 10MHz/15kHz

  - TDD 40MHz/30kHz, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

Observation 1: It is difficult to see the gain configured with MCS17 and rank 8

Observation 2: Results with MCS13 demonstrate highly stable gain, and the SNR point required to achieve 70% of maximum throughput is lower compared to MCS19 and MCS17

Proposal 4:

- Antenna configuration

ULA low

- Rank

  - Rank 2

  - Rank 4

  - Rank 8

- Propagation condition

  - TDLA30-10

- MCS

  - MCS13

Proposal 5: Consider the assumption assumed for Rel-17 CQI reporting fading tests with inter-cell interference as baseline

Proposal 6: Consider only 1 co-schedule UE in the interference model

Proposal 7: Consider following rank configuration for initial evaluations

- For 2 layers

  - Target UE + interference UE:

    - 1+1

- For 4 layers

  - Target UE + interference UE:

    - 2+2

- For 8 layers

  - Target UE + interference UE:

    - 2+6

    - 4+4

Proposal 8: Consider following antenna configurations:

- 2T8R

  - For rank 1+1

- 4T8R

  - For rank 2+2

- 8T8R

  - For rank 2+6 and/or 4+4

Proposal 9: Consider Type I Single Panel only. For 8Tx scenario, choose one configuration, e.g., (N1, N2, O1, O2) = (4, 1, 4, 1)

Proposal 10: Reuse the PDSCH & PDSCH DMRS Precoding configuration of Rel-18 NR demodulation performance evolution WI

Proposal 11: 2PRBs for PRB bundling size and precoding granularity

Proposal 12: Assume Random 16QAM symbols for interfering PDSCH

Proposal 13: Consider following parameter assumptions for intra-cell inter-user interference scenario

- Duplex CBW/SCS

  - For FDD 15kHz SCS: Cover 10MHz CBW

  - For TDD 30kHz SCS: Cover 40MHz CBW, TDD DL/UL configuration for 30kHz SCS: 7D1S2U(S=6D+4G+4U)

- Antenna configuration

  - 2T8R ULA low

  - 4T8R ULA low

  - 8T8R ULA low

- MCS for target UE for evaluation

  - MCS17, MCS13

- Channel model

  - TDLA30-10

- PDCCH and PDSCH allocation in time domain

  - Use symbols #0 and #1 of each slot for PDCCH

  - PDSCH mapping type A, Start symbol 2, Duration 12

- PDSCH allocation in frequency domain

  - Full PRB

Observation 3: MCS13 under TDLC300-100 propagation conditions demonstrates significant performance gains, particularly in rank combinations 2+2 and 4+4

Observation 4: Both MCS19 and MCS17 achieve substantial improvements over the reference receiver, with relatively low SNR requirements at 70% of maximum throughput, indicating its feasibility

# References

[1]	RP-241297: WF on NR demodulation performance: Phase 5, China Telecom, RAN4 #112, Maastricht, Netherlands

[2]	R4-2416274: Initial simulation results for IRC with inter-cell and intra-cell interference, Ericsson, RAN4 #112bis, Hefei, China