3GPP TSG-RAN WG4 Meeting #112-bis 	R4-2416278

Hefei, China, Oct 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	DraftCR to 38.141-2 – Test procedure

Agenda Item:	6.11.4

Document for:	Approval

1 Introduction

In RAN4#112 the WF [1] was agreed and allocated the drafting of the conformance parts of the EEIRP requirement to different companies.

This paper covers the test procedure part of 38.141-2.

2 Discussion

The draft CR text below has a number of open issues associated with the WF [1] from last meeting.

Issue 4-2: The EEIRP test beam set is yet to be agreed

We have been referring to this as test vectors but have been told this is confusing, “test beams” was suggested. “EEIRP test beam set” seems like a suitable name. Possibly this could be added to the definitions?

Issue 4-3: Measurement grid -  for the TS suggest using the same approach as for TRP and refer to the annex for the correct measurement grid settings.

Issue 4-4: RF channels – still an open issue, currently in square brackets in the draft CR text.

Issue 4-7: Measurement uncertainty, still not agreed but does not affect the test requirements as its been agreed that TT =0. MU values are captured elsewhere.

There also remain open issues about how the “steering range” is captured in the declarations, the declarations sections are captured by another sub author but obviously the terms need to be aligned. This draft CR text has been written based on using the existing OTA peak directions set framework with modifications to the declaration D.10 as follows:

| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the BS is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. For BS required to meet EEIRP requirements in sub-clause 6.9. the following beams must also be declared 6) A beam which meets the EEIRP requirements with minimum mechanical down tilt and the maximum electrical steering range. If the maximum steering range and the minimum mechanical down tilt are not simultaneously supported following two instances: - Minimum mechanical down tilt and the reduced electrical steering range - Maximum electrical steering range and increased mechanical down tilt (Note 3) |
| --- | --- | --- |

In addition it is assumed a maximum PSD is declared as follows:

| D.xx | EEIRP rated carrier OTA BS PSD, Prated,c,PSD | Prated,c,PSD is declared as TRP OTA PSD per carrier, declared for frequencies where the EEIRP requirement applies. |
| --- | --- | --- |

References

[1]	R4-2413509 WF for [112][305] NR_BS_RF– ZTE

Annex A – draft CR to 38.141-2 v18.6.0

<start of changes>

3.2	Symbols

For the purposes of the present document, the following symbols apply:

	Percentage of the mean transmitted power emitted outside the occupied bandwidth on the assigned channel

BeWθ	The beam width in θ

BeWφ	The beam width in ϕ

BeWθ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the θ-axis in degrees, applicable for FR1 only

BeWφ,REFSENS	The beamwidth equivalent to the OTA REFSENS RoAoA in the φ-axis in degrees, applicable for FR1 only

BWChannel	BS channel bandwidth

BWChannel_CA	Aggregated BS channel bandwidth, expressed in MHz. BWChannel_CA = Fedge_high- Fedge_low

BWConfig	Transmission bandwidth configuration, expressed in MHz, where BWConfig = NRB x SCS x 12

BWContiguous	Contiguous transmission bandwidth, i.e. BS channel bandwidth for single carrier or aggregated BS channel bandwidth for contiguously aggregated carriers. For non-contiguous operation within a band the term is applied per sub-block.

BWtot	Total RF bandwidth

f	Separation between the channel edge frequency and the nominal -3 dB point of the measuring filter closest to the carrier frequency

fmax	f_offsetmax minus half of the bandwidth of the measuring filter

ΔFGlobal	Global frequency raster granularity

ΔfOBUE	Maximum offset of the operating band unwanted emissions mask from the downlink operating band edge

ΔFR2_REFSENS	Offset applied to the FR2 OTA REFSENS depending on the AoA

ΔfOOB	Maximum offset of the out-of-band boundary from the uplink operating band edge

ΔminSENS	Difference between conducted reference sensitivity and EISminsens

ΔOTAREFSENS	Difference between conducted reference sensitivity and OTA REFSENS

ΔFRaster	Channel raster granularity

Δsample	The difference between the nominal and extreme power measurements during extreme EIRP testing, Pmax,sample,nom - Pmax,sample,ext

ΔSUL	Channel raster offset for SUL

EISminsens	The EIS declared for the minSENS RoAoA

EISREFSENS	OTA REFSENS EIS value

EISREFSENS_50M	Declared OTA reference sensitivity basis level for FR2 based on a reference measurement channel with 50MHz BS channel bandwidth

FFBWhigh	Highest supported frequency within supported operating band, for which fractional bandwidth support was declared

FFBWlow	Lowest supported frequency within supported operating band, for which fractional bandwidth support was declared

FC	RF reference frequency on the channel raster

FC,block, high	Fc of the highest transmitted/received carrier in a sub-block

FC,block, low	Fc of the lowest transmitted/received carrier in a sub-block

FC_low	The Fc of the lowest carrier, expressed in MHz

FC_high	The Fc of the highest carrier, expressed in MHz

Fedge_low	The lower edge of Aggregated BS channel bandwidth, expressed in MHz. Fedge_low = FC_low - Foffset_low

Fedge_high	The upper edge of Aggregated BS channel bandwidth, expressed in MHz. Fedge_high = FC_high + Foffset_high

Fedge,block,low	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset_low

Fedge,block,high	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset_high

Foffset_high	Frequency offset from FC_high to the upper Base Station RF Bandwidth edge, or from F C,block, high to the upper sub-block edge

Foffset_low	Frequency offset from FC_low to the lower Base Station RF Bandwidth edge, or from FC,block, low to the lower sub-block edge

f_offset	Separation between the channel edge frequency and the centre of the measuring filter

f_offsetmax	The offset to the frequency ΔfOBUE outside the downlink operating band

FREF	RF reference frequency

FREF,SUL	RF reference frequency for Supplementary Uplink (SUL) bands

Fstep,X	Frequency steps for the OTA transmitter spurious emissions (Category B)

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Iuant	gNB internal logical interface between the implementation specific O&M function and the RET antennas and TMAs control unit function of the gNB

Ncells	The declared number corresponding to the minimum number of cells that can be transmitted by an BS type 1-H in a particular operating band

NRB	Transmission bandwidth configuration, expressed in resource blocks

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

NRXU,active	The number of active receiver units. The same as the number of demodulation branches to which compliance is declared for chapter 8 performance requirements

PEM,n50,ind	Declared emission level for Band n50 in the band 1518-1559 MHz; ind = a, b

Pmax,c,EIRP	The maximum carrier EIRP when the BS is configured at the maximum rated carrier TRP (Prated,c,TRP)

Pmax,c,EIRP, extreme	The maximum carrier EIRP when the BS is configured at the maximum rated carrier TRP (Prated,c,TRP) under extreme conditions, either measured directly or calculated

Pmax,c,TRP	Maximum carrier TRP measured at the RIB(s), and corresponding to the declared rated carrier TRP (Prated,c,TRP)

Pmax,sample,nom	The measured sample power in extreme conditionals chamber when the BS is configured at the rated carrier TRP (Prated,c,TRP), under nominal conditions

Pmax,sample,ext	The measured sample power in extreme conditionals chamber when the BS is configured at the rated carrier TRP (Prated,c,TRP), under extreme conditions

Prated,c,EIRP	The rated carrier EIRP when the BS is configured at the rated carrier TRP (Prated,c,TRP)

Prated,c,FBWhigh	The rated carrier EIRP for the higher supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,FBWlow	The rated carrier EIRP for the lower supported frequency range within supported operating band, for which fractional bandwidth support was declared

Prated,c,PSD	The rated carrier maximum power spectral density where EEIRP requireet apply.

Prated,c,TRP	Rated carrier TRP declared per RIB

Prated,t,TRP	Rated transmitter TRP declared per RIB

PREFSENS	Conducted reference Sensitivity power level

SSREF	SS block reference frequency position

TRPEstimate	Numerically approximated TRP

TTOTA	Test tolerance for OTA requirements

Wgap	Sub-block gap or Inter RF Bandwidth gap size

The angle in the reference coordinate system between the projection of the x/y plane and the radiation vector defined between -90° and 90°. 0° represents the y/z plane. The angle is aligned with the down-tilt angle

The angle in the reference coordinate system between the x-axis and the projection of the radiation vector onto the x/y plane defined between -180° and 180°

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AA	Antenna Array

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

AoA	Angle of Arrival

AWGN	Additive White Gaussian Noise

BS	Base Station

BW	Bandwidth

CA	Carrier Aggregation

CACLR	Cumulative ACLR

CATR	Compact Antenna Test Range

CPE	Common Phase Error

CP-OFDM	Cyclic Prefix-OFDM

CLTA	Co-Location Test Antenna

CW	Continuous Wave

DFT-s-OFDM	Discrete Fourier Transform-spread-OFDM

DM-RS	Demodulation Reference Signal

EUT	Equipment Under Test

EIRP	Equivalent Isotropic Radiated Power

EIS	Equivalent Isotropic Sensitivity

FBW	Fractional Bandwidth

FR	Frequency Range

GSCN	Global Synchronization Channel Number

ICS	In-Channel Selectivity

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LA	Local Area

LNA	Low Noise Amplifier

MR	Medium Range

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OBUE	Operating Band Unwanted Emissions

OCC	Orthogonal Covering Code

OSDD	OTA Sensitivity Directions Declaration

OTA	Over The Air

PT-RS	Phase Tracking Reference Signal

PSD	Power Spectral Density

PWS	Plane Wave Synthesizer

RB	Resource Block

RDN	Radio Distribution Network

REFSENS	Reference Sensitivity

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RS	Reference Signal

RV	Redundancy Version

RX	Receiver

RoAoA	Range of Angles of Arrival

SCS	Sub-Carrier Spacing

SSB	Synchronization Signal Block

TAB	Transceiver Array Boundary

TAE	Time Alignment Error

TDD	Time Division Duplex

TDL	Tapped Delay Line

TRP	Total Radiated Power

TT	Test Tolerance

UCI	Uplink Control Information

ZF	Zero Forcing

<Next change>

  - 6.9	OTA spatial emission

6.9.1	Definition and applicability

OTA spatial emission requirements are defined to set upper limits on radiated power in specific directions.

6.9.2	Minimum requirement

The minimum requirement for BS type 1-O operation is defined in TS 38.104 [2], clause 9.9.2.

The OTA spatial emission requirement is not applicable for BS type 2-O.

6.9.3	Test purpose

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to radiated power in specified elevation angle ranges with respect to the horizon.

6.9.4	Method of test

6.9.4.1	Initial conditions

Test environment: normal; see annex B.2.

RF channels to be tested for single carrier:	[M] for the applicable frequency band over which EEIRP requirements apply; see clause 4.9.1.

Directions to be tested: [For the beam identifiers (D3) which apply to the EEIRP requirement (D3_5, D3_6) and the associated OTA peak directions set reference beam direction pair (D8) and OTA peak directions set maximum steering direction(s) (D10) the EEIRP test beam set is defined as (θ,φ):]

θD8-0.66(θD8-θD10_4), φD8-0.66(φD8-φD10_2θD8-0.66(θD8-θD10_4), φD8θD8-0.66(θD8-θD10_4),φD8-0.66(φD8-φD10_1)θD8, φD8-0.66(φD8-φD10_2)θD8, φD8θD8,φD8-0.66(φD8-φD10_1)θD8-0.66(θD8-θD10_3), φD8-0.66(φD8-φD10_2)θD8-0.66(θD8-θD10_3), φD8θD8-0.66(θD8-θD10_3),φD8-0.66(φD8-φD10_1)

Where: D10_1 is Declaration D10 beam direction pair point 1), D10_2 is Declaration D10 beam direction pair point 2), D10_3 is Declaration D10 beam direction pair point 3), and D10_4 is Declaration D10 beam direction pair point 4),

6.9.4.2	Procedure

The following procedure for measuring EEIRP is based on directional power measurements as described in annex M.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	Measurements shall use a measurement bandwidth in accordance to the conditions in clause 6.9.5.

4)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

-	The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1.

5)	Set the BS to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test model in clause 4.9.2 at manufacturer's declared rated PSD (D.xx) Prated,c,PSD.

6)	[Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine EEIRP can be performed (see annex M).

7)	Sweep the beam direction over the directions required by the EEIRP test beam set in sub-clause 6.9.4.1.

8)	Measure the emission at the specified frequencies with specified measurement bandwidth.

9)	Repeat step 6-8 for all measurement directions in the appropriated EEIRP measurement grid (see annex M), and all beam steering directions in the steering test vector.]

NOTE 1:	The EEIRP measurement grid may not be the same for all measurement frequencies.

NOTE 2:	The frequency sweep, the EEIRP measurement grid sweep or the beam steering directions sweep may be done in any order.

10)	Calculate EEIRP for each elevation angular range, applying the mechanical tilt offsets if necessary at each specified frequency using the directional measurements.

6.9.5	Test requirements

6.9.5.1	Requirement for BS type 1-O

For BS type 1-H and BS type 1-O operating in band n104, the Expected EIRP (EEIRP) in the frequency range 6425 – 7075 MHz , the power measured in step 10 shall not exceed the values specified in table 6.9.5.1-1.

Table 6.9.5.1-1: EEIRP limits as function of elevation above horizon

| Elevation angular range [θL ≤ θ < θH] (Degrees) | EEIRP limit (dBm/MHz) |
| --- | --- |
| 0<<5 | 27 |
| 5<<10 | 23 |
| 10<<15 | 19 |
| 15<<20 | 18 |
| 20<<30 | 16 |
| 30<<60 | 15 |
| 60<<90 | 15 |
| Note 2: The requirement shall apply to all supported mechanical tilts. |  |

<Next change>