3GPP RAN WG4 Meeting #112bis		R4-2416283

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	Huawei, HiSilicon

Title: 	RF requirements for Ambient IoT BS

Agenda Item:	6.21.3.4

Document for:	Approval

1 Introduction

The new SID on solutions for Ambient IoT was approved at the RAN#102 meeting [1] and later revised [2] at the RAN#103 meeting. The RAN4-led objectives are copied as below:

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

In this contribution, we provide discussion on Ambient IOT BS RF requirements.

2 Discussion

Agreement in RAN4#111:

- To further investigate output power, emission requirements for CW node

  - FFS for other requirements.

- Agreement in RAN4#112:

  - FFS on methods and feasible values for CW cancellation

The CW cancellation capability can be divided into 3 aspects: (1) Spatial isolation; (2) Analog cancellation and (3) Digital cancellation.

Spatial isolation

If the carrier-wave source is separately deployed from the D2R receiver (e.g., D1T1-A1 and D1T1-B scenarios), the spatial isolation relies on the distance between the CW node and A-IOT node. For example, the path loss is 57 dB for an isolation distance of 20 meters, assuming free-space propagation. Normally the CW node and A-IOT node deploy at the same height, the antenna gain is about -5 dB at 90°offset from the boresight for both TX antenna and RX antenna. For D1T1-B scenarios the received power at the antenna connector is 23-57-10= -44 dBm and 23-51-10= -38 dBm for 20 meters and 10 meters distance respectively.

Analog cancellation

Interference cancellation in the RF front-end can be applied especially when the interference power exceeds the blocking threshold of receiver. In principle, it can be implemented by reconstructing a local carrier-wave according to the received interference, and subtracting it from the received signal. The performance heavily depends on the similarity between the reconstructed and actual received interference. Consequently, the optimal waveform for carrier-wave is the one which can be easily reconstructed according to the received carrier-wave interference. A suppression of 10-30 dB can usually be expected for RF interference cancellation. Assuming different reference clock between the carrier-wave transmitter and D2R receiver (e.g., in D1T1-B scenario without wired connection between the basestation and the carrier-wave node), 10 dB or higher suppression can be achieved after an initial frequency calibration by e.g. Hz-level CFO estimation based on the received carrier-wave interference. Assuming the same reference clock between the carrier-wave transmitter and D2R receiver (e.g., in D1T1-A1/A2 scenario, or D1T1-B scenario with wired connection between the basestation and the carrier-wave node), higher interference suppression can be reached, such as 20 dB or higher.

Digital cancellation

In the digital baseband, a significant suppression can be simply achieved by high-pass filtering, as the received single-tone carrier-wave is shifted to direct current (DC) at baseband. Figure 6 shows the amplitude-frequency response of a 33-order FIR high-pass filter. Regarding the bitwidth of 11-bits or 12-bits for the ADC of the D2R receiver, oversampling is an effective way to increase the resolution or equivalent bitwidth of ADC. As a general guideline, oversampling the ADC by a factor of four provides one additional bit of resolution, or a 6 dB increase in dynamic range. For example, the sampling rate of the ADC usually exceeds 100 Msps. As the sampling rate is far beyond the transmission bandwidth of the D2R signal, the equivalent dynamic range is able to cover the interference suppression of 80 dBc for the digital high-pass filter.

Figure: Carrier-wave interference suppression at Ambient IoT uplink receiver

Overall carrier-wave interference

In the case of separated carrier-wave transmitter and D2R receiver in D1T1-A1 or D1T1-B, through spatial isolation, RF interference cancellation and high-pass filtering, the overall suppression to the carrier-wave interference can reach about (61+10+80) = 151 dB at 10 meters distance. For larger spatial isolation corresponding to longer distance between the carrier-wave transmitter and D2R receiver (e.g., ~60 dB @ 25 m), the carrier-wave interference can be even higher (e.g., 70+10+80 = 160 dB).

In the case of integrated carrier-wave transmitter and D2R receiver in D1T1-A2, with ~40 dB interference suppression provided by a circulator/directional coupler and/or the antenna with high isolation between Tx and Rx, the carrier-wave interference suppression can reach about (40+20+80) = 140 dB.

|  | CW power | CW cancellation capability | Residual CW interference | Thermal Noise power (15KHz) | baseline IN | Degradation |
| --- | --- | --- | --- | --- | --- | --- |
| D1T1-A1/B | 23 | 150 | -127 | -122.2390874 | -120.9871228 | 1.251964646 |
| D1T1-A2 | 33 | 140 | -107 | -122.2390874 | -106.8719274 | 15.36716 |

3	Conclusion

This contribution provides Text Proposals on Ambient CW cancellation.

Proposal 1: it is proposed to endorse the attached TP for TR 38.769 clause 6.7.5.

References

[1] RP-240829, New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing, RAN4 Chair (Huawei)

[2] R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei

TP for TR 38.769

6.7.5	CW interference cancelation

The CW cancellation capability can be divided into 3 aspects: (1) Spatial isolation; (2) Analog cancellation and (3) Digital cancellation.

Spatial isolation

If the carrier-wave source is separately deployed from the D2R receiver, the spatial isolation relies on the distance between the CW node and A-IOT node. For example, the path loss is 57 dB for an isolation distance of 20 meters, assuming free-space propagation. Furthermore, some beam isolation can be achieved since the interference direction is far from the boresight.

Analog cancellation

Interference cancellation in the RF front-end can be applied especially when the interference power exceeds the blocking threshold of receiver. In principle, it can be implemented by reconstructing a local carrier-wave according to the received interference, and subtracting it from the received signal. The performance heavily depends on the similarity between the reconstructed and actual received interference. Consequently, the optimal waveform for carrier-wave is the one which can be easily reconstructed according to the received carrier-wave interference. A suppression of 10-30 dB can usually be expected for RF interference cancellation.

Digital cancellation

In the digital baseband, a significant suppression can be simply achieved by high-pass filtering, as the received single-tone carrier-wave is shifted to direct current (DC) at baseband.

For D1T1, the CW cancellation analysis summary is shown in Table 6.7.5-1.

Table 6.7.5-1: CW cancellation analysis summary for D1T1

| Source | CW cases | Spectrum | CW TX power | Spatial domain | Analog domain | Digital Domian | Others | Blocking level at LNA input | Total IC capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R4-24xxxxx | D1T1-A1/B | Band 8 UL | 23 | 60 | 10 | 80 | n/a | -47 | 150 |
| R4-24xxxxx | D1T1-A2 | Band 8 DL | 33 | 40 | 20 | 80 | n/a | -27 | 140 |

For D2T2, the CW cancellation analysis summary is shown in Table 6.7.5-2.

Table 6.7.5-2: CW cancellation analysis summary for D2T2

| Source | CW cases | Spectrum | CW TX power | Spatial domain | Analog domain | Digital Domian | Others | Blocking level at LNA input | Total IC capability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |