3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416287

# Hefei, China, 14th - 18th, Oct 2024

Source: 	Samsung, TELUS, Bell mobility

Title: 	Discussion on UE architecture and PSD difference for fragmented carriers

Agenda Item:	6.6.2

Document for:	Discussion

- Introduction

In RAN#103, a SID was approved [1] with following objectives on the study of how to process the fragmented carriers with one Rx Chain. Initial discussion was organized in RAN4#112 and the agreements are captured in [9].

We further present our consideration in this paper.

- 2. Discussion

2.1 SID clarification

The agreement reached in last meeting is excerpted as below. As stated, the two non-contiguous CCs is still under current CA framework which implies “single cell multi carriers” concept is not considered thus no RAN1 impact foreseen.

It has been argued a lot during online that the scope for study should be the bands with frequency span within 100MHz which means n41/n77/n78/n79 are precluded, or the fragment carriers being fully confined within 100MHz. The latter got more supporters though the SID says the former. We are fine with the latter interpretation (i.e., fragments being fully confined within 100MHz) as long as the study focuses on the bands ≤2700MHz. In addition, we are wondering if operators have such case that 100MHz are allocated for two operators with fragmented carriers, for the bands ＞2700MHz.

Proposal 1: The scope should be those bands, where the fragments are fully confined within 100MHz, and with frequency ≤2700MHz.

2.2 UE RF architecture

One issue left open is the referenced UE architecture. According to TR 36.823 [3] Figure 6.2.2.1-1, the Rx requirements in current spec for collocated DL intra-band NC CA was specified based on “Partially shared Rx Chain” architecture, a guess is that this architecture was selected as baseline owing to the possibility of support of DL CA along with DL MIMO/Rx diversity. While from what we know, the popular implementation in the field assumes “fully separated Rx Chain” with existing requirements (which are defined based on “partially shared Rx Chain”) being complied with, so in our understanding the only architecture deserves to be studied in Rel-19 is “fully shared Rx Chain” architecture.

Moreover, it was already agreed in last meeting the goal is to use “a single Rx RF Chain”, which implies “fully shared Rx Chain” architecture. Further, we think it is necessary to further align the understanding of the “Rx Chain”, and from chipset vendor perspective, our understanding on the concept of “Rx Chain” is from antenna to the end of ADC (before digital BB filter), so “fully shared Rx Chain” is from antenna to ADC fully sharing while digital BB could be either shared or separated. Better attenuation capability for in-gap interference is expected with separate digital BB due to separate digital BB filters, it is preferred to keep both shared and separated digital BB on the table at the initial stage. [10][11] also show interest on both aspects. In addition, A/D requires four-time sampling thus ADC cannot separate CC1 and CC2.

For better interpretation, the “fully shared Rx Chain” architecture and “partially shared Rx Chain” architecture are illustrated in Figure 1 and Figure 2 separately, while the “fully separated Rx Chain” architecture with totally separated chains for each CC is easy to understood so is not shown here.

(→ Separate FFT)

(→ Single FFT or separate FFT)

Figure 1: “Fully shared Rx Chain” architecture

→ Separate FFT

Figure 2: “Partially shared Rx Chain” architecture

For “partially shared Rx Chain” architecture, as shown above Figure 2, CC1 and CC2 share the antenna and external LNA, while the Rx Chains are split afterwards, which leads to larger size of transceiver, but the size for antenna and RFFE which are outside of transceiver remain the same. For instance, if we assume a handheld UE is equipped with 2 antennas for low bands, then only 2CC NC CA can be supported with “fully separated Rx Chain” architecture without MIMO/Rx diversity. And with “partially shared Rx Chain” architecture, 2CC NC CA+MIMO/Rx diversity is feasible, but probably we should say the Rx Chain number is 4 rather than 2 in this case. While by “fully shared Rx Chain” architecture, the number of Rx Chains would be reduced to 2 in this case. One obvious advantage for “partially shared Rx chain” and “fully separated Rx Chain” is their ability to process two separate carriers located arbitrarily within a frequency band owing to the presence of 2 separate LOs. In contrast, the “fully shared Rx Chain” can only support intra-band DL CA with FS≤100MHz due to the following limitations:

- Capability of LO

- Analog filter

- FFT size

The receiver would operate in low-IF mode with two CCs sharing one LO, meaning that the down-converted image signal could degrade the performance of the receiver.

As mentioned above, we think Rx performance for “partially shared Rx Chain and “fully separated Rx Chain” should be quite similar/comparable. CC1 and CC2 can be picked up by different analog filters in both architectures, and the digital filter can provide further attenuation for ACS/in-band blocking/narrow band blocking interferers. Further, from what we know, “fully separated Rx Chain” architecture as popular implementation for pure intra-band DL CA, can satisfy the existing requirements.

Observation 1: According to TR 36.823, the Rx requirements in current spec are specified based on “partially shared Rx Chain” architecture.

Observation 2: From what we know the popular implementation for pure intra-band DL CA is “fully separated Rx Chain”, with the existing requirements being complied with. One obvious advantage for “fully separated Rx Chain” is that it can process two separate carriers located arbitrarily within a frequency band. One disadvantage is that it supports fewer NC CCs.

Observation 3: The performance for “partially shared Rx Chain” and “fully separated Rx Chain” are comparable, and the spec is implementation agnostic.

Observation 4: For intra-band DL NC CA, current RAN4 spec accommodate both “partially shared architecture” and “fully separated architecture”.

Observation 5: It was already agreed in last meeting; the goal is to use “single Rx RF Chain”.

Observation 6: In our understanding, the concept of “Rx Chain” is from antenna to the end of ADC (before digital filter).

Observation 7: Better attenuation capability for in-gap interferer is expected for separate digital BB due to separate digital filters.

With observation 1-7, we re-propose,

Proposal 2: The referenced architecture for this SI is “fully shared Rx Chain” architecture (i.e., a single Rx RF Chain) with either shared digital BB or separate digital BB.

- The concept of “Rx Chain” is from antenna to the end of ADC (before digital BB filter).

Companies also suggest to further discuss whether a single FFT are expected to cover the fragmented carriers or separate FFT are expected for each fragmented carrier. It is worth nothing as [10] mentioned “if a single FFT is used to receive two non-contiguous CCs that are SCS aligned, if the CCs are 15KHz SCS that maximum BW is 50MHz and if 30KHz it is 100MHz”. Because maximum BW is related not only RF considerations but also sampling rate which is a function of FFT size and SCS selection.

Therefore, we think both single FFT and separate FFT can be considered for implementation in case digital BB is shared. Single FFT could be used assuming aligned SCS between CCs. Separate FFT can be used for not aligned SCS, or when frequency separation (CC1+gap+CC2) is larger than 50MHz and SCS is 15KHz for both CCs. In case digital BB is not shared, separate FFT should be assumed.

Proposal 3: Both single FFT and separate FFT could be considered for “shared digital BB” architecture, separate FFT should be assumed for “separate digital BB” architecture.

- Single FFT could be used if SCS between CCs are aligned

- With single FFT, maximum supported frequency separation (CC1+gap+CC2) is 50MHz with SCS 15KHz, and 100MHz with SCS 30KHz.

There is a FFS bullet in WF suggests RAN4 to study the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz (our understanding is this is specific for intra-band CA, rather than for single CC operation). More specific, for example, n7 is mandatory to support 4Rx operation, so n7(2A) shall at least support 4Rx in total, note 4Rx per CC is not requested/mandated by current spec. As a result, if UE is equipped with 4Rx, Rx0 and Rx1 would be for CC1 and Rx2 and Rx3 would be for CC2, as current popular implementation. While, with “fully shared Rx Chain” architecture, Rx0/Rx1 can support both CC1 and CC2 without maximum MIMO layer change, then Rx2 and Rx3 can be freed up and released to other bands/CCs, which is beneficial.

Proposal 4: When 2CC NC CA is configured in “fragmented carriers” mode with fully shared Rx Chain, UE is allowed to only support 2Rx in total, in the mandatory 4Rx band, thus the freed-up Rx chains can be used for other bands/carriers, per NW configuration.

Note: It should be clear that for single CC operation, 4Rx shall still be mandated, in the mandatory 4Rx band.

2.3 PSD difference consideration

It is generally assumed that the PSD difference between 2 collocated CCs of one operator is within 6 dB (equal PSD assumed with 6 dB tolerance), in which both CCs are wanted signal. However, it appears RAN4 has never had a discussion on the PSD difference between 2 collocated operators in which CC1 and CC2 from operator A are wanted signals and another CC from operator B is the interferer from operator A’s perspective. So we think it is reasonable to leverage this SI to determine an acceptable value for such a difference from UE perspective with acceptable performance by operators. During RAN-P#104, [4] raised a similar question which is illustrated in Figure 3.

Figure 3. The PSD between the wanted signal and the PSD between the wanted signal and interference

We learned from Canadian operators, that there is no “co-construction and co-site” deployment (meaning sharing infrastructure such as RU/BS antenna) in Canada but only “co-location” (meaning only location is shared but not infrastructure). Other operators also expressed that “co-location” is the most common scenario.

Given the above, we think in the field it would be difficult to predict the power/PSD imbalances for any band as the maximum EIRP and the transmitted CBW could be different for two gNBs of two operators in the same location. In addition, 3GPP has not specified a rated power for Wide Area BS so the limitation of the power is more reliant on national regulation.

More specifically, after looking at the regulatory requirements of several bands, it appears that for many bands the maximum EIRP is specified as XX dBm/MHz (in the term of PSD) for the channel bandwidth ≥1MHz. The regulation requirements for n66 and n25 are illustrated below. The n7 band has similar technical requirements as band n25.

Illustration of Canadian regulation for 3GPP band n66, SRSP-513 [7]

Illustration of Canadian regulation for 3GPP band n25, SRSP-510 [8]

The regulation specifies the maximum allowed EIRP, so chances are the power level is different if BSs are not transmitting at maximum power, if different channel bandwidths are taken into account, the power level / PSD differences become more unpredictable in the field.

Observation 8: From NW perspective, in the field, it is difficult to predict the power level difference/PSD difference between two co-location operators given the EIRP and CBW of gNBs for transmission could be different.

Proposal 5: If NW deployment assumption needs to be decided, prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure).

Considering Observation 8, we feel it may be easier to perform the study more from a UE perspective instead of determining the PSD difference in the field from NW perspective, so the focus could be to evaluate the UE performance with different in-gap interferer (i.e., different PSD difference values between wanted signal and interferer) at initial stage, and determine the threshold for PSD difference with acceptable UE performance in the end per the SID instruction.

To this end, first we suggest companies to align the PSD difference values selected for evaluation purpose.

Observation 9: It may be easier to perform the study more from a UE perspective, instead of determining the PSD difference in the field from NW perspective.

In our understanding, the RF component that determines the acceptable PSD imbalance is mainly the ADC, for which the analog filter serves to attenuate the interferer in the adjacent channel, preventing it being saturated.

For evaluation purpose, we think at least 0dB, 6dB, 10dB, 25dB PSD difference between wanted signal and interferer could be considered. 0dB is justified when two collocated inter-operators are both transmitting with maximum allowed EIRP and same BS antenna Type/pointing direction, 6dB is the classic value as collocated deployment assumption, 10dB is justified when two collocated inter-operators are transmitting with maximum allowed EIRP and same BS antenna Type, but 60°pointing direction difference[12]. 25dB is the PSD difference assumption for non-collocated deployment for which separate Rx Chains are required. Another value between 10dB and 25dB could be considered, such as 15dB.

Proposal 6: Companies to select several PSD difference values for UE performance evaluation purpose.

- 0dB, 6dB,10dB, 15dB, 25dB could be considered, other values within 25dB are not precluded

- PSD difference here means the PSD difference between the interferer and any wanted signal is within XdB

2.4 Potential RRM work

Our expectation is RAN4 RF would have a thorough examination on the UE performance degradation with “fully shared Rx Chain” architecture, and if it is agreed in future the level of the in-gap interferer/PSD difference would be the determining factor for the semi-static switch between legacy mode (i.e., “fully separated Rx Chain”) and fragmented carrier mode (i.e., “fully shared Rx Chain”), as next step we can further discuss how to translate the threshold of the PSD difference or in-gap interferer into 3GPP spec and UE implementation. There are following two possible directions.

Alt 1) Define the threshold for PSD difference/in-gap interferer as the hardware resource switching point→ It requires additional RF component (power sensor) for UE implementation thus additional cost

Alt 2) Translate the agreed threshold of PSD difference/in-gap interferer to RSRP/RSRQ/CSI-RS →  More measurement work and more impact on RAN4 RRM

Overall, we see RRM involvement is needed in future but we do not see any rush and necessity to involve RRM discussion at this initial stage.

Proposal 7: RAN4 to focus on UE RF performance examination at the initial stage. There is no rush to start RRM work.

2.5 UE fallback behavior

We echo the proposal in [10] that UE fallback behavior is supposed to be considered. As [10] pointed out, the determination of the in-gap interferer should consider that the intra-band DL CA performance in fragmented carrier mode not be worse than the performance of single CC operation. Further, we are wondering if we should further say no worse than the Pcell single CC operation.

Further, when the significant in-gap interferer precludes the UE to work in fragmented carrier mode, UE shall fallback to either single CC or regular intra-band DL CA implementation, per NW configuration. This is more scheduler perspective and has no impact on RAN4 spec.

Proposal 8: The determination of the reasonable PSD difference should consider the intra-band DLCA performance in fragmented carrier mode not be worse than the performance of Pcell single CC operation.

Proposal 9：When the significant in-gap interferer precludes the UE to work in fragmented carrier mode, UE shall fallback to either single CC or regular intra-band DL CA implementation, per NW configuration.

- Conclusion

（SID clarification）

Proposal 1: The scope should be those bands, where the fragments are fully confined within 100MHz, and with frequency ≤2700MHz.

（UE RF architecture）

Observation 1: According to TR 36.823, the Rx requirements in current spec are specified based on “partially shared Rx Chain” architecture.

Observation 2: From what we know the popular implementation for pure intra-band DL CA is “fully separated Rx Chain”, with the existing requirements being complied with. One obvious advantage for “fully separated Rx Chain” is that it can process two separate carriers located arbitrarily within a frequency band. One disadvantage is that it supports fewer NC CCs.

Observation 3: The performance for “partially shared Rx Chain” and “fully separated Rx Chain” are comparable, and the spec is implementation agnostic.

Observation 4: For intra-band DL NC CA, current RAN4 spec accommodate both “partially shared architecture” and “fully separated architecture”.

Observation 5: It was already agreed in last meeting; the goal is to use “single Rx RF Chain”.

Observation 6: In our understanding, the concept of “Rx Chain” is from antenna to the end of ADC (before digital filter).

Observation 7: Better attenuation capability for in-gap interferer is expected for separate digital BB due to separate digital filters.

Proposal 2: The referenced architecture for this SI is “fully shared Rx Chain” architecture (i.e., a single Rx RF Chain) with either shared digital BB or separate digital BB.

- The concept of “Rx Chain” is from antenna to the end of ADC (before digital BB filter).

Proposal 3: Both single FFT and separate FFT could be considered for “shared digital BB” architecture, separate FFT should be assumed for “separate digital BB” architecture.

- Single FFT could be used if SCS between CCs are aligned

- With single FFT, maximum supported frequency separation (CC1+gap+CC2) is 50MHz with SCS 15KHz, and 100MHz with SCS 30KHz.

Proposal 4: When 2CC NC CA is configured in “fragmented carriers” mode with fully shared Rx Chain, UE is allowed to only support 2Rx in total, in the mandatory 4Rx band, thus the freed-up Rx chains can be used for other bands/carriers, per NW configuration.

Note: It should be clear that for single CC operation, 4Rx shall still be mandated, in the mandatory 4Rx band.

(PSD difference consideration)

Observation 8: From NW perspective, in the field, it is difficult to predict the power level difference/PSD difference between two co-location operators given the EIRP and CBW of gNBs for transmission could be different.

Proposal 5: If NW deployment assumption needs to be decided, prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure).

Observation 9: It may be easier to perform the study more from a UE perspective, instead of determining the PSD difference in the field from NW perspective.

Proposal 6: Companies to select several PSD difference values for UE performance evaluation purpose.

- 0dB, 6dB,10dB, 15dB, 25dB could be considered, other values within 25dB are not precluded

- PSD difference here means the PSD difference between the interferer and any wanted signal is within XdB

(RRM)

Proposal 7: RAN4 to focus on UE RF performance examination at the initial stage. There is no rush to start RRM work.

(UE fallback behavior)

Proposal 8: The determination of the reasonable PSD difference should consider the intra-band DLCA performance in fragmented carrier mode not be worse than the performance of Pcell single CC operation.

Proposal 9：When the significant in-gap interferer precludes the UE to work in fragmented carrier mode, UE shall fallback to either single CC or regular intra-band DL CA implementation, per NW configuration.

- 5. Reference

| [1] | RP-241669, New SID: Study on NR FR1 DL Fragmented Carriers, MTK, RAN#104 |
| --- | --- |
| [2] | R4-2407414, Fragmented spectrum scenarios, TELUS, Bell Mobility, Telstra |
| [3] | TR 36.823 |
| [4] | RP-241269, Fragmented carrier, Apple, RAN#104 |
| [5] | R4-1347431, Non-contiguous intra-band Band 3 reference sensitivity, Qualcomm, RAN4#68bis |
| [6] | R4-134091, Non-contiguous intra-band Band 3 reference sensitivity, Qualcomm, RAN4#68 |
| [7] | SRSP-513 — Technical Requirements for Advanced Wireless Services in the Bands 1710–1780 MHz and 2110–2180 MHz [SRSP-513 — Technical Requirements for Advanced Wireless Services in the Bands 1710–1780 MHz and 2110–2180 MHz (canada.ca)] |
| [8] | SRSP-510 — Technical Requirements for Personal Communications Services (PCS) in the Bands 1850-1915 MHz and 1930-1995 MHz [SRSP-510 — Technical Requirements for Personal Communications Services (PCS) in the Bands 1850-1915 MHz and 1930-1995 MHz (canada.ca)] |

[9] R4-2414311, WF on fragmented DL carriers study, MediaTek Inc., RAN4 #112

[10] R4-2413031, On architecture options for fragmented spectrum reception, Skyworks Solutions, Inc., RAN4 #112

[11] R4-2413339, Discussion on methods for reducing the number of UE Rx chains for Fragmented Carriers, Nokia, RAN4 #112

[12] R4-2412278， Operator’s initial views on FR1 fragmented carriers study, CHTTL, RAN4 #112