3GPP TSG-RAN WG4 Meeting #112bis	R4-2416291

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Discussion on LP-WUS RRM core requirements

Agenda item:	6.23.4.2

Document for:	Discussion

Introduction

In this contribution we discuss about LP-WUS RRM requirements related open

Discussion

General aspects

LP-SS related requirements and focusing on LR supporting PSS/SSS

Currently issues related to PSS/SSS and LP-SS related entry / exit, relaxation requirements and serving cell offloading is discussed in many scenarios together. For example, when combining of the measurements is discussed in the Issue 1-1-1, both LP-SS and PSS/SSS measurements are discussed together which does not make the issues particularly clear.

Proposal 1:	Distinguish clearly issues on PSS/SSS and LP-SS and resolve first PSS/SSS issues

In the last meeting, some companies were proposing to update the RAN4#110bis agreement to be more specific about OOK-based LP-WUR and OFDM based LP-WUR. According to our understanding this issue is related to whether UE with OFDM based LR (or MR) is able to receive LP-SS, and whether to define requirements for this case..From the requirements point of view we can consider defining requirements with the assumption that OOK-based receiver is receiving LP-SS and OFDM based LR is receiving existing PSS/SSS.

Update the previous 110bis agreement:

  - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

  - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

RRM relaxation scenarios

RRM relaxation scenarios for PSS/SSS based MR and LR

Scenario 3 refers to the case where MR serving cell and neighbouring cells are being performed but relaxed while LR measurements are also being performed. Related to, at least, this case, combining of the measurements across radio’s (MR and LR) was briefly discussed in the last RAN4 meeting. Combining of the measurements may be understood as combining of the measurement samples or combining of the measurement results after averaging of the samples (e.g., higher layer combining). Also depending on whether both measurements are based on PSS/SSS or LR is performing LP-SS, the combining may mean different things.  If companies wish to continue discussing measurement combining, it should be clarified what is means, and in which scenarios. For example, it is not clear how to perform combining of LR based LR and MR radios having different accuracy requirements.

LR and MR are expected to have different accuracy requirements so it’s not clear how combining of the measurement samples/results would work

Even if the measurements from MR and LR are not combined, the MR and LR measurements can be evaluated against corresponding threshold, hence there is no need to combine MR and LR measurements to have simultaneous evaluation of MR and LR.

Without combining the measurement results, the MR and LR measurements may be evaluated simultaneously against their corresponding thresholds.

If both MR and LR are performing PSS/SSS measurements, there may be still measurement interleaving.

As it is very important for the network to know how and on what basis the UE perform measurements, we do not see it as an option leaving it up to UE implementation whether to combine / interleave measurements among the two radios.

Do not consider leaving it up to UE implementation whether to combine measurements among the two radios.

Further, as there is no agreement on the detailed design of LP-SS as well as there is no visibility to the possible accuracy levels of measurements performed using the LP-SS, agreeing on combining the different measurements seems like agreeing on the unknown.

From RAN4 requirements perspective, do not allow combining of the measurements across two radios, i.e., MR and LP-WUR.

From the specification point of view, existing legacy UE requirements including measurement delays and measurement accuracy shall not be impacted by LP-WUS/WUR. Also, for MR evaluation, the measurements shall be performed as legacy, and averaged per RS as usual.

UE will perform measurements per radio (MR and LP-WUR) and measurement samples and results are averaged per radio and per RS.

When LP relaxations are not allowed the existing legacy measurement delays and measurement accuracy requirements remain unchanged.

Existing MR measurement delays and accuracy requirements are kept unchanged.

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation

To receive power saving gains on the UE side, MR serving and neighbouring cell relaxations can be considered while UE is monitoring LP-WUS. MR relaxations of serving cell measurements can be supported when the serving cell measurements are offloaded to LR and UE is monitoring LP-WUS. For neighbouring cell measurements, the relaxations can be specified at least when conditions / thresholds defined by RAN2 and RAN4 has been met. We think that it’s ok to make a baseline agreement for this:

RAN4 specifies MR relaxation requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting either PSS/SSS based LP-WUR or LP-SS based LP-WUR. The relaxation conditions and thresholds are FFS.

About relaxation thresholds

Currently, RAN2 has made the following agreements:

### Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-SS configuration - Entry/exit condition for LP-WUS monitoring (FFS if it is always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates. ### RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

RAN4 should consider the monitoring of LP-WUS and measurement relaxation and offloading separately. From the network point of view, it is important to be able to set meaningful thresholds for LP-WUS monitoring and measurement relaxation and offloading thresholds for both LP-SS and PSS/SSS.

The purpose of the LP-WUS monitoring thresholds is to allow UE to stop legacy PO monitoring. If the UE is required to monitor paging occasions, MR is required to read an SSB before every paging occasion, and with this effort the MR would be able to perform and evaluate serving cell measurements also. Therefore, the LP-WUS entry – exit thresholds exists to control paging monitoring.

For MR to receive paging, at least one SSB needs to be measured

LP-WUS monitoring thresholds are for stopping / relaxing paging monitoring on MR

While UE is monitoring LP-WUS instead of paging occasions, UE may, if configured and if the conditions are fulfilled:

- relax MR neighbouring cell measurements and / or offload the MR serving cell measurements to LR

MR offloading / relaxation thresholds are introduced to control when MR serving / neighbouring cell measurements can be relaxed

From the procedure point of view, LP-WUS monitoring may start already before relaxations / offloading occur, hence the requirements should consider LP-WUS monitoring entry/exit independently. Entering LP-WUS monitoring may occur at different time to MR relaxation / offloading entry.

UE may stop paging monitoring once the LP-WUS monitoring conditions are fulfilled. During this time, LP-WUS monitoring requirements apply.

As agreed by RAN2, LP-WUS monitoring thresholds, which allow UE to stop monitoring legacy PO, are configured independently from MR offloading and relaxation thresholds. RAN4 shall define LP-WUS monitoring requirements independently from the MR measurement relaxation and offloading.

Relaxation criteria

RAN2 is currently discussing about the relaxation criteria, for instance Mobility criteria and Cell Edge criteria. Cell edge The discussion in RAN4 can wait until there are outcomes from RAN2.

RAN4 will postpone the discussion on relaxation criteria related requirements (e.g., mobility criteria and cell edge criteria) until RAN2 has made progress.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR serving cell measurement Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring

RAN4 to work on the following issues once RAN2 has made progress:

  - Criteria (entry/exit conditions) for LP-WUR serving cell measurement

  - Criteria (entry/exit conditions) for MR RRM measurement relaxation

  - Criteria (entry/exit conditions) for LP-WUS monitoring

Entry / Exit evaluation

From RAN4 point of view there should be no problem of using either PSS/SSS based LP-WUR measurement or LP-SS based LP-WUR measurements for entry threshold. For each reference signal the configurations of the thresholds may be different. The configuration aspects can be left to RAN2 but the accuracy of the LP-WUR for PSS/SSS or LP-SS measurements will impact on how well the network may configure the thresholds.

Both LP-SS and PSS/SSS based measurement can be used for LP-WUR to evaluate the entry/exit condition.

Agreement From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols - LP-RSRQ - LP-RSRQ = LP-RSRP/LP-RSSI - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options: - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols. - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols. - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols. - FFS: LP-SINR Note: The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions. Agreement The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows: LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS. LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

For LP-SS based LP-WUR it is still unclear of RSRQ measurements can be used. For PSS/SSS based LP-WUR both RSRP and RSRQ measurements can be used.

RAN4 to define requirements for:

  - Entry / Exit evaluation for SS-RSRP / SS-RSRQ measurements for OFDM based LP-WUR supporting PSS/SSS

  - Entry / Exit evaluation for LP-RSRP / LP-RSRQ measurements for OOK based LP-WUR supporting LP-SS

Issue 1-1-11: Considerations on higher priority frequency layer - P1: higher priority layer neighbor cell measurement could also be OFF for case #1. (Apple vivo) - P2: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor. (ZTE)

High priority frequency layer search may be relaxed but not turned off without network configuration. The network may always select not to configure high priority frequency layers which addresses the turning off scenario.

No need to define new requirements for higher priority frequency layer.

LP-WUR operating carrier frequency

LP-WUR operating carrier frequency - P1: RAN4 assumed LR and MR are operating on the same carrier frequency as baseline, and FFS on the other scenarios if RAN1 has agreement. (Apple QC)

At least the serving cell measurements performed by LP-WUR and MR should be on the same carrier frequency.

RAN4 assumes that LP-WUR (PSS/SSS and LP-SS) and MR (PSS/SSS) measurements are operating on the same carrier frequency.

Issue 1-2-6: Nubmer of Rx for LP-WUR requirements - P1: Single Rx is assumed for LR based RRM measurement. (Apple) Recommendations: Encourage companies to check whether P1 of this issue is agreeable or not.

Currently RAN1 simulations have already been assuming single Rx. This is also ok from RAN4 point of view. However, if the simulations show that the performance is not acceptable, or there are scenarios where 2rx is needed, then this may be revisited.

Single Rx is assumed for LR based RRM measurement. This may be revisited based on simulation results, if necessary.

Issue 1-2-7: Other considerations - P1: Discuss whether the relaxations can be activated once the UE enters idle-mode - Relaxation / offloading may be applied based on connected mode MR measurements (Nokia)

The UE may enter LP-WUS mode, i.e., at least some of the entry conditions for measurement relaxation or serving cell offloading are fulfilled. This may occur either directly when being released to the idle-mode, or alternatively during the idle-mode. It is not yet clear whether the UE is required to read idle-mode configuration information, for instance LP-WUS specific SIB information prior to applying relaxation and / or offloading. These details are up to RAN2, but this may have impact on whether the relaxations can be activated once the UE enters idle-mode, for instance based on MR connected mode measurements.

Discuss whether the relaxations can be activated once the UE enters idle-mode - Relaxation / offloading may be applied based on connected mode MR measurements

Conclusion

The following Observations and Proposals were made:

Proposal 1: Update the previous 110bis agreement:

  - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

  - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Observation 1: LR and MR are expected to have different accuracy requirements so it’s not clear how combining of the measurement samples/results would work

Observation 2: Without combining the measurement results, the MR and LR measurements may be evaluated simultaneously against their corresponding thresholds.

Proposal 2: Do not consider leaving it up to UE implementation whether to combine measurements among the two radios.

Proposal 3: From RAN4 requirements perspective, do not allow combining of the measurements across two radios, i.e., MR and LP-WUR.

Proposal 4: UE will perform measurements per radio (MR and LP-WUR) and measurement samples and results are averaged per radio and per RS.

Proposal 5: Existing MR measurement delays and accuracy requirements are kept unchanged.

Proposal 6: RAN4 specifies MR relaxation requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting either PSS/SSS based LP-WUR or LP-SS based LP-WUR. The relaxation conditions and thresholds are FFS.

Observation 3: For MR to receive paging, at least one SSB needs to be measured

Observation 4: LP-WUS monitoring thresholds are for stopping / relaxing paging monitoring on MR

Observation 5: MR offloading / relaxation thresholds are introduced to control when MR serving / neighbouring cell measurements can be relaxed

Proposal 7: UE may stop paging monitoring once the LP-WUS monitoring conditions are fulfilled. During this time, LP-WUS monitoring requirements apply.

Proposal 8: As agreed by RAN2, LP-WUS monitoring thresholds, which allow UE to stop monitoring legacy PO, are configured independently from MR offloading and relaxation thresholds. RAN4 shall define LP-WUS monitoring requirements independently from the MR measuremen relaxation and offloading.

Proposal 9: RAN4 will postpone the discussion on relaxation criteria related requirements (e.g., mobility criteria and cell edge criteria) until RAN2 has made progress.

Proposal 10: RAN4 to work on the following issues once RAN2 has made progress:

  - Criteria (entry/exit conditions) for LP-WUR serving cell measurement

  - Criteria (entry/exit conditions) for MR RRM measurement relaxation

  - Criteria (entry/exit conditions) for LP-WUS monitoring

Proposal 11: Both LP-SS and PSS/SSS based measurement can be used for LP-WUR to evaluate the entry/exit condition.

Proposal 12: RAN4 to define requirements for:

  - Entry / Exit evaluation for SS-RSRP / SS-RSRQ measurements for OFDM based LP-WUR supporting PSS/SSS

  - Entry / Exit evaluation for LP-RSRP / LP-RSRQ measurements for OOK based LP-WUR supporting LP-SS

Proposal 13: No need to define new requirements for higher priority frequency layer.

Proposal 14: RAN4 assumes that LP-WUR (PSS/SSS and LP-SS) and MR (PSS/SSS) measurements are operating on the same carrier frequency.

Proposal 15: Single Rx is assumed for LR based RRM measurement. This may be revisited based on simulation results, if necessary.

Proposal 16: Discuss whether the relaxations can be activated once the UE enters idle-mode - Relaxation / offloading may be applied based on connected mode MR measurements