3GPP TSG-RAN4 Meeting #112bis	R4-2416297

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on IoT-NTN HPUE Rx requirements

Source:	Samsung

Agenda item:	6.8.2.3.2

Document for:	Approval

1	Introduction

In the RAN4#112 meeting, the WF R4-2414452 [1] had initially discussed the reference sensitivity degradation (RSD) requirements for NTN HPUE as following:

Issue 4-2: RSD for NR-NTN - Agreement - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE. - Further discuss the following aspects: - Use similar way in TN spec by introducing a new table of RSD compared to the PC3 - Whether default Tx-Rx frequency separation is used for test point

In this paper, we continue to discuss the RSD requirements for NR-NTN HPUE.

2	Discussions

The power classes of HPUEs, including PC3, 2 and 1, generally impact the reference sensitivity as discussed. The factors of the RSD include the RFIC characteristics and duplexer and PA characteristics impact by bearing higher power level for HPUE.

The Tx-Rx frequency separation for the currently defined IoT-NTN bands is listed as follows:

5.4A.3 TX–RX frequency separation a) The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.4A.3-1 for the TX and RX channel bandwidth defined in Table 5.3A-1. Table 5.4A.3-1: Default UE TX-RX frequency separation E-UTRA Operating Band; TX – RX carrier centre frequency separation. 256; 190 MHz. 255; -101.5 MHz. 254; 873.5 MHz. 253; -150 MHz. b) The use of other TX channel to RX channel carrier centre frequency separation is not precluded and is intended to form part of a later release. 5.4B.3 TX–RX frequency separation For UE category NB1/NB2 operation in stand-alone mode, the default TX-RX frequency separation shall be as specified in Table 5.4A.3-1 for the NB-IoT TX and RX channel bandwidth defined in Table 5.3B-1.

For some IoT-NTN devices, including those handheld devices, it can use similar RF components and designs as NR-NTN HPUE devices discussed in another thread. For those devices, it could be possible to re-use the same RSD values as we proposed for NR-NTN HPUE RSDs.

Proposal 1: For some IoT-NTN devices, including handheld IoT-NTN devices, which uses similar PA/duplexer as TN smartphone, it can adopt similar PC2 RSD values as n1/n3 in 38.101-1.

For other IoT-NTN devices with different RFFE/RFIC design, the RSD values should be further discussed based on the assumption of RF components.

3	Conclusion

For the IoT-NTN HPUE Rx requirements, we propose the following.

Proposal 1: For some IoT-NTN devices, including handheld IoT-NTN devices, which uses similar PA/duplexer as TN smartphone, it can adopt similar PC2 RSD values as n1/n3 in 38.101-1.

4	Reference

[1] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung