3GPP TSG-RAN4 Meeting #112bis	R4-2416298

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on Ku-band coexistence studies

Source:	Samsung

Agenda item:	6.9.2

Document for:	Approval

1	Introduction

In the RAN4#112 meeting, the WF R4-2413519 [1] had documented the way forwards for coexistence study. In this paper, the remaining issues will be discussed and the solutions are proposed to further progress the work.

2	Discussions

In our last contribution [3], we proposed to not consider the geographical isolation/separation between TN and NTN networks at the beginning of the co-ex study, considering the NTN device in 14GHz band may not be in the same fform factor as the TN devices.

Observation 1: The NTN devices in Ku-band is not to re-use the existing TN devices, given the fact that the 15GHz TN network is still under study.

Proposal 1: To not consider isolation distance or geographical separation between TN and NTN in Ku-band study at the beginning.

In our last contribution [3], we also proposed to consider a different uplink configurations, including transmission bandwidth, target SNR for Ku-band uplink study. It is because of the 15dB SNR target is from TN network assumptions, which considers the throughput target and modulation target in TN network topology. However, for the Ku-band NTN, given the fact that the uplink will have longer distance between NTN VSAT and NTN SAN, and the target use scenario may have different target SNR/throughput for NTN use cases, the SNR target and uplink bandwidth could be assumed differently.

Observation 2:  The SNR target of 15dB was assumed in Rel-17 based on the similar assumption of TN network. And the associated 2RB uplink bandwidth also comes from the link budget from GEO scenario and in FR1-NTN frequencies.

Proposal 2: The uplink bandwidth and its SNR target in Rel-17 was derived based on FR1-NTN frequencies and TN network assumptions, it can be different based on the input from satellite companies considering different use cases in Ku-band.

In the WF[1], the NTN channel model was agreed as following:

For NTN channel model, reuse parameters from TR 38.811 and collect the following missing information: - Shadow fading and clutter loss tables for the reference frequencies - Tropospheric scintillation.

For the clutter loss, in our view, at least the ITU-R Recommendation P.2108 [2] can be used as a reference. More specifically, the section 3.3 of ITU-R P.2108 can be used for the ’Earth-space and Aeronautical statistical clutter loss model’.

The corresponding section is provided below:

3.3 Earth-space and Aeronautical statistical clutter loss model This section provides equations to calculate the statistical distribution of clutter loss where one end of the interference path is within man-made clutter, and the other is a satellite, aeroplane, or other platform above the surface of the Earth. An additional loss, Lces, is calculated which can be added to the basic transmission loss of a path calculated. This model is applicable to urban and suburban environments. The method used to develop this model is described in Report ITU-R P.2402-0. Frequency range: 10 to 100 GHz Elevation angle range: 0 to 90 degrees Percentage locations range: 0 < p < 100 3.3.1 Input parameters Input parameters are given in Table 5. TABLE 5 Earth-space clutter loss model input parameters Input; Symbol; Unit. Frequency; f; GHz. Elevation angle; θ; degrees. Percentage of locations; p; %. 3.3.2 Model description The clutter loss not exceeded for p% of locations Lces for the terrestrial to airborne or satellite path is given by: Lces=-K1ln1-p100cotA11-θ90+πθ1800.590-θ90-1-0.6 Q-1(p/100) dB (7) with K1=93f0.175, A1=0.05 where Q–1(p/100) is the inverse complementary normal distribution function, and the elevation angle, , is the angle of the airborne platform or satellite as seen from the terminal. Figure 2 shows the cumulative distribution of clutter loss not exceeded curves for percentage locations at varying elevation angles for 30 GHz. FIGURE 2 Cumulative distribution of clutter loss not exceeded for 30 GHz

The above model in section 3.3 of [2] requires input of frequency, elevation angle and percentage locations, where the input frequency can use the assumed frequency point for co-ex study, the elevation angle can use the derived elevation angle from simulation topology, and the percentage location can use 50% for the purpose of study.

Observation 3: The section 3.3 of ITU-R P.2108[2] can be used as a reference propagation model for clutter loss in addition to the free space propagation loss of ITU-R P.525, the input parameters can be derived from simulation topology and the percentage location can be assumed as 50% for the median/average purpose.

Proposal 3: To use the section 3.3 of ITU-R P.2108-1[2] model for the earth-space (NTN VSAT – SAN) clutter loss in the study as following. This model is to be used as an addition to the free-space propagation loss model for clutter loss.

Lces=-K1ln1-p100cotA11-θ90+πθ1800.590-θ90-1-0.6 Q-1(p/100) dB (7) with K1=93f0.175, A1=0.05 Where Input; Symbol; Unit; Assumptions. Frequency; f; GHz; 11GHz DL [1] 14GHz UL [1]. Elevation angle; θ; degrees; Derived in simulation. Percentage of locations; p; %; Propose to use 50% as median case.

For NTN SAN parameters, in [1], the parameters for transmit EIRP, and the rest are still missing for the study of Ku-band. It requires input from Satellite operators/companies to provide the applicable parameters for the Ku-band NTN SAN.

Observation 4: The NTN SAN parameters, including EIRP and rest, are still missing, and pending for input from satellite companies.

Proposal 4: To encourage companies to contribute missing parameters for NTN SAN in Ku-band.

For NTN VSAT parameters, the proposals from CATT [4] and Intelsat [5] are both for directional antennas with mechanical steering. It can be workable for co-ex study for mechanical steeering VSAT, however it does not come with the antenna pattern for electrical steering antenna.

Observation 5: The NTN VSAT antenna assumption using section 6.4.1 of TR 38.811 only applies to the mechanical steering VSAT, it’s better to have a separated and dedicated antenna pattern for electrical steering VSAT.

Proposal 5: To encourage companies to provide antenna model/pattern for electrical steering VSAT for Ku-band.

3	Conclusion

For the NTN Ku-band co-ex study assumption, we propose the following.

Observation 1: The NTN devices in Ku-band is not to re-use the existing TN devices, given the fact that the 15GHz TN network is still under study.

Proposal 1: To not consider isolation distance or geographical separation between TN and NTN in Ku-band study at the beginning.

Observation 2:  The SNR target of 15dB was assumed in Rel-17 based on the similar assumption of TN network. And the associated 2RB uplink bandwidth also comes from the link budget from GEO scenario and in FR1-NTN frequencies.

Proposal 2: The uplink bandwidth and its SNR target in Rel-17 was derived based on FR1-NTN frequencies and TN network assumptions, it can be different based on the input from satellite companies considering different use cases in Ku-band.

Observation 3: The section 3.3 of ITU-R P.2108[2] can be used as a reference propagation model for clutter loss in addition to the free space propagation loss of ITU-R P.525, the input parameters can be derived from simulation topology and the percentage location can be assumed as 50% for the median/average purpose.

Proposal 3: To use the section 3.3 of ITU-R P.2108-1[2] model for the earth-space (NTN VSAT – SAN) clutter loss in the study as following. This model is to be used as an addition to the free-space propagation loss model for clutter loss.

Lces=-K1ln1-p100cotA11-θ90+πθ1800.590-θ90-1-0.6 Q-1(p/100) dB (7) with K1=93f0.175, A1=0.05 Where Input; Symbol; Unit; Assumptions. Frequency; f; GHz; 11GHz DL [1] 14GHz UL [1]. Elevation angle; θ; degrees; Derived in simulation. Percentage of locations; p; %; Propose to use 50% as median case.

Observation 4: The NTN SAN parameters, including EIRP and rest, are still missing, and pending for input from satellite companies.

Proposal 4: To encourage companies to contribute missing parameters for NTN SAN in Ku-band.

Observation 5: The NTN VSAT antenna assumption using section 6.4.1 of TR 38.811 only applies to the mechanical steering VSAT, it’s better to have a separated and dedicated antenna pattern for electrical steering VSAT.

Proposal 5: To encourage companies to provide antenna model/pattern for electrical steering VSAT for Ku-band.

4	Reference

[1] R4-2410567, WF for [112][314] NR_NTN_Ku_Band_General, Eutelsat

[2] Recommendation ITU-R P.2108-1 (09/2021), ITU-R

[3] R4-2412560, Discussion on Ku-band coex studies, Samsung

[4] R4-2411120, Discussion on coexistence evaluation for Ku-band for NR NTN, CATT

[5] R4-2411507, Ku band co-existence, Intelsat