3GPP TSG-RAN4 Meeting #112bis	R4-2416299

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on Ku-band UE RF requirements

Source:	Samsung

Agenda item:	6.9.4

Document for:	Approval

1	Introduction

In this paper, we will start discussing the UE RF requirements for NTN Ku-band.

2	Discussions

In the WID[1], the objectives of this WI considers following:

- Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing

Given the objectives in [1] and agreements in [2] both support the view that the Ku-band UE would be VSAT like NTN UEs, the major structure of the UE RF specifications should follow the FR2-NTN UE requirements structure defined in TS 38.101-5.

Proposal 1: The Ku-band UE RF requirements structures/impacts could follow the FR2-NTN VSAT requirements defined in TS 38.101-5.

In FR2-NTN discussions, RAN4 only defines the radiated requirements for FR2 NTN VSAT. Given the Ku-band is in the middle of FR1 and FR2 bands, it’s a question mark whether the NTN VSAT need conducted RF requirements.

Proposal 2: To discuss whether conducted RF requirements need to be specified for Ku-band NTN VSAT considering the potential RF architecture of Ku-band VSAT.

The electrical and mechanical steering antennas were considered in TS 38.101-5 for VSAT types, however, the electrical steering antenna patterns was not thoroughly discussed during Rel-18.

The electrical steering antenna patterns, i.e. the phase array or flat-panel antenna patterns, would impact the min peak EIRP requirements, and also the co-ex study assumptions.

It is proposed to encourage the satellite UE vendors, especially for those who provided flat panel VSAT in Ku-band, to contribute the antenna patterns/modelling for the phase array antenna or flat-panel antennas.

Proposal 3: To encourage Ku-band VSAT vendors to contribute the flat-panel or phase-array antenna patterns and modelling.

3	Conclusion

For the Ku-band UE RF requirements, we propose the following.

Proposal 1: The Ku-band UE RF requirements structures/impacts could follow the FR2-NTN VSAT requirements defined in TS 38.101-5.

Proposal 2: To discuss whether conducted RF requirements need to be specified for Ku-band NTN VSAT considering the potential RF architecture of Ku-band VSAT.

Proposal 3: To encourage Ku-band VSAT vendors to contribute the flat-panel or phase-array antenna patterns and modelling.

4	Reference

[1] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, et, al.

[2] R4-2413519, WF for [112][314] NR_NTN_Ku_Band_General, Eutelsat.