3GPP TSG-RAN4 Meeting #112bis	R4-2416300

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on Ambient IoT CW cancellation

Source:	Samsung

Agenda item:	6.21.3.4

Document for:	Approval

1	Introduction

In the RAN4#112 meeting, the WF R4-2414304 [1] has agreed to further discuss the CW cancellation feasibility and potential values.

In this paper, we share our view on the potential CW cancellation feasibilities.

2	Discussions

The CW cancellation capability used for co-ex evaluation is critical to the co-ex study results. In our view, the cancellation capability can falls into different categories, and this could possibly help the discussion and converge the cancellation values for different scenarios/types.

The WF of CW cancellation is provided below:

ssue 3-3-2: CW cancellation capability Agreement in RAN4#112: - Companies to report the CW cancellation capability used for co-existence evaluation, CW cancellation capability for inside topology and outside topology can be different. - total interference cancellation capability = spatial isolation + RF cancellation + digital cancellation CW transmission impact on the received SINR, e.g. degradation

For the spatial isolation part of the CW cancellation, it could vary between different scenarios, especially between wide area and indoor (local area) scenarios. And the alternative 1(A1) and alternative 2(A2) also plays important roles.

In our view, we can take SBFD discussions as a reference somehow, and the mapping between AIoT and SBFD scenarios can be find as follows.

| AIOT scenarios | SBFD |
| --- | --- |
| D1T1-A1 | Indoor(Local area BS) self-interference cancellation |
| D1T1-A2 | Indoor(Local area BS) inter-site cancellation |
| D2T2-A1 | Urban(Wide area BS) self-interference cancellation |
| D2T2-A2 | Urban(Wide area BS) inter-site cancellation |

Following this method, the candidate CW cancellation capability can be listed as follows.

For D1T1-A1:

The SBFD Indoor self-interference cancellation as follows:

Table 9.4.1.1-1: SI feasibility study for FR1 Local Area BS FR1; Ericsson (preliminary); ZTE; CATT; Huawei; Nokia; Samsung. BS class; Local Area BS (3GPP minimum); Local Area BS (Realistic RX); LA; LA; LA; Local Area BS; Local Area BS. BS TX Power dBPTX = ① dBm; 24; 24; 23; 24; 24; 23; 24. Component capability and parameters; Frequency isolation at TX; Frequency isolation capability dBαSI-frequency, TX = ② dBc; 45; 45; 45; 45; 45; 45; 45. Frequency isolation techniques used; Digital filtering, CFR, DPD; DPD utilized; CFR; DPD; Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion; DPD utilized. Spatial isolation; Spatial isolation capability dBαSI-spatial = ③ dBc; 70; 70; 60; 70; 70; 60 (if omnidirectional antennas are used, this would be less); 80. Spatial isolation techniques used; Physical distance, isolation structures; TX/RX panel separation; TX/RX panel separation, isolation structures, isolation material, cross polarization; Spatial separation between TX panel to single RX; Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels; TX/RX panel separation and RF barrier structure. TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc; 0; 0; 0; 0; 0; 0; 0. DL EIRP impact due to beam nulling in TX sub-band; TX beam nulling not assumed due to array size; N/A; TX Beam nulling is not assumed due to the array size; N/A; TX Beam nulling is not assumed due to the array size; N/A. Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1); -91; -91; -101; -104.01; -97; -88 (①-②-③-④- 10*log10(80/20)); -101 Note: provided by ①-②-③-⑨. RF IC and other tech. (before LNA); RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc; 0; 0; 0; 0; N/A; 0; 0. RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc; 0; 0; 0; 0; N/A; 0; 0. RF IC techniques and other tech. (before LNA); Analogue IC could be considered for this case, but is restrictive on pre-coding and multi-carrier. Digital IC has instead been assumed.; subband filtering; 0; N/A; 0; N/A. Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA; 0; 0; 0; 0; N/A; 0; N/A. Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1); -46; -46; -50; -46; -46; -37; -56 Note: provided by ①-③-④-⑤. Blocker Suppression at RX; Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc; xxx dBc; 60; 55; 60-80; 40. Frequency isolation techniques; Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain.; Digital Filtering; Digital filter for ACS; Digital filtering; Digital filtering. RX IMD; Rx IIP3 capability (dBm); -24.6; -10; -5; -16; -5; -10; -20. Rx IM3 contribution (dBm); -88.8; Negligible; -140; -106; -128; Negligible; -128. Other RX; Any other RX impacts if significant (e.g. ADC noise, phase noise etc.); No other significant impacts other than those mentioned above; Marginal; No other significant impacts; negligible; No other significant impacts; N/A. Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) (dBm); -88.8; -110; -110; -101; negligible; -87 (at 50 dB ACS); -96 Note: provided by ①-③-④-⑤-⑥. RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc; 0-3 dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining.; 0; 0; N/A; 0; 0. RX sensitivity degradation caused by RX beam nulling; N/A; N/A; N/A. Digital IC dBαSI-digtial = ⑦ dBc; 15 dBc on transmitter; 20; 10; N/A; 5 to 10; 10. Overall RSIC capability dBRSIC (Note 1); 112 dBc; 128 dBc; 130.5; 122.33; 121; 110 to 115; 129. Noise floor ⑩dBm; -87 dBm/CBW; -87 dBm/CBW; -91.0; -87.99; -87 dBm/20 MHz; -91.0; -87. Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB); -93 dBm; -93 dBm; -97.0; -93.99; -93 dBm; -97.00; -93. Required RSIC budget (①-⑪dBc); 117 dBc; 117 dBc; 127.0; 117.99; 117; 120.0; 117. SBFD configuration; 40-20-40; 40-20-40; 40-20-40; 40-20-40; 40-20-40; DUD(40-20-40MHz). Guardband assumption (if exist); 5 PRB; Existing SU; 5 PRB; 5 PRB. bandwidth over which suppression is achieved; <300MHz; 20MHz. Others; The conclusion does not take into account interference increase due to scattering effects, or the possibility for receiver algorihms to mitigate scattering, inter-sector and inter-site and self-interference (reference scenarios needed).. Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics..

In spatial domain, the isolation can be around 70 dB based on above table.

In RF cancellation domain, similarly, this depends on the RF design, especially on whether there could be additional filters equipped on the reader.

In digital domain, the isolation could be around 10 dB, based on previous SBFD discussions.

Observation 1: For D1T1-A1, a potential CW cancellation capability can be around 80 dBc. And higher cancellation capability can be achievable if beam nulling, and other techniques adopted.

For D2T2-A1:

The SBFD outdoor self-interference cancellation as follows:

Table 9.2.1.1-1: FR1 WA BS Self-interference Analysis Summary FR1; Samsung; Samsung; Ericsson; Huawei; Huawei; Qualcomm; CATT; Nokia. BS class; Wide Area BS (subband filter-1); Wide Area BS (subband filter-2); Wide Area BS; Wide Area BS example 1; Wide Area BS example 2; Wide Area BS; Wide Area BS; Wide Area BS. BS TX Power dBPTX = ① dBm; 49 dBm; 49 dBm; 53 dBm; 47; 53; 49 dBm; 49; 54. Component capability and parameters; Frequency isolation at TX; Frequency isolation capability dBαSI-frequency, TX = ② dBc; 45 dBc; 45 dBc; 45 dBc; 45; 45; 45 dBc; 45; 45 dBc. Frequency isolation techniques used; DPD utilized; DPD utilized; Digital filtering, CFR, DPD; DPD; DPD; DPD; CFR、DPD; Digital filtering or windowing to clean UL sub-band; DPD to suppress PA distortion. Spatial isolation; Spatial isolation capability dBαSI-spatial = ③ dBc; 80 dBc; 80 dBc; 70 dB; 80; 80; 80 dBc; 70; 65 dBc. Spatial isolation techniques used; TX/RX panel separation and RF barrier structure; TX/RX panel separation and RF barrier structure; A combination of spatial isolation, chokes, absorption, mushroom EBG. 70dB is indicative average; isolation varies from around 55dB to more than 80dB depending on beam direction.; Spatial separation between TX/RX panel with absorbing material and choke structure.; Spatial separation between TX panel with absorbing material and choke structure.; Two separate panels with added electro-magnetic spatial duplexer for additional cancellation; TX/RX panel separation、 isolation structures, isolation material, cross polarization; Spatial separation between TX/RX panels; EM shielding structures between TX/RX panels 65dB is indicative average; isolation varies from around 53 dB to 73dB depending on beam direction... TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc; 10 dBc; 10 dBc; 10 dBc TX beam nulling reduces the variation with beam direction, and hence spatial isolation + TX nulling can be around 80dB for most directions.; 10; 10; 15 dBc; 10; 12 dBc. DL EIRP impact due to beam nulling in TX sub-band; Limited, ~0dB; Limited, ~0dB; Up to 1-5dB EIRP loss, depending on beam direction; Less than 0.5 dB loss; Less than 0.5 dB loss; 0.8 dB maximum. Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1); -86 dBm Note: provided by ①-②-③-⑨ dBm; -86 dBm Note: provided by ①-②-③-⑨ dBm; -72 dBm; -94; -88; -91 dBm (=①-②-③-④); -89.01 ①-②- ③- ⑨-⑦-10*log10(40/20); -62 dBm/20 MHz. RF IC and other tech. (before LNA); RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc; 15 dBc; 11.8 dBc; 0 dBc; N/A; 10; 0; 0 dBc. RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc; 0dBc; 0dBc; 0 dBc; N/A; N/A; 0; 0 dBc. RF IC techniques and other tech. (before LNA); subband filtering (20MHz passband, 2* 5PRB transition band used for roll-off between passband/stopband); subband filtering (24.8MHz passband, 2*3.8MHz transition band used for roll-off between passband/stopband); None; see section 9.2.1.2.2 for analysis.; N/A; Analog filter is put after LNA; None apply due to feasibility concerns. Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA; Limited; Limited; N/A; N/A; 0; N/A dBc. Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1); -56 dBm Note: provided by ①-③-④-⑤dBm; -52.8 dBm Note: provided by ①-③-④-⑤dBm; -27 dBm; -43; -37; -31 Rx is blocked ①-③-④-⑤; -23 dBm with some variations depending on TX beam. Blocker Suppression at RX; Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc; 40 dBc; 40 dBc; 0 dBc; digital filter: 60-80 dB; sub-band analog filter: 10 dB digital filter: 60-80 dB; 15 dBc; 65; 0 dBc. Frequency isolation techniques; Filtering; Filtering; The receiver is in high non-linearity; no possibility for interference mitigation as part of the digital receive combining algorithms. .; digital filtering; sub-band analog filter and digital filtering; Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done).; Digital filter for ACS; None apply due to feasibility concerns. RX IMD; Rx IIP3 capability (dBm); With subband filtering, RX non-linearity impact is neglectable; With subband filtering, RX non-linearity impact is neglectable; -32dBm (Minimum for RAN4 requirement) -22dBm (Realistic for AAS) -10dBm (optimistic for AAS); -10; 0; Not a significant contributor on the gNB Rx capability.; -10; -10 dBm at maximum sensitivity; +10 dBm at maximum linearity (at NF penalty). Rx IM3 contribution (dBm); Even without ADC overload: -17 dBm (RAN4 minimum receiver) -37 dBm (Realistic) -61 dBm (Optimistic); -109; -121; -73 (①-③-④-⑤)-2*(IIP3-(①-③-④-⑤)); Negligible (at NF penalty). Other RX; Any other RX impacts if significant (e.g. ADC noise, phase noise etc.); N/A; N/A; ADC overload can be mitigated with filtering prior to ADC except for direct conversion architectures.; ADC noise: -109 reciprocal phase noise mixing:-112; ADC noise: -113 reciprocal phase noise mixing:-116; Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model.; -116; Negligible. Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2); -96 dBm Note: provided by ①-③-④-⑤-⑥dBm; -92.8 dBm Note: provided by ①-③-④-⑤-⑥dBm; Receiver in high non-linearity; -105; -111; -96 ①-③-④-⑤-⑥; -73 dBm/20MHz (at 50 dBc ACS). RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc; 10 dBc; 10 dBc; RX processing does not mitigate analogue non-linearity; 10; 10; 10; 0 dBc. RX sensitivity degradation caused by RX beam nulling; Limited, ~0dB; Limited, ~0dB; Receiver saturated; Less than 0.5 dB loss; Less than 0.5 dB loss; 0 dBc; should not assume further UL beamforming loss to maintain any UL gains. Digital IC dBαSI-digtial = ⑦ dBc; 20 dBc; 20 dBc; Digital IC not possible due to receiver non-linearity and would anyhow be highly complex due to large number of TX/RX for wide area.; 15; 15; 15 dB; 10; 0 dBc. Overall RSIC capability dBRSIC (Note 1); 154.6 dBc; 154.2 dBc; Transmitter: 125 dB Receiver: N/A due to receiver saturation; -150.6; -155.3; 155 dB (②+③+④+⑦); 121.86; 122 dBc. Noise floor ⑩dBm; -96dBm/20MHz; -96dBm/20MHz; -96 dBm/CBW; -96 dBm/20 MHz; -96 dBm/20 MHz; -96 dBm/20 MHz @ 5dB noise figure; -95.99; -96 dBm/CBW (20 MHz). Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB); -102 dBm; -102 dBm; -102 dBm; -102 dBm; -102 dBm; -102 dBm; -101.99; -102 dBm. Required RSIC budget (①-⑪dBc); 151 dBc; 151 dBc; 155 dBc; 149; 155; 151 dBc; 150.99; 156 dBc. SBFD configuration; DUD(40-20-40MHz); DUD(40-20-40MHz); 40-20-40 MHz; DUD [40, 20, 40]; DUD [40, 20, 40]; DUD; 40-20-40; DUD (40/20/40 MHz). Guardband assumption (if exist); 5 PRB; 5 PRB; 5 PRB.; Existing SU; Existing SU; 5 PRBs; 5 RB (1.8 MHz). bandwidth over which suppression is achieved; 100MHz; 100MHz; >300 MHz; Several hundred MHz; Several hundred MHz; 100MHz. Others.

In spatial domain, the isolation can be around 80 dB based on above table.

In RF cancellation domain, similarly, this depends on the RF design, especially on whether there could be additional filters equipped on the reader. The potential capability can be around 15dB.

In digital domain, the isolation could be around 10 dB, based on previous SBFD discussions.

Observation 2: For D2T2-A1, a potential CW cancellation capability can be around 105 dBc. And higher cancellation capability can be achievable if beam nulling, and other techniques adopted.

For D1T1-A2 and D2T2-A2:

This can be totally up to the ACLR/ACS-like calculation, given in A2 scenario the CW signal is outside the topology, and its residual interference can be calculated from study.

Observation 3: For D1T1-A2 and D2T2-A2, the CW cancellation can be calculated through the study with the assumptions of Tx leakage of CW signal and Rx rejection on Rx side.

3	Conclusion

For the Ambient IoT CW cancellation discussion, we propose the following.

Observation 1: For D1T1-A1, a potential CW cancellation capability can be around 80 dBc. And higher cancellation capability can be achievable if beam nulling, and other techniques adopted.

Observation 2: For D2T2-A1, a potential CW cancellation capability can be around 105 dBc. And higher cancellation capability can be achievable if beam nulling, and other techniques adopted.

Observation 3: For D1T1-A2 and D2T2-A2, the CW cancellation can be calculated through the study with the assumptions of Tx leakage of CW signal and Rx rejection on Rx side.

4	Reference

[1] R4-2414304, WF on co-existence evaluation for ambient IoT, CMCC