3GPP TSG-RAN4 Meeting #112bis	R4-2416301

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on Ambient IoT co-existence assumptions

Source:	Samsung

Agenda item:	6.21.2.2

Document for:	Approval

1	Introduction

In the RAN4#112 meeting, the WF R4-2414304 [1] had achieved progress for the co-existence evaluation on Ambient-IoT, but also left open issues for further discussions. In this paper, the remaining issues will be discussed and the solutions are proposed to further progress the work.

2	Discussions

In the WF [1], the discussion of the performance metrics has reached some early consensus, however the details still remain open.

Issue 3-2-1: Performance metric for AIOT Agreement in RAN4#110bis: - For NR system, use 5% throughput loss as performance metric as legacy. - For AIOT system, including reader, device, intermediate UE, further discuss the performance metric: - Option 1: [10%] BLER, [Rx power] - Option 2: SINR degradation - Other options are precluded Agreement in RAN4#111: - Use SINR for calibration purpose - FFS on performance metric for co-existence evaluation and requirements definition.. Agreement in RAN4#112: For inter-system interference (between AIOT and NR): - If SINR degradation is smaller than or equal to [1]dB, it can be considered that inter-system interference is negligible. - If SINR degradation is lager than [1]dB, consider following criteria: - Outage percentage consider SINR level with [10%] BLER - FFS on the outage percentage - Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR For intra-system interference (between AIOT and AIOT): - Outage percentage consider SINR level with [10%] BLER - FFS on the outage percentage

When we agreed on the outage percentage considering SINR level with [10%] BLER, the mapping between the SINR level and BLER is still not clear.

To better facilitate the discussion of the A-IoT performance evaluation, it’s better that RAN4 can reach some consensus on the mapping or at least the SINR level of this [10%] BLER criteria.

In this paper, we provide our LLS simulation results from RAN1 for helping determine a common basis in RAN4 to agree on a single value or a certain range of this SINR mapping to 10% BLER.

R1-2406652, Samsung Figure 3 shows the BLER performance of device 2 for R2D coverage evaluation, with a data size of 20 bits, CRC6, and an exact data rate of 5.77 kbps. Table 2. Link level simulation assumption for R2D coverage evaluation Figure 3. BLER performance of R2D transmission The required CNR for 10% BLER targets are as follows: - SFO = [0.1~1]*103 ppm, 10% BLER target: required CNR = 1.1 dB - SFO = [0.1~1]*103 ppm, 1% BLER target: required CNR = 11.35 dB Figure 4 shows the BLER performance of device 2 for R2D coverage evaluation, with a data size of 96 bits, CRC6, and an exact data rate of 7.06 kbps. Figure 4. BLER performance of R2D transmission

In our link-budget calculation in above reference, for R2D BLER of 10%, we results in CNR of around 1 dB for 20 information bits, and CNR of around 3dB for 96 information bits.

For D2R BLER of 10%, our LLS results shows the SINR would in the range of -10 to -5 dB.

Observation 1: For R2D BLER 10%, the CNR target is around 1dB for 0 information bits, and CNR of around 3dB for 96 information bits.

Observation 2: For D2R BLER 10%, the SINR target would be within the range of -10 to -5 dB.

3	Conclusion

For the Ambient IoT co-existence study assumption, we propose the following.

Observation 1: For R2D BLER 10%, the CNR target is around 1dB for 0 information bits, and CNR of around 3dB for 96 information bits.

Observation 2: For D2R BLER 10%, the SINR target would be within the range of -10 to -5 dB.

4	Reference

[1] R4-2414304, WF on co-existence evaluation for ambient IoT, CMCC