3GPP TSG-RAN4 Meeting #112bis	R4-2416302

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on NR-NTN Phase 3 UE RF

Source:	Samsung

Agenda item:	6.26.2.1

Document for:	Approval

1	Introduction

In previous discussions, most redCap requirements had been agreed, but some details are still need to be confirmed.

2	Discussions

In WF R4-2406610 [1], it was agreed to consider eRedCap BW reduction, however there’s still lack of details.

Sub-topic 1-5: eRedCap BW reduction Agreement: Specify both eRedCap variants, i.e. with and without bandwidth reduction.

In this contribution, we’d like to further discuss the BW reduction in details.

5.3I Channel bandwidth for RedCap The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths. 3MHz channel bandwidth is not applicable for RedCap UE in the current release.

Proposal 1: To clarify whether the legacy redcap bandwidth reduction in 5.3I of 38101-1 applied to the NTN redcap.

The bandwidth reduction would also impact the Refsens requirements, as shown in TN spec as follows

7.3I.3 Reference sensitivity power level for eRedCap For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply. For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, the reference sensitivity requirements for 5 MHz channel bandwidth with 15 kHz SCS defined in clause 7.3I.2 apply. These reference sensitivity requirements for 5 MHz channel bandwidth apply also for 10, 15 and 20 MHz channel bandwidths with 15 kHz SCS. In case the reference sensitivity requirements for 5 MHz channel bandwidth are not defined, the reference sensitivity requirements for 10 MHz channel bandwidth apply with the reference sensitivity level reduced by 3.2 dB, and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4 and 25 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 25RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 25 RBs for 15 kHz SCS. For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, for 30 kHz SCS, the reference sensitivity requirements defined for 10 MHz channel bandwidth in clause 7.3I.2 apply with reference sensitivity level reduced by 3.0 dB and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4 and 12 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 12RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths with 30 kHz SCS. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 12 RBs for 30 kHz SCS. NOTE: It is not necessary to repeat verification when same requirement applies for multiple channel bandwidths.

Proposal 2: To clarify whether the same principle applies to the REFSENS requirements when adopting the reduced bandwidth for NTN redcap.

3	Conclusion

For the NTN Phase3 UE RF requirements, we propose the following.

Proposal 1: To clarify whether the legacy redcap bandwidth reduction in 5.3I of 38101-1 applied to the NTN redcap.

Proposal 2: To clarify whether the same principle applies to the REFSENS requirements when adopting the reduced bandwidth for NTN redcap.

5	Reference

[1] R4-2406610, WF for Rel-19 NTN UE RF, Qualcomm