3GPP TSG-RAN WG4 Meeting #112bis					R4-2416312

Hefei, China, Oct 14 – Oct 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | draftCR | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.141-2 with Annex N Expected EIRP calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Perf |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is draft CR to TS 38.141-2 with new Annex N that provides EEIRP mask calculations according to work split agreed during RAN4#112 meeting in R4-2413509. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New annex N added with EIRP calculation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will not include details on EEIRP calculations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Annex N |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of draft CR>

# Annex N (normative):

# Expected EIRP calculation

# N.1	General

The key requirements of ITU Resolution 220 [31] are:

- compliance with values of Expected EIRP (EEIRP) above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to ITU Resolution 220 [31].

The EEIRP calculation is based on two averaging processes:

- Averaging of beamforming directions

- Averaging over horizontal and vertical angles

These two averaging processes are inter-related and should not be viewed as the order in which the calculation is done.

## N.1.1	The calculation for EEIRP mask

    - N.1.1.1 Angle definition

The angles in the spherical coordinate system used in this section is as shown in figure N.1.1.1-1, and the arrow indicates the direction of increasing angle from the base station perspective:

Figure N.1.1.1-1: Angle definition for EEIRP

    - N.1.1.2 Averaging over horizontal and vertical angles

One of the average calculations for the Expected EIRP is averaging the EIRP over horizontal and vertical angles. There are seven elevation angular ranges with different elevation bin sizes. Each bin is affectively a spherical strip (or spherical cap for the last range) bounded by θHL and θHH as shown in Figure N.1.1.2-1.

Figure N.1.1.2-1: Spherical strip (blue shade)

The average EIRP for each elevation bin is defined as:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ dφ dθ        ,                1

The EIRP EIRPθ,φ will be measured in discrete steps and equation (1) needs to be written in discrete form, and the next section describes how this is considered.

It should be noted that all the averaging is done on linear units.

    - N.1.1.2.1 Approximating the double integral in the EIRP averaging over an elevation angle range

The EEIRP evaluated over a spherical strip involves taking EIRP measurements, EIRPθ,φ, around the ‘point source’ (i.e. the Base Station) bounded by the elevation range θL and θH. Figure N.1.1.2.1-1 is an illustration of the measured EIRP points and the area of the spherical strip for one of the elevation bin. Note that the measured EIRP points are over three discrete elevation angles with equal number of points in azimuth.

Figure N.1.1.2.1-1: Measured EIRP points and the spherical strip

The double integral in equation (1) can be approximated in discrete form, and Figure N.1.1.2.1-2 illustrates some of the relevant discrete parameters used in the approximation. It is important to note that this approximation is based on equal grid method.

Figure N.1.1.2.1-2: Illustration of the parameters involved

For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth φ (over the range of [-π to π]), and with N and M points respectively, this average EIRP can be approximated to:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ ∆φ ∆θ                      (2)

By substituting the following:

dφ≈∆φ=2π÷M

dθ≈∆θ=π2÷N

the overall equation can be written as:

EIRPθL , θH=π2MNsinθH-sinθLn=nLnHm=1MEIRPθn,φm cosθn    ,          (3)

where

θn	is the midpoint between the elevation intervals

φm	is the midpoint between the azimuth intervals

nL	is the lowest elevation point within the θL , θH bounding range

nH	is the highest elevation point within the θL , θH bounding range

m	is the azimuth point within the -π , π bounding range

Note: the equation is assuming the angles in radians and the elevation angle is from the geographical horizon rather than from the antenna panel as used in 3GPP technical specifications previously. Anyhow, these different angle definitions are related and can be easily mapped.

    - N.1.1.2.2 Example illustration of average EIRP for an elevation angular range

This example uses the ‘elevation angular range’ of 30° ≤ θ < 60° (bin number 6) as defined at WRC-23 for ITU-R Resolution 220 [x]:

| Elevation angular range [θL ≤ θ < θH] (Degrees) | Bin number |
| --- | --- |
| 0<<5 | 1 |
| 5<<10 | 2 |
| 10<<15 | 3 |
| 15<<20 | 4 |
| 20<<30 | 5 |
| 30<<60 | 6 |
| 60<<90 | 7 |

Table N.1.1.2.2-1: The elevation angular ranges defined in ITU-R Resolution 220 [x]

- Let’s say we have a 10° interval for both azimuth and elevation. This will result to M = 36 (i.e. 360°/10°), and N = 9 (i.e. 90°/10°) points.

- Let’s consider the spherical strip between θL = 30° and θH = 60° (bin number 6)

- This will result to:

  - θn = 35°, 45° and 55°

  - φm = -175°, -165°, -155°, …, 155°, 165° and 175°

  - nL = 35°

  - nH = 55°

  - m = 1, 2, 3, …, 35, 36.

- Note: equation is in radians

- Figure N.1.1.2.2-1 is the illustration of this Example

Figure N.1.1.2.2-1: Illustration of the Example

It is worth noting that this Example shows that we have 3 vertical sets of EIRP points (i.e. θn = 35°, 45° and 55°) for 1 of the elevation angle range, i.e. bin number 6.

    - N.1.1.3 EEIRP calculation

The EEIRP is the mean power from the averaging of two different processes highlighted in clause N.1.1.2 where it averages the measurement points over the defined elevation angular range around the base station, and averages over the defined beamforming directions. This EEIRP calculation for both averaging processes are required to be sufficiently accurate, therefore, it is important to define appropriate angular step sizes (for averaging over the elevation angular range) and sufficient number of beamforming directions.

Number of stored datapoints=Number of elevation points ×Number of azimuthal points×Number of beamforming directions

Therefore, minimising any of the elevation/azimuthal points and/or the number of beamforming directions will significantly reduce the amount of data storage (due to the multiplicative effect) for the EEIRP calculation.

<End of draft CR>