3GPP TSG-RAN WG4 Meeting #112bis	R4-2416313

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	TP to TR 38.908 clause 6 – Conformance testing

Agenda item:	6.11.4

Document for:	Discussion

Introduction

This is text proposal to Technical Report TR 38.908 v0.1.0. with clause 6 for conformance testing and procedures.

TP to TR 38.908

<Start of TP>

# 6	Conformance testing requirement and procedures

## 6.1	General

The key requirements of ITU Resolution 220 [2] are:

- compliance with values of Expected EIRP (EEIRP) above the horizon and

- providing a calculation methodology to the theoretical procedure given in the Annex to ITU Resolution 220 [x].

The EEIRP calculation is based on two averaging processes:

- Averaging of beamforming directions

- Averaging over horizontal and vertical angles

These two averaging processes are inter-related and should not be viewed as the order in which the calculation is done.

## 6.2	The calculation for EEIRP mask

    - 6.2.1. Angle definition

The angles in the spherical coordinate system used in this section is as shown in figure 6.2.1-1, and the arrow indicates the direction of increasing angle from the base station perspective:

Figure 6.2.1-1: Angle definition for EEIRP

    - 6.2.1 Averaging over horizontal and vertical angles

One of the average calculations for the Expected EIRP is averaging the EIRP over horizontal and vertical angles. There are seven elevation angular ranges with different elevation bin sizes. Each bin is affectively a spherical strip (or spherical cap for the last range) bounded by θHL and θHH as shown in Figure 6.2.1-1.

Figure 6.2.1-1: Spherical strip (blue shade)

The average EIRP for each elevation bin is defined as:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ dφ dθ        ,                1

The EIRP EIRPθ,φ is be measured in discrete steps and equation (1) needs to be written in discrete form, and the next section describes how this is considered.

It should be noted that all the averaging is done on linear units.

    - 6.2.1.1 Approximating the double integral in the e.i.r.p. averaging over an elevation angle range

The EEIRP evaluated over a spherical strip involves taking EIRP measurements, EIRPθ,φ, around the ‘point source’ (i.e. the Base Station) bounded by the elevation range θL and θH. Figure 5.1.1.1-1 is an illustration of the measured e.i.r.p. points and the area of the spherical strip for one of the elevation bin. Note that the measured EIRP points are over two discrete elevation angles with equal number of points in azimuth.

Figure 6.2.1.1-1: Measured EIRP points and the spherical strip

The double integral in equation (1) can be approximated in discrete form, and Figure 6.2.1.1-2 illustrates some of the relevant discrete parameters used in the approximation. It is important to note that this approximation is based on equal grid method.

Figure 6.2.1.1-2: Illustration of the parameters involved

For an equal angle grid method where the grid spacing is uniform in both the elevation θ (over the range of [0 to π/2]) and azimuth φ (over the range of [-π to π]), and with N and M points respectively, this average e.i.r.p. can be approximated to:

EIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ ∆φ ∆θ                      (2)

By substituting the following:

dφ≈∆φ=2π÷M

dθ≈∆θ=π2÷N

the overall equation can be written as:

EIRPθL , θH=π2MNsinθH-sinθLn=nLnHm=1MEIRPθn,φm cosθn    ,          (3)

where

θn	is the midpoint between the elevation intervals

φm	is the midpoint between the azimuth intervals

nL	is the lowest elevation point within the θL , θH bounding range

nH	is the highest elevation point within the θL , θH bounding range

m	is the azimuth point within the -π , π bounding range

Note: the equation is assuming the angles in radians and the elevation angle is from the geographical horizon rather than from the antenna panel as used in 3GPP technical specifications previously. Anyhow, these different angle definitions are related and can be easily mapped.

    - 6.2.1.2 Example illustration of average e.i.r.p. for an elevation angular range

This example uses the ‘elevation angular range’ of 30° ≤ θ < 60° (bin number 6) as defined at WRC-23 for ITU-R Resolution 220 [x]:

| Elevation angular range [θL ≤ θ < θH] (Degrees) | Bin number |
| --- | --- |
| 0<<5 | 1 |
| 5<<10 | 2 |
| 10<<15 | 3 |
| 15<<20 | 4 |
| 20<<30 | 5 |
| 30<<60 | 6 |
| 60<<90 | 7 |

Table 6.2.1.2-1: The elevation angular ranges defined in ITU-R Resolution 220 [x]

- Let’s say we have a 10° interval for both azimuth and elevation. This will result to M = 36 (i.e. 360°/10°), and N = 9 (i.e. 90°/10°) points.

- Let’s consider the spherical strip between θL = 30° and θH = 60° (bin number 6)

- This will result to:

  - θn = 35°, 45° and 55°

  - φm = -175°, -165°, -155°, …, 155°, 165° and 175°

  - nL = 35°

  - nH = 55°

  - m = 1, 2, 3, …, 35, 36.

- Note: equation is in radians

- Figure 6.2.1.2-1 is the illustration of this Example

Figure 6.2.1.2-1: Illustration of the Example

It is worth noting that this Example shows that we have 3 vertical sets of e.i.r.p. points (i.e. θn = 35°, 45° and 55°) for 1 of the elevation angle range, i.e. bin number 6.

  - 6.2.2 EEIRP calculation

The EEIRP is the mean power from the averaging of two different processes highlighted in section 6.2.1 where it averages the measurement points over the defined elevation angular range around the base station, and averages over the defined beamforming directions. This EEIRP calculation for both averaging processes are required to be sufficiently accurate, therefore, it is important to define appropriate angular step sizes (for averaging over the elevation angular range) and sufficient number of beamforming directions.

Number of stored datapoints=Number of elevation points ×Number of azimuthal points×Number of beamforming directions

Therefore, minimising any of the elevation/azimuthal points and/or the number of beamforming directions will significantly reduce the amount of data storage (due to the multiplicative effect) for the EEIRP calculation.

- 6.2.2.1 Angular step size

Using equation (3), different angular step sizes can be considered for different elevation bin. This section analyses how different angular step sizes impact the accuracy of the EEIRP calculation.

First, the parameters defined for the studies leading to the discussions at WRC-23 are used as an example to illustrate the impact of different step size to the overall EEIRP. This is considered as the reference AAS beam and the relevant parameters are defined in Table 3.

|  | Parameter | Value |
| --- | --- | --- |
| 1 | Element gain | 5.5 dBi |
| 2 | Horizontal 3 dB beamwidth | 90° |
| 3 | Vertical 3 dB beamwidth | 90° |
| 4 | Front-to-back ratio (horizontal & vertical) | 30° for both |
| 5 | Antenna polarisation | Linear ±45° |
| 6 | Array configuration (Row x Column) | 16 x 8 |
| 7 | Element spacing (horizontal & vertical) | 0.5λ for both |
| 8 | Electrical tilt range (etilt) | 0° to 20° (in 1° step) |
| 9 | Electrical scan range (escan) | -60° to 60° (in 1° step) |

Table 6.2.2.1-1: AAS parameters used for analysis

The resulting AAS beam in the elevation and azimuth cut is shown in Figure 6.2.2.1-2, and the horizontal 3 dB beamwidth is approximately 12.6° whilst the vertical 3 dB beamwidth is approximately 6.3°.

Figure 6.2.2.1-2: Reference AAS beam

There are 7 elevation angular ranges (bins) with different bin size for the EEIRP limit (in the ITU-R Resolution 220), ranging from 5° to 30° as shown in Table 6.2.2.1.-2.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size (°) | Bin number |
| --- | --- | --- |
| 0<<5 | 5 | 1 |
| 5<<10 | 5 | 2 |
| 10<<15 | 5 | 3 |
| 15<<20 | 5 | 4 |
| 20<<30 | 10 | 5 |
| 30<<60 | 30 | 6 |
| 60<<90 | 30 | 7 |

Table 6.2.2.1-2: The elevation angular ranges defined in ITU-R Resolution 220 and the bin sizes

As the bin size for the first 4 elevation ranges are 5°, the maximum step size is therefore 5° for these elevation ranges. Similarly for bin 5 (20° ≤ θ < 30°), the maximum step size is 10°. This 10° step size is also set for bin 6 and 7 in this analysis.

Next, the analysis creates three sets of step sizes (Set A, Set B and Set C), and compared against a reference case where a step size of 0.25° is used for both elevation and azimuth. The beamforming directions are equally spaced with 1° resolution; with horizontal electrical scan (escan) of ±60° and elevation electrical tilt (etilt) of 0° to 20°. The result is shown in Table 6.2.1-3.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size | Set | Elevation step size | Azimuthal step size | Number of points per elevation bin | Number of azimuth points | Difference from reference (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0< θ <5 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.10 |
|  |  | C | 5° | 10° | 1 | 36 | 0.51 |
| 5< θ <10 | 5° | A | 1° | 10° | 5 | 36 | 0.02 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.11 |
|  |  | C | 5° | 10° | 1 | 36 | 0.20 |
| 10< θ <15 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.07 |
|  |  | C | 5° | 10° | 1 | 36 | 0.45 |
| 15< θ <20 | 5° | A | 1° | 10° | 5 | 36 | 0.00 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.02 |
|  |  | C | 5° | 10° | 1 | 36 | 0.10 |
| 20< θ <30 | 10° | A | 2° | 10° | 5 | 36 | 0.01 |
|  |  | B | 5° | 10° | 2 | 36 | 0.04 |
|  |  | C | 10° | 10° | 1 | 36 | 0.15 |
| 30< θ <60 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.01 |
|  |  | C | 10° | 10° | 3 | 36 | 0.26 |
| 60< θ <90 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.02 |
|  |  | C | 10° | 10° | 3 | 36 | 0.09 |
| Total number of measurement points |  | A | 1764 |  |  |  |  |
|  |  | B | 792 |  |  |  |  |
|  |  | C | 396 |  |  |  |  |
| Total number of points (with 0.25° step size for both elevation & azimuth) = 129 600 |  |  |  |  |  |  |  |

Table 6.2.1-3: Simulation result comparing the impact of different step size to the reference case

    - 6.2.2 Narrower AAS beam

The next analysis looks at the impact of narrower AAS beam by changing only the array configuration to 32 x 8 (row x column) while keeping the other parameters in Table 3 the same. This will create a narrower beam in the elevation plane as shown in Figure 10 where the horizontal 3 dB beamwidth is still approximately 12.6° whilst the vertical 3 dB beamwidth is reduced to approximately 3.2°.

Figure 6.2.2-1: AAS beam with array configuration of 32 x 8 (row x column)

The result is shown in Table 6, and it illustrates that the error is more significant when the elevation step size is larger than the vertical 3 dB beamwidth for some of the elevation bins.

| Elevation angular range [θL ≤ θ < θH] (°) | Bin size | Set | Elevation step size | Azimuthal step size | Number of points per elevation bin | Number of azimuth points | Difference from reference (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0< θ <5 | 5° | A | 1° | 10° | 5 | 36 | 0.04 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.35 |
|  |  | C | 5° | 10° | 1 | 36 | 3.50 |
| 5< θ <10 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.15 |
|  |  | C | 5° | 10° | 1 | 36 | 0.47 |
| 10< θ <15 | 5° | A | 1° | 10° | 5 | 36 | 0.01 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.09 |
|  |  | C | 5° | 10° | 1 | 36 | 0.26 |
| 15< θ <20 | 5° | A | 1° | 10° | 5 | 36 | 0.00 |
|  |  | B | 2.5° | 10° | 2 | 36 | 0.08 |
|  |  | C | 5° | 10° | 1 | 36 | 0.40 |
| 20< θ <30 | 10° | A | 2° | 10° | 5 | 36 | 0.02 |
|  |  | B | 5° | 10° | 2 | 36 | 0.12 |
|  |  | C | 10° | 10° | 1 | 36 | 0.31 |
| 30< θ <60 | 30° | A | 2.5° | 10° | 12 | 36 | 0.01 |
|  |  | B | 5° | 10° | 6 | 36 | 0.17 |
|  |  | C | 10° | 10° | 3 | 36 | 0.10 |
| 60< θ <90 | 30° | A | 2.5° | 10° | 12 | 36 | 0.00 |
|  |  | B | 5° | 10° | 6 | 36 | 0.00 |
|  |  | C | 10° | 10° | 3 | 36 | 0.11 |
| Total number of measurement points |  | A | 1764 |  |  |  |  |
|  |  | B | 792 |  |  |  |  |
|  |  | C | 396 |  |  |  |  |
| Total number of points (with 0.25° step size for both elevation & azimuth) = 129 600 |  |  |  |  |  |  |  |

Table 6.2.2-2: Simulation result for AAS beam array 32 x 8 (row x column)

## 6.3	Test beams

The EEIRP calculation involves calculating the average e.i.r.p. over a defined set of elevation angles (around the base station, i.e. 360° azimuth) for the full range of beamforming directions. An example of the full range of beamforming directions is shown in Figure 6.3.1, with the electrical steering ranges set to 0° ≤ θetilt ≤ 16° and -60° ≤ ϕescan ≤ +60°. The number of beamforming directions for this example is 2057 for a 1° steering resolution (i.e. 17 x 121).

Figure 6.3-1: Reference Set of beamforming directions

Theoretically, this full range of beamforming directions are meant to be used for the EEIRP calculation, however, a representative subset of these beamforming directions can be considered as long as the overall averaging of the EEIRP is sufficiently accurate (as described in the Annex to the ITU Resolution 220). The motivation to create a small subset of beamforming directions (or Test Beams) is to reduce the test effort needed to calculate the EEIRP.

    - 6.3.1 Subset of beamforming directions

It is undoubtably that using the full range of beamforming directions will amount to extensive test effort as the e.i.r.p. for the entire upper hemisphere (around the base station) needs to be measured for every beamforming direction. Simplistically, halving the number of beamforming directions could potentially half the amount of effort needed to comply with the EEIRP requirement. Therefore, it is important to find a representative subset of these beamforming directions (Test Beams set) for the EEIRP calculation.

Three different sets of Test Beams are investigated to evaluate the accuracy compared to the Reference Set (i.e. full range of beamforming directions). The three sets are shown in Figure 6.3.1-1 to Figure 6.3.1-3 with different number of Test Beams and locations. This is based on the example electrical steering ranges shown in Figure 6.3-1.

Figure 6.3.1-1: Set 1 (25 Beams)

Figure 6.3.1-2: Set 2 (9 Beams – rectangular)

Figure 6.3.1-3: Set 3 (9 Beams – diamond)

It is important to note that no weighting is needed for each of these Test Beams as each Test Beams set is intended to be a representation of the Reference Set shown in Figure 6.3-1. The Reference Set fulfils the requirement that the EEIRP is calculated over the entire electrical steering range.

    - 6.3.2 Antenna configurations for evaluation

These three different Test Beams sets are evaluated over three different antenna configurations to show how suitable these Test Beams sets are. The antenna parameters for these configurations are shown in Table 6.3.2-1 where Configuration 1 and 2 are for the non-sub-array model, whilst Config 3 is for the sub-array model.

| Parameter |  | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Config 1 | Config 2 | Config 3 |
| Element | Vertical HPBW (°) | 90 | 90 | 65 |
|  | Horizontal HPBW (°) | 90 | 90 | 90 |
|  | Element gain (dBi) | 5.5 | 5.5 | 6.4 |
|  | Front-to-back ratio (H/V) (dB) | 30 | 30 | 30 |
|  | Element power (dBm/100 MHz) | 22 | 16 | 21 |
| Array configuration | Number of rows | 16 | 32 | 8 |
|  | Number of columns | 8 | 16 | 16 |
|  | Sub-array | - | - | 3x1 |
|  | Sub-array pre-tilt: (°) | - | - | 6 |
|  | Vertical element separation (λ) | 0.5 | 0.5 | 2.1 |
|  | Horizontal element separation (λ) | 0.5 | 0.5 | 0.5 |

Table 6.3.2-1: Antenna parameters

These parameters are just some example antenna configurations for the purpose of evaluating the suitability of the Test Beams sets rather than for evaluating against the specific EEIRP values required for the different elevation ranges. In addition, no specific antenna techniques such as sidelobe suppression are used where these techniques could potentially lower the EIRP of the peak of the main lobe and sidelobes.

The antenna patterns generated from these parameters are shown in Figure 6.3.2-1 to Figure 6.3.2-3.

Figure 6.3.2-1: Antenna pattern of Configuration 1

The antenna configuration in Figure 6.3.2-1 is based on the parameters which was provided to ITU-R for the used in sharing studies which led to ITU Resolution 220.

Figure 6.3.2.2: Antenna pattern of Configuration 2

The antenna configuration in Figure 6.3.2-2 is based on Configuration 1 with doubling the number of antenna elements in the horizontal and vertical domain and adjusting the power per element accordingly.

Figure 6.3.2-3: Antenna pattern of Configuration 3

The antenna configuration in Figure 6.3.2-3 is based on a sub-array model with a sub-array pre-tilt of 6°.

    - 6.3.3 Evaluation of Test Beams set

The Test Beams sets in clause 6.3.1 are used to evaluate against the antenna configurations in clause 6.3.2. The Reference Set illustrated in Figure 6.3.1-1 are based on 1° steering resolution, and the calculation of the EEIRP above the horizon (upper hemisphere) is based on 0.5° sampling resolution in order to capture the beam as accurately as possible.

The antenna pattern generated from these configurations were normalised, and just to note that the normalisation factor is only up to around 1.5 dB. The results shown in Table 6.3.3-1 to Table 6.3.3-3 is in the form of difference (delta) in dB comparing the EEIRP generated from those Test Beams set to the Reference Set as shown in Figure 6.3-1.

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 1 (25 Beams) |  |  |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0<q<5 | 1.5042 | 2.9003 | 2.8642 |
| 2 | 5<q<10 | 0.3517 | 0.9653 | 1.6473 |
| 3 | 10<q<15 | 0.59 | -0.2142 | 0.978 |
| 4 | 15<q<20 | 0.0508 | 0.7039 | -0.2957 |
| 5 | 20<q<30 | -0.1745 | -0.1444 | 0.7201 |
| 6 | 30<q<60 | -0.655 | -0.6488 | -0.0819 |
| 7 | 60<q<90 | -0.7833 | -0.8611 | -0.0944 |

Table 6.3.3-1: Difference of Test Beams set 1 from Reference Set

For Test Beams set 1, the largest difference is around 2.9 dB of over-estimation in elevation bin 1 (Configuration 2 and Configuration 3), and up to 0.9 dB of under-estimation in elevation bin 7 (Configuration 2).

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 2 (9 Beams - rectangular) |  |  |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0<q<5 | 3.2695 | 4.8185 | 4.7302 |
| 2 | 5<q<10 | -0.1618 | 1.8493 | 3.1198 |
| 3 | 10<q<15 | 0.903 | 0.1029 | 2.4126 |
| 4 | 15<q<20 | 1.1424 | 0.8559 | -4.7315 |
| 5 | 20<q<30 | -0.9639 | 0.074 | 1.7426 |
| 6 | 30<q<60 | -1.3376 | -1.6784 | -0.5279 |
| 7 | 60<q<90 | 0.0858 | 0.5385 | 3.0346 |

Table 6.3.3-2: Difference of Test Beams set 2 from Reference Set

For Test Beams set 2, the largest difference is around 4.8 dB of over-estimation in elevation bin 1 (Configuration 2), and up to 4.7 dB of under-estimation in elevation bin 4 (Configuration 3).

| Elevation Bin | Elevation angular range | Delta (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Set 3 (9 Beams - diamond) |  |  |
|  |  | Config 1 | Config 2 | Config 3 |
| 1 | 0<q<5 | -0.3157 | 0.7923 | 0.7893 |
| 2 | 5<q<10 | -0.0121 | 0.0559 | 0.0498 |
| 3 | 10<q<15 | 0.1368 | -0.5011 | -0.8413 |
| 4 | 15<q<20 | -0.2029 | 0.5704 | 0.4144 |
| 5 | 20<q<30 | -0.0294 | -0.3323 | -0.4852 |
| 6 | 30<q<60 | -0.0323 | 0.0938 | 0.2308 |
| 7 | 60<q<90 | 0.7835 | 1.029 | 3.1612 |

Table 6.3.3.3: Difference of Test Beams set 3 from Reference Set

For Test Beams set 3, the largest difference is around 3.2 dB of over-estimation in elevation bin 7 (Configuration 3), and up to 0.8 dB of under-estimation in elevation bin 3 (Configuration 3).

## 6.4	Measurement uncertainty and confidence intervals

Resolution 220 of the ITU Radio Regulation [2] contains the EEIRP technical condition for the protection of the FSS (Earth-to-space) and the Annex accompanying this Resolution outlines the theoretical calculation of this EEIRP. This Annex describes that the EEIRP is calculated in two steps where the beamforming directions are averaged in step 1 while step 2 describes the averaging of the required horizontal and vertical angles. The averaging of these two steps needs to be accurate and this can be done in different ways where one example suggested in the Resolution is by means of using confidence interval of 95%. This does not exclude other metrics to evaluate the accuracy of the EEIRP calculation that is done in step 1 and step 2.

For conformance testing, it could be more practical to define the accuracy by means that has already be done such as measurement uncertainty or even evaluating the over-estimation or under-estimation of the EEIRP calculation due to sampling of the EIRP when the measurements are done and/or using a representative Test Beams set. As described, the Test Beams set is a subset of all the beamforming directions within the steering range of the base station and this Test Beams set can be considered as long as the overall averaging of the EEIRP is sufficiently accurate.

From the regulation perspective, under-estimation of the EEIRP compared to the required limits is more concerning than over-estimation. Therefore, evaluating the accuracy of the EEIRP calculation can be based solely on the under-estimation of the EEIRP calculation.