3GPP TSG-RAN WG4 Meeting #112bis	R4-2416314

Maastricht, Netherlands, August 19th – 23rd 2024

Agenda Item:	6.17.2.1

Source:	Ericsson

Title:	CSI compression simulations and considerations

Document for:	Discussion

# 1	Introduction

During RAN#105, the decision was taken to extend the study on CSI compression for the remainder of Release 19. In our understanding, a key purpose for the study is to improve understanding of the following issues:

- How to enable interoperability between different network and UE combinations

- How to ensure that the UE provides consistent and known behaviour to the network

If interoperability is not ensured then, even if the 2-sided CSI compression provides useful gains, the value of standardizing the method in 3GPP vastly diminishes (If collaboration between specific network and UE vendors is needed on a proprietary basis to obtain the gains, then the vendors can collaborate and achieve the gains, but developing the 3GPP standard in either RAN1 or RAN4 adds little value). If predictability or measurability of the performance is not obtained then it may be hard for the network to efficiently manage the diversity of UEs and achieve KPIs, regardless of gains.

Whether or not the CSI compression use case is eventually standardized, the learnings on these two issues may in general be useful for 2-sided use cases (although specific solutions may need to differ depending on the use case).

# 2	Achieving interoperability for the 2-sided use case

The value of the 3GPP specification is that it (in principle) enables an ecosystem with a diversity of terminals and UEs can be provided to the market. UEs of different vendors can operate in a network, and UEs can operate in networks provided by different network vendors. Robust Uu specifications in RAN1/2 together with performance requirements in RAN4 and conformance testing in RAN5 ensure this. (To some extent, IODT is still needed by UEs, but this is vastly simplified by the existence of the RAN1-5 specifications).

2-sided AI challenges this paradigm since it implies the need to co-ordinate the models on the network and UE side. Without some measures to ensure interoperability, every network node would need to simultaneously run decoder models for every chipset AI variant. Similarly, every chipset variant would need to be able to obtain and run a model for every network. Development work would need to take place between every combination of network and chipset.

An obvious way to achieve interoperability would be to standardize the model on at least one side. This is analogous to e.g. channel coding, in which the channel encoder is standardized so that all UEs implement the same encoder and all networks can develop decoders based on the standardized encoder. However, directly standardizing an encoder (or decoder) that is mandatory to implement may prove inflexible for different AI platforms or training. This is the basic tradeoff involved in standardization; interoperability vs flexibility.

An alternative, known as “option 3” in RAN4 is to standardize a test decoder together with performance requirements in specified test conditions. UEs would be expected to demonstrate that they can meet the performance requirements in the test conditions when their output is decoded using the test decoder. Network equipment would be free to implement decoders as preferred but would need to take into account that the UE meets performance requirements based on the test decoder.

RAN1 has discussed collaboration between the UE and network vendor to derive models. If taken together with the RAN4 requirements, then this would mean the following:

- UE encoders (and potentially network decoders) would always achieve minimum performance across the requirement conditions based on the test decoder.

- If a UE would enter a network with which it has not collaborated (or alternatively, a network would encounter a UE with which it has not collaborated) then at least the minimum performance would be achieved.

- If a NW and UE vendor collaborate, then some extended performance (e.g. for a specific scenario) may be achieved. Also, the RAN4 performance requirements would continue be achieved with such collaborations.

Our understanding is that in RAN1, there has been some suggestion that the RAN4 test decoder could be a “simple” decoder that is low complexity and not trained in a variety of conditions. In our understanding, however, it is important that the RAN4 test decoder is well trained for all likely conditions. It is also important that the requirement conditions cover reasonably well the range of expected conditions (e.g. channel propagation, Doppler etc.) in which CSI should be compressed.

The RAN4 test decoder in option 3 should be trained with a sufficiently wide and rich dataset that it can provide good performance over the range of normal operating conditions (i.e. channel type, Doppler etc.)

The reason for the above observation is that the test decoder in effect specifies a latent space that the UE encoder must cover. Furthermore, the requirement conditions ensure that the UE encoder can provide good performance over the range of channel conditions, Doppler etc. that the AI model may encounter. If the requirement conditions are too narrow, then the performance of the UE within networks with which it has not collaborated may not be guaranteed. If the test model is too simple, then the decoder may not be capable of ensuring proper performance in all of the requirement conditions.

A sufficiently well trained test decoder and sufficiently rich range of test conditions is needed to ensure that the test decoder approach provides basic interoperability.

In our understanding, for option 3 it is useful and important to also specify a reference encoder. The reason is that the reference encoder can also capture the latent space of the decoder and can be used by networks for developing their real decoders. Furthermore, the reference encoder can be used to verify the implementation of the test decoder in test equipment. An additional advantage of a reference encoder is that it could be directly used by any UE vendor that would prefer not to develop their own encoder.

For option 3, also provide a reference encoder.

The reference encoder is useful for training network side decoders and for validating TE implementations.

# 3	Option 4 considerations

Option 3 (standardized test decoder) is intended to provide a means to ensure basic interoperability for 2-sided CSI compression. Option 4 is a variant on option 3, for which instead of directly standardizing a test decoder, instead sufficient information is standardized such that any TE vendor can develop a test decoder. It is important that then the decoders developed by all TE vendors have the same latent space representation and performance so that testing on different equipment is consistent. The information provided according to option 4 must ensure this.

For option 4, it is essential to ensure that the information is sufficient that all TE vendors get to the same ordering of the latent space representation

In the WF from RAN4#112, it is suggested that a dataset could be included in the specification and the question is asked whether anything more is needed. We note that, since performance tests will be specified, it is obvious that also the test criteria will be available.

With the dataset and test conditions, it is possible for any TE vendor to develop a decoder (and encoder) that can achieve the performance requirements. However, it is far from obvious that the different test decoders developed by different test vendors would contain the same ordering of the latent space. A mechanism is required of ensuring that the same latent space ordering is achieved across all TE vendors.

A good way to achieve latent space ordering across all TE vendors is to standardize a reference encoder.

We welcome other suggestions as to how such ordering could be achieved.

Discuss means to ensure convergence in the latent space representation for differently trained decoders for option 4. Standardization of a reference decoder is one potential option.

# 4. “Monitoring” considerations

“Monitoring” is a term used to refer to the action on either the UE or network side of determining whether the model, in current operation can continue to meet performance expectations. It is important to consider that monitoring should not just indicate whether the model is functioning or not in a binary sense. If the CSI recovered from the decoder is obviously trash and the performance is poor, then it is obvious that a switch back to non-AI functionality is needed. The challenge with monitoring is to detect when the performance has degraded to a non-negligible, but not catastrophic extent (e.g. 5-10% degradation).

It is important that monitoring can observe and quantify partial reduction in performance (not just a large or catastrophic loss).

Such degradations could lead to unacceptable degradations in network performance. However, identifying that degradations have come from a specific AI model may be difficult to achieve. What may also be difficult is to identify whether changing to non-AI may actually improve performance. The reason is that UE performance depends on a very large range of factors, including but not limited to e.g. the radio environment, interference conditions, RF performance, adjacent channel interferers, network traffic and scheduling and potentially interaction between a variety of AI algorithms in future UEs.

It is non-trivial to ensure that a monitoring function can determine the performance of an AI model in isolation, since in the real world a very large number of interacting factors influence performance.

Monitoring of 2-sided models is further complicated by the fact that it would not be clear where the solution to performance degradation would lie, on the network side or on the UE side.

For the 2-sided model, a further challenge with monitoring is allocating observed performance degradation to the two half models.

The monitoring solutions should be discussed in RAN1/2. From a RAN4 perspective, though, the following considerations are important. If the UE sends measurements, or monitoring reports to the network, then it is important that the measurements or reports are consistent and comparable between different UE types. For measurements, this may necessitate setting requirements on e.g. accuracy, timing of measurements in a similar style to RRM. For UE monitoring reports, it is important that different UEs provide similar reports when their models have a similar level of reliability. However, since the reliability of different UEs models may differ, a discussion is needed on how requirement definition and testing could be carried out in a way that forces UEs to specific reliability levels and then tests the output. Also, a definition of metrics for testing of the output is needed.

Monitoring requirements should define the constancy/accuracy of monitoring reports from the UE if they are defined.

Testability of monitoring report accuracy needs discussion, since the reliability of underlying models will differ between UEs and a test of accuracy needs to force UEs into known reliability levels.

If monitoring is entirely on the network side then it may be discussed in RAN1/2. Since the network needs to manage a large amount of UEs, the complexity of monitoring, ability to diagnose specific models within the UE need improvement and also identify what improvement is needed may be challenges.

# 4. Simulation results

## 4.1	CSI compression performance based on the agreed parameters

During and after RAN4#111, an exercise was commenced to assess the viability for option 3. This involves identifying an agreed structure for a test model as well as a training dataset and evaluation criteria. Following RAN4#111, the training set, test set and structure were identified in a relatively arbitrary manner. The intention was not to identify a best training set and decoder structure, but rather to bypass this step, in order to experiment with model aligning and sharing between companies. In a real normative phase, this step in itself would be difficult and time consuming.

The current RAN4 study has not identified the best training set or decoder structure, since the aim is to investigate the ability to converge to a common model, not to select the right model.

During RAN4#112, first results with the assumed dataset and structure were presented and the following general steps were identified to arrive at a test decoder:

- Check on performance alignment -> see simulation results from contributing companies

  - repeat simulations until good alignment is achieved

  - move to next step after alignment

- Share models(encoder or decoder or both)/datasets (training/testing/inference)

  - framework to share data among companies(dataset/model) to be discussed

- Select one or more decoder for further analysis

  - selection criteria to be discussed

- Each company brings results for training of “own encoder” with selected decoder(s)

  - performance alignment to be checked/discussed

  - using “own” data or data shared by other companies

- Conclude on overall feasibility of Option 3

  - consider the conditions under which Option 3 is feasible if found feasible

  - FFS on what the feasibility criteria

It was observed that there was still significant divergence between different companies results. As part of step 1 of the above sequence, a round of alignment on assumptions on training methodology and model structure was performed. The details of the alignment are provided in the annex.

Based on the alignment assumptions, our results are as follows.

Table 1	SGCS performance for the agreed RAN4 CNN-based model with some baselines.

| Compression mechanisms | SGCS | Diff |
| --- | --- | --- |
| Rel. 16 eType II PC 1 | 0.7001 |  |
| CNN-based compression (unquantized) | 0.7584 | 8.3% |
| CNN-based compression (64 bit) | 0.6977 | -0.3% |

To give a better picture on the performance of the agreed model, we plot the SGCS of the three compression mechanisms in the above table in the following figure.

Figure 1 CDF of SGCS Performance for the agreed model both for unquantized and quantized cases. Performance of Rel. 16 eType II with PC is plotted for comparison.

From the above figure, we can observe that the mean SGCS value may not entirely represent the SGCS characteristic of a CSI compression method. Looking at the quantized CNN-based compression vs the Rel. 16 eType II for example, there is a region in which the CNN model performs better while in the other region, the legacy mechanism performs better. In short, a better representation of performance evaluation may be needed on top of the mean value. If a more precise performance characteristic is desired, a per-sample basis comparison may also be used as an option.

As well as mean SGCS, SGCS distributions should be compared.

Note that the above results are obtained with the agreed parameters and without any intention to optimize, e.g., the hyperparameters. For example, different initialization, batch size, and learning rate may offer different performances with some of them may potentially give better performance compared to the above results.

## 4.2	Some statistics for comparing training/test datasets

Except for training related parameters, the discrepancy on the SGCS value reported by companies in the last meeting may also comes from differences in the channel characteristic used for training and/or testing. In the below figure, the average amplitude of the channel (averaged across resource block and RX-TX pairs) of all samples in the training/test datasets are plotted in a CDF format with semi-log axis.

Figure 2 CDF of average channel’s amplitude.

Below we present a related view, but that is reflecting received power on MIMO layer 1. We consider a frequency average of the largest eigenvalue of the subband transmit-covariance, per channel realization, i.e., 1Ff=1FλmaxH,f, where f denotes a subband of the channel realization H. This number is normalized by comparing to the average of this number, across a set of channel realizations, i.e., EH1Ff=1FλmaxH,f. We note that the CDF in Figure 1 shows a similar characteristic to the CDF of channel amplitudes above, which is likely explained by the strongest layer contribution to a large part of the total received energy.

Figure 3 CDF of relative eigenvalues per channel realization, for a UMA scenario with 80% indoor user (3 km/h) and 20% outdoor users (30 km/h). The set contains 100k samples.

Figure 4 CDF of relative eigenvalues per channel realization, for a UMA scenario with 100% outdoor users (30 km/h). The set contains 20k samples.

Figure 5 CDF of relative eigenvalues per channel realization, for a UMA scenario with 100% indoor users (3 km/h). The set contains 80k samples.

Figure 2 – Figure 4 generally shows the basic channel characteristic that may or may not impacting the performance of a certain compression mechanism (either or both the legacy mechanism or AI-based compression). While it is useful as a source to give a better understanding on what degree companies are aligned in terms of dataset, it may also be useful to obtain a hint on the compressibility aspect of the channel. One possible option to obtain such goal is to compare the performance from companies when the legacy mechanism is used. Note, however, the legacy mechanism may contain several UE implementation steps which may differ between companies.

To simplify the comparison, the SGCS performance of a simplified version of Rel. 16 eType II is plotted in the in Figure 5. In the simplified version, only 2 and 4 strongest beams are selected, strongest in the sense of wideband, to represent the compression. There is no selection on the taps, no selection on NZC, and no quantization for the compression.

(a) Indoor 3 km/h scenario                            (b) Outdoor 30 km/h scenario

Figure 6 SGCS performance of channel when 2 and 4 beams are selected as CSI reports. No selection on the taps, no selection on NZC, and no quantization.

From the above figure, we can observe that two different channel statistic/characteristic may give different SGCS performance. This may suggest that different SGCS performance reported by companies may come from different statistics of the channels.

Compare the following between companies to obtain insight into the degree of alignment of the training/testsets: Amplitude CDF, strongest Eigenvalue CDF, SGCS for 2 and/or 4 strongest beams.

# Conclusion

In the previous sections we made the following observations:

Observation 1	A sufficiently well trained test decoder and sufficiently rich range of test conditions is needed to ensure that the test decoder approach provides basic interoperability.

Observation 2	The reference encoder is useful for training network side decoders and for validating TE implementations.

Observation 3	For option 4, it is essential to ensure that the information is sufficient that all TE vendors get to the same ordering of the latent space representation

Observation 4	It is important that monitoring can observe and quantify partial reduction in performance (not just a large or catastrophic loss).

Observation 5	It is non-trivial to ensure that a monitoring function can determine the performance of an AI model in isolation, since in the real world a very large number of interacting factors influence performance.

Observation 6	For the 2-sided model, a further challenge with monitoring is allocating observed performance degradation to the two half models.

Observation 7	Monitoring requirements should define the constancy/accuracy of monitoring reports from the UE if they are defined.

Observation 8	Testability of monitoring report accuracy needs discussion, since the reliability of underlying models will differ between UEs and a test of accuracy needs to force UEs into known reliability levels.

Observation 9	The current RAN4 study has not identified the best training set or decoder structure, since the aim is to investigate the ability to converge to a common model, not to select the right model.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The RAN4 test decoder in option 3 should be trained with a sufficiently wide and rich dataset that it can provide good performance over the range of normal operating conditions (i.e. channel type, Doppler etc.)

Proposal 2	For option 3, also provide a reference encoder.

Proposal 3	Discuss means to ensure convergence in the latent space representation for differently trained decoders for option 4. Standardization of a reference decoder is one potential option.

Proposal 4	As well as mean SGCS, SGCS distributions should be compared.

Proposal 5	Compare the following between companies to obtain insight into the degree of alignment of the training/testsets: Amplitude CDF, strongest Eigenvalue CDF, SGCS for 2 and/or 4 strongest beams.

# Annex

| Parameter/Configurations | Options/Values | Company comments |
| --- | --- | --- |
| Channel/data-set related |  |  |
| UE antenna element assumptions | - Option 1: Omni-directional antenna elements (Ericsson) - Option 2: Directional antenna elements - Option 3: Other | [Apple] We provided 2 simulation results: Directional (both horizontal/vertical) Omni (both horizontal/vertical) [QC] Option 1 (both horizontal and vertical) [Intel] Option 1 [OPPO]:Option1 [CMCC] option 1 |
|  | Configuration for alignment: - Ideal omni-directional antenna elements following RAN1 assumptions |  |
| Sub-band channel averaging | - [Nokia, QC, Apple, vivo, Ericsson,OPPO] Average of covariance channel matrix per RBs in the sub-band before SVD - [ZTE]: Per sub-band - [CMCC]: Average of raw channel matrix per RBs in the sub-band before SVD | CATT: Prefer not to do the averaging before SVD. |
|  | Configuration for alignment: - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected |  |
| Any other pre-processing | - To be listed if used - [QC, Ericsson] Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - [OPPO] Normalize the phase of all entries of the eigenvector based on the phase of the last entry of the eigenvector - [Apple]: Normalize the phase of all entries of the eigenvector based on the phase of that entry that has the maximum amplitude averaged over the sub-bands | [Intel] prefer no other pre-processing |
|  | Configuration for alignment: - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1. |  |
| Data collection assumptions | - [Nokia, CATT, ZTE, Ericsson, CMCC] Each channel realization is collected from independent UE drop - Multiple channel realization can be collected from the same UE - Other - [QC] Multiple channels realizations are collected from multiple UEs and multiple drops of same UE - [Apple, Intel, OPPO]: Multiple drops. Each drop collect data from multiple UEs. Additionally collect samples from channel realization from each UE spaced at de-correlated interval. |  |
|  | Configuration for alignment: - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs. |  |
| Training dataset size | - [Nokia]: 630K - [QC]: 450K - [Samsung]: 25K - [R&S]: 614,440 - [MTK]: 700K - [Ericsson]: 800K - [Intel]: 200K - [CATT]: 102,600 - [ZTE]: 600k - [Apple]: 600K - [vivo]: 243000 - [OPPO]：600k - [CMCC]: 100K |  |
|  | Configuration for alignment: - Aim to use the dataset around 600K. - At least 100K sample to be used. |  |
| Dataset quantization | - Float32 |  |
| Other data-set characteristics/statistics | - Companies to propose |  |
|  | - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII. |  |
| Model-related |  |  |
| Quantization of encoder output | - [Nokia]: Non-trainable quantization (bypassed during training) - [QC, CMCC] Quantization aware training - [Apple]: Quantization in forward and approximation in backward to enable learning. (avoid zero gradient of round function) - [vivo] VQ with quantization aware training - [Intel] Quantization aware training. Scalar quantization with non-trainable quantization grid. - [Ericsson] 2 bit scalar quantization, quantizer aware training | CATT: Consider to align the activation function and quantization method. |
|  | - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization |  |
| Normalization before quantization in the encoder | - [Nokia, Ericsson]: Normalization with tanh function after FC layer - [CATT, ZTE, vivo, CMCC]: Sigmoid function used after FC layer - [QC] Normalization not done due to the use of quantization aware training - [Apple]: Normalization with tanh function after FC layer. We don’t apply inverse of that in de-quantization - [Intel] Sigmoid after FC layer. Inverse sigmoid after de-quantization stage in decoder. |  |
|  | Configuration for alignment: - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional. |  |
| Use and configuration of drop-out | - [Nokia]: Use of on single drop-out layer after FC layer in the encoder and decoder, Drop-out rate = 0.4. - [CATT, QC, ZTE, Apple, vivo, Intel, Ericsson, CMCC]: No drop-out used. |  |
|  | Configuration for alignment: - Not to use drop-out layers |  |
| Training loss function, use of regularization | - [Nokia]: - SGCS with L2 regularization with weight decay, λ = 10-2 - [QC]: SGCS + additional terms for quantization aware training - [Apple, CATT, CMCC]: 1- SGCS - [R&S, MTK, Intel]: SGCS - [Ericsson]: NMSE - [ZTE]: MSE - [vivo]: 1-SGCS plus VQ loss for encoder output (MSE between original and quantized encoder output) - [Intel]: SGCS, no L1/L2 regularization |  |
|  | Configuration for alignment: - Use (1-SGCS) as loss function as reference |  |
| Optimizer | - [Samsung, QCM, Apple, MTK, Intel, CATT, Ericsson, CMCC]: Adam - [Nokia]: RMSProp (Root Mean Square Propagation) - [ZTE, vivo]: AdamW |  |
|  | Configuration for alignment: - Use Adam |  |
| Learning rate | - [Nokia]: - [QC, R&S, CATT, Ericsson]: 10^-4 - [R&S]: Learning rate adaptation: Reduce on plateau by factor 0.5 with patience of 50 epochs. - [Apple]: 0.0025 - [Samsung, Intel, MTK, CMCC]: 0.001 - [ZTE]:1.5e-3 - [vivo]: Warm-up to initial lr 1e-3 and then linearly decay to final lr 8e-4, update after each batch |  |
|  | Configuration for alignment: - Fixed learning rate of 10^-4 |  |
| Batch size | - [Nokia]:1024 - [QC, MTK, CMCC]: 128 - [Apple]: 256 - [Samsung]: 200 - [R&S, CATT, vivo, Ericsson]: 512 - [Ericsson]: 32 - [Intel]: 10 - [ZTE]:400 |  |
|  | Configuration for alignment: - 256 |  |
| (Max) Number of epochs | - [QC]: 150 - [Apple, vivo, Intel]: 100 - [R&S, Nokia, CATT]: 200 - [MTK, ZTE]: 300 - [CMCC]: 500 |  |
|  | Configuration for alignment: - 150 epochs |  |
| Stopping criteria | - [Nokia, Apple, CATT, QC, ZTE, Intel, CMCC]: No change in metric over a given number of epochs - Validation patience: [Intel, Nokia] 10, [QC] 50 - + Ericsson with constraint on max Epochs - [vivo] Stop when reaching the configured number of epochs |  |
|  | Configuration for alignment: - No improvement over 25 epochs. |  |
| Weight initialization | - [Apple, CATT, QC, vivo, Intel, CMCC]: Default - Ericsson: Variance scaling |  |
|  | Configuration for alignment: - Default |  |
| Kernel Parameters: - Dilation - Padding Type The value of groups (VG) | - [CMCC, Samsung, Vivo, CATT]: Dilation:1, Padding type=0, VG= 1 - [QC, ZTE, Apple, Intel]: Default |  |
|  | Configuration for alignment: - Default |  |