3GPP TSG-RAN WG4 Meeting #112bis	R4-2416316

Hefei, China, October 14th – 18th 2024

Agenda Item:	5.22.3

Source:	Ericsson

Title:	HAPS ACLR and ACS

Document for:	Discussion

# 1	Introduction

During RAN#105, a WI was initiated to add the possibility of HAPS to certain bands according to the outcome of WRC-23. The bands are around 1.7GHz, 2.5GHz and 700MHz.

Up to now, HAPS has been defined for band n1. When HAPS was defined, co-existence simulations were carried out to establish the acceptable ACLR and ACS for HAPS basestations, and to ensure that UEs would not interfere to TN basestations when HAPS is operated.

For the current WI, there is a need to either confirm the ACLR and ACS levels determined for HAPS or to create new values. The existing co-existence study for 2GHz should act as a baseline.

In this contribution, it is demonstrated that the existing ACLR and ACS are more than sufficient for all of the considered frequencies by means of link budget considerations.

# 2. Existing co-existence simulations and ACLR/ACS

The existing co-existence simulations are documented in 38.863. For ACLR, and ACLR of down to 20 dB is seen as acceptable to  avoid throughput losses of more than 5% in the TN. The ACLR was set at 45dB (same as TN BS), and thus there is a margin of around 25 dB.

For ACS, the simulation results suggest that an ACS of around 15 dB is sufficient to avoid degradation of more than 5% to the HAPS network. The ACS was set at 46dB (same as TN BS) and thus a margin of more than 30 dB exists.

# 3. Downlink link budget and noise floor considerations

It may be expected that the HAPS has near LoS propagation to the TN network on the ground. Assuming a HAPS altitude of 20km, then the minimum distance of the HAPS to a TN UE is 20 km.

Table 1 compares the pathloss between the HAPS and the ground considering FSPL for 0.7, 1.7, 2 and 2.5GHz.

| Frequency | FSPL | Difference in FSPL compared to 2GHz |
| --- | --- | --- |
| 700 MHz | 115 dB | -10 dB |
| 1700 MHz | 123 dB | -2 dB |
| 2000 MHz | 125 dB | 0 dB |
| 2500 MHz | 126 dB | 1 dB |

As can be seen, for the propagation conditions to the ground TN UE can improve by 10 dB at 700MHz compared with 2GHz.

Table 2 presents a calculation of the interference level at a TN UE due to an NTN HAPS based on the assumptions in 38.863 and assuming worst case; maximum EIRP and minimum FSPL. As can be seen, if the ACLR is 45dB then even in this worst case the interference level from the HAPS BS is well below the UE noise floor.

| Parameter | Value |
| --- | --- |
| HAPS EIRP/MHz (Maximum) | 46.8 dB / MHz |
| FSPL | 115 dB |
| UE antenna gain | 0 dBi |
| HAPS ACLR | 45 dBc |
| Interference level in adjacent channel from HAPS | 46.8 – 115 – 0 – 45 = -113.2 dBm / MHz |

Thus we observe that (i) the propagation loss difference at 700MHz compared to 2GHz is much less than the margin in the ACLR at 2GHz and (ii) the expected ground power level in the adjacent channel at 700MHz is below the UE noise floor.

# 3. Uplink link budget and noise floor considerations

In the uplink, the propagation loss difference will be the same as in table 1. Again, the difference in propagation loss for 700MHz compared to 2GHz is smaller than the margin in the ACS.

A link budget calculation for the RX power from a single UE in the adjacent channel at the HAPS receiver is depicted in table 3. Compared to the BS noise floor, the noise power is around 30 dB lower than the noise floor in e.g. 10MHz. This implies that if, e.g. 100 UEs would all transmit at full power and receive maximum HAPS antenna gain with the minimum propagation loss to the HAPS, nevertheless the HAPS would not be significantly degraded. This is a pessimistic estimate since in reality not all of the UEs would have minimum propagation loss to the HAPS, and not all UEs would transmit with minimum power.

| Parameter | Value |
| --- | --- |
| UE TX power | 23 dBm |
| FSPL | 115 dB |
| UE antenna gain | 0 dBi |
| UE ACLR | 33 dBc |
| HAPS antenna gain | 14 dBi |
| Interference level in adjacent channel from HAPS | 33 – 115 – 0 – 33 – 14 = -129 dBm |

# Conclusion

Based on the observations in the previous sections, we conclude that there is sufficient margin in the ACLR and ACS derived for 2GHz that the same values can be applied for all of the new HAPS bands.

Adopt ACLR 45dB and ACS 46dB for all of the new HAPS bands