3GPP TSG-RAN WG4 Meeting # 112-bis 	    R4-2416332

Hefei, CN, 14  – 18 October 2024

Agenda Item:	6.18.2

Source: 	Ericsson

Title:	On requirements for AI/ML based mobility

Document for:	Approval

# Introduction

The updated SID has the following objectives for RAN2 as primary WG for the SI and RAN4 as secondary WG involved in the SI. The objectives from RP-242393 are copied below for reference, where the objectives for RAN4 WG are highlighted for further discussion.

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

  - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed

  - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.)

  - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability.

  - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

- NOTE 1: RAN1/3 work can be triggered via LS

- NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3

- NOTE 3: Two-sided model is not included

According to the updated SID RAN4 has two objectives, one is to study impact on RAN4 requirements based on RAN2 assumptions and the  other is to evaluate testability and interoperability issues related to AI/ML for mobility. In this contribution impact on RAN4 requirements due to RAN2 framework is analyzed. In our companion paper R4-2416333, testability and interoperability issues are elaborated.

# Discussion

## Impact of core requirement

The existing procedure for NR mobility relies on the measurements reported by the UE based on the measurement reporting configuration (periodic or event triggered) provided by the network. For AI/ML based mobility, the possibility of UE reporting the predicted measurement/event is being discussed in RAN2. To quantify the gains of using AI/ML model for mobility, RAN2 is performing system level simulations to evaluate the accuracy of the predictions done by an AI/ML model deployed at UE to support mobility functionality in UMa and UMi scenarios.

### Predictions for AI/ML based mobility

To identify the gain AI/ML model can bring over existing non-AI/ML based solution to UE mobility, RAN2 is evaluating the accuracy of RRM measurements predicted by UE in temporal domain, spatial domain, and frequency domain.

Temporal prediction: Predict serving and/or neighbour cell(s)/beam(s) becoming worse so that unintended event like radio link failure (RLF) or short-stay handover can be avoided. Temporal prediction of serving and/or neighbour cell(s)/beam(s) can also help to reduce the measurement overhead or delay for mobility purposes.

Serving cell time domain prediction:

- Input: Cell measurements and/or beam measurement on serving cell(s) at time t0.

- Output: Time domain prediction of measurements on serving cell(s) at time t0+T, where T denotes the future reference of time where the predicted measurement should remain valid.

Neighbouring cell time domain prediction:

- Input: Cell measurements and/or beam measurement on neighbouring cell(s) at time t0.

- Output: Time domain prediction of measurements on neighbouring cell(s) at time t0+ T, where T denotes the future reference of time where the predicted measurement should remain valid.

Spatial prediction: Predict a set of neighbour beam(s)/cell(s) by measuring a smaller set of beams/cells. Spatial domain prediction is limited to the intra-frequency cells in RAN2 evaluations.

Spatial domain predictions:

- Input: Available serving/neighbour cell level measurements and/or beam level measurements.

- Output: Prediction of other neighbour cell(s) level measurements.

- Note: spatial domain prediction for serving cell refers to ongoing work for beam management use case. For mobility use case, spatial prediction is limited to prediction of neighbour cell(s) level measurements only.

Frequency prediction: Predict measurements of cells in one frequency based on the measurements performed by the UE on cells in another frequency.

Frequency domain predictions:

- Input: Available serving/neighbour cell level measurements and/or beam measurements in frequency F1.

- Output: Prediction of other neighbour cell level measurements in frequency F2.

- F1 and F2 are frequencies in same frequency region are being considered in RAN2 evaluations.

All prediction types above, in our view, will have an impact on RAN4 requirements and therefore a careful investigation needs to be done.

Observation 1: All prediction types being explored by RAN2, in our view, will have an impact on RAN4 requirements.

Before a detail analysis of impact on RAN4 requirements can be done, a common understanding of the term “prediction” and “predicted measurement/event” needs to be established in RAN4. “prediction” in our view should refer to the process of using a trained AI/ML model to produce output(s). And the term “predicted measurement/event” should refer to the output of the AI/ML model. “prediction” as a process will have an impact on the core requirement for mobility and the accuracy of “predicted measurement/event” will have an impact on the performance requirement for mobility. For mobility purposes, depending on the framework defined by RAN2, UE may be configured to report predicted measurement or event. For measurement prediction the UE can be configured to report temporal/spatial/frequency prediction. For event prediction direct and in-direct event predictions are under discussion in RAN2. From RAN4 point of view, the impact of measurement and event predictions on RAN4 requirements needs to be studied.

To further clarify prediction as a process, an illustration is given in Figure 1 that shows both pre-prediction phase and prediction phase. Pre-prediction phase represents the time taken by UE to perform L1/L3 measurements to generate input for the AI/ML model. Prediction phase highlights the process of providing an input to the AI/ML model and receiving an output from the model. The prediction phase shown in Figure 1 is like AI/ML model inference process defined in TR38.843, where “AI/ML model inference” is defined as a process of using a trained AI/ML model to produce a set of outputs based on a set of inputs. The same definition can be used for spatial and frequency predictions. For temporal prediction it needs to be clarified that “temporal prediction” is a process of using a trained AI/ML model to produce a set of output(s) based on a set of input(s), where the output of the model should remain valid at a future reference time.

Figure 1. Illustration of prediction.

Proposal 1:  RAN4 to study requirements for temporal prediction, spatial prediction, and frequency prediction based on the outcome of RAN2 evaluations.

Proposal 2: RAN4 to study requirements for event prediction based on the outcome of RAN2 evaluations.

Proposal 3: RAN4 to use definition of AI/ML model inference from TR38.843 for spatial and frequency predictions while discussing the impact of spatial and frequency predictions on RAN4 requirements.

Proposal 4: RAN4 to use the following definition of temporal prediction while discussing the impact of temporal prediction on RAN4 requirements. “Temporal prediction is a process of using a trained AI/ML model to produce a set of output(s) based on a set of input(s), where the output of the model should remain valid at a future reference time.”

Proposal 5: Send an LS to RAN2 requesting to capture definition of temporal, spatial, and frequency prediction in TR38.744.

## Generalization of AI/ML based mobility

RAN2#126 (R2-2404711) agreed on a work plan in Table 1 for the AI/ML for mobililty study item.

Table 1. RAN2 work plan agreed in RAN2#126.

| Time slot | Meeting# | RAN2 work plan | RAN4 work plan |
| --- | --- | --- | --- |
| April 2024 | RAN2#125bis | 1, Discussion on work plan and TR skeleton 2, Discuss mobility use cases i.e. RRM measurement, measurement event and RLF/HO Failure prediction and corresponding metrics 3, Initial discussion on evaluation methodology, common metrics and simulation assumption etc. |  |
| May | RAN2#126 | Discuss simulation assumption for RRM measurement prediction and also other use cases Discuss use case specific (RRM and RLF) methodology and simulation assumptions Further clarify RRM sub use case, metrics etc Start discuss RLF/HOF use case including definition, metrics etc Endorse skeleton TR and catpure text proposal based on agreements made so far |  |
| August | RAN2#127 | 1, Continue discuss RRM measurement event use case,RLF/HOF inlcuding definition, metrics, methodology and use case specific simulation assumptions 2, Performance evaluation on simulation result for RRM measurement prediction and RLF. 3, Clarify what could be RAN4 work and Send LS to RAN4 and RAN |  |
| September | RAN#105 | 1, Identify working scope of RAN4 based on LS from RAN2 and company contributions 2, May adjust other working scope, if any |  |
| October | RAN2#127bis/ RAN4#112bis | 1, Further performance evaluation of simulation result of RRM measurement, measurement event and RLF use cases | Detail is up to RAN#105 discussion. |
| November | RAN2#128/ RAN4#113 | 1, Further performance evaluation of simulation result of RRM measurement, measurement event and HOF use cases 2, Start discussing simulation assumptions for generalization study 3, Start discusing AI mobility specific LCM aspects and their spec impact based on latest progress made in WID NR_AIML_Air. The intention is to identify what issues to be discussed in RAN2 | Detail is up to RAN#105 discussion. |
| Feb. 2025 | RAN2#129/ RAN4#114 | 1, Further performance evaluation of simulation result of RRM measurement, measurement event and HOF use cases 2, Continue discussing simulation assumptions for generalization study 3, Discuss identified LCM issues and their spec impact | Detail is up to RAN#105 discussion. |
| April | RAN2#129bis/ RAN4#114bis | 1, Start with generalization work on e.g. RRM measurement prediction 2, Discuss identified LCM issues and their spec impact | Detail is up to RAN#105 discussion. |
| May | RAN2#130/ RAN4#115 | 1, Continue with generalization work 2, Discuss identified LCM issues and their spec impact | Detail is up to RAN#105 discussion. |
| June | RAN#108 | Initial TR for information | Detail is up to RAN#105 discussion. |
| August | RAN2#131/ RAN4#116 | 1, Conclude identified LCM issues and their spec impact 2, Discuss conlcusion and recommendation for potential normative work | Detail is up to RAN#105 discussion. |
| September | RAN#109 | 1,TR for approval 2, Close the SID | Detail is up to RAN#105 discussion. |

Generalization is very important to understand whether the performance gain achieved by deploying AI/ML model to support UE mobility is limited to a specific scenario or not. In this regard, an understanding of generalization capability of AI/ML model deployed to support UE mobility functionality is critical to evaluate the impact of AI/ML based mobility on RAN4 performance requirements. Based on the agreed work plan, as highlighted in Table 1, RAN2 is yet to start discussing simulation assumptions for generalization evaluation. RAN4 can take this as an opportunity and proactively provide input to RAN2 on parameters and assumptions that are relevant to generalization study. The details regarding which parameter and assumption need to be considered can be discussed in the next RAN4 meeting.

Proposal 6: RAN4 to provide input to RAN2 on parameters and assumptions relevant to generalization study.

# Summary

In this contribution impact on RAN4 requirements based on RAN2 assumptions are discussed. The proposals below summarize the discussion presented in this paper.

Proposal 1:  RAN4 to study requirements for temporal prediction, spatial prediction, and frequency prediction based on the outcome of RAN2 evaluations.

Proposal 2: RAN4 to study requirements for event prediction based on the outcome of RAN2 evaluations.

Proposal 3: RAN4 to use definition of AI/ML model inference from TR38.843 for spatial and frequency predictions while discussing the impact of spatial and frequency predictions on RAN4 requirements.

Proposal 4: RAN4 to use the following definition of temporal prediction while discussing the impact of temporal prediction on RAN4 requirements. “Temporal prediction is a process of using a trained AI/ML model to produce a set of output(s) based on a set of input(s), where the output of the model should remain valid at a future reference time.”

Proposal 5: Send an LS to RAN2 requesting to capture definition of temporal, spatial, and frequency prediction in TR38.744.

Proposal 6: RAN4 to provide input to RAN2 on parameters and assumptions relevant to generalization study.

# References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur).