3GPP TSG RAN WG4 Meeting #112bis	R4-2416335

Hefei, China, 14 Oct. 2024 – 18 Oct. 2024

Agenda Item:	6.19.3.2

Source: 	Ericsson

Title:	Discussion on other RRM requirements for MIMO

Document for:	Discussion

# Introduction

In RP-234007, WID for MIMO phase 5 is agreed. In RAN4#112 meeting, RRM requirements impact was analyzed and the following WF [1] is agreed.

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

In this contribution we analyze the potential RAN4 RRM impacts for the FFS parts of the above WF.

# Discussion

CJT enhancements

In previous releases, RAN1 specified CJT under ideal backhaul and ideal synchronization conditions. Since the ideal backhaul and ideal synchronization may not exist in real NW deployments, in Rel-19 RAN WGs were tasked to specify CJT under non-ideal backhaul and non-ideal synchronization. To enable this, RAN1 introduced following aperiodic standalone CJT calibration reporting.

- Frequency offset (FO) reporting (ReportQuantity is cjtc-F)

- Delay offset (DO) reporting (ReportQuantity is cjtc-Dd)

- Joint delay offset and frequency offset (cjt-Dd-F) report

- DL/UL phase offset (PO) reporting (ReportQuantity is cjtc-P)

Through these reports, CJT could be possible under real NW deployment conditions by NW compensating for the synchronization error or other impairments. Having said that, the feasibility of the CJT depends on the newly introduced measurement reports and the accuracy of these reports from the UE. To get clearer picture on the importance of the accuracy of the measurement, we present different CJT applications and their expected accuracy level for different applications.

Different applications, like only coherent combining and nulling, depending on region of operation w.r.t SNR, need different level of accuracy of the phase estimation from the UE. Different cases will also require different total budgets. As examples, we will use a total budget of 50 degrees for only coherent combining and then 30 and 10 degrees for nulling, depending on SNR:

- only coherent combining total budget: 50 degrees

- nulling total budget, low and high SNR: 30 and 10 degrees.

To enable different applications at the NW side, NW needs to know what is the UE estimated accuracy of the measurement so that NW may or may not schedule the UE with different feature based on the accuracy of the measurement.

- Different CJT applications need different levels of accuracy.

- RAN4 to define the requirements for the Frequency offset (FO), Delay offset (DO), Joint delay offset and frequency offset (DO-FO), DL/UL phase offset (PO) reporting.

RAN4 measurement requirements traditionally consist of measurement delay and measurement accuracy requirements. In the subsequent section, we discuss about measurement delay and accuracy.

Measurement delay requirements:

The CJT calibration reports introduced by RAN1 are aperiodic. The calibration measurements are based on the periodic TRS and is aperiodically requested by the NW. When an aperiodic report is requested by the NW, UE needs to report the measurement on the corresponding reporting resources (based on the measurement performed on the CSI reference resource (CSI-RS for tracking)) as defined in the RAN1 specification.  Since UE does not know when the NW will request the report, UE is expected to at least store the CSI resource in the buffer and when a CSI report is requested, UE measures on the CSI resource stored and sends the report during the grant allocated for the report. The number of instances of CSI resources UE stores depends on whether time restriction is enabled or not. This is enabled by parameter called timeRestrictionForChannelMeasurements. When time restriction is OFF, UE is allowed to average over the previous CSI resource instances of the CSI reference resource to average the measurement. When the time restriction is ON, UE shall take the single shot measurement corresponding to CSI reference resource without averaging.

Among the CJT calibration reports introduced in Rel-19, FO, DO and joint DO-FO are based on the TRS, and time restriction cannot be enabled for TRS. That means UE is allowed to use multiple instances of the CSI reference resource to average the measurement to achieve better accuracy of the measurement. Whereas for PO reporting, NW can configure whether time restriction is enabled or not.

Why measurement delay requirements for aperiodic reporting for CJT calibration?

While defining legacy measurement requirements for L1-RSRP or L1-SINR, RAN4 defined periodic and semi-persistent measurement reporting requirements. However, in other measurements such as L1-RSRP or L1-SINR, where RAN4 defined periodic reporting and semi-persistent reporting, measurement behaviour defined in periodic reporting can act as a guidance for aperiodic reporting regarding the measurement behaviour when time restriction is enabled or disabled. For example, in L1-RSRP reporting, when time restriction is not enabled, RAN4 defined number of samples for L1-RSRP averaging. In L1-RSRP, same number of samples (M=3) can be assumed for aperiodic reporting too when NW do not enable time restriction for aperiodic reporting.

In the case of CJT calibration reporting, time restriction can be enabled or not enabled. When time restriction is not enabled, we need to define measurement behaviour on number of samples allowed for averaging. One can also argue that for TDCP measurement, RAN4 did not define measurement delay. We think the TDCP measurement is for adjusting the resource configuration. However, CJT report is used to enable the coherent joint transmission in non-ideal synchronized NWs. Based on the UE report, NW may be offsetting or adjusting its timing offset of the transmission. Unless the report is with good accuracy, CJT report may have negative impact on the NW adjustment. Since single shot measurements may not always serve well, RAN4 should study the number of samples required to ensure a good quality measurement. One drawback of not defining delay requirements is that different UEs may consider different numbers of samples for the measurement computation or averaging.

- RAN4 to define the measurement delay or measurement behavior (number of samples for measurement averaging) for the aperiodic standalone CJT reporting.

- RAN4 to define measurement accuracy requirements.

Other aspects of the CJT calibration reporting:

In RAN1#118 meeting, it was agreed to increase the resolution of frequency offset report to MFO = 256 levels (8-bits) and same is produced below for reference. Though the MFO is increased to 256 levels, we think that it won’t address the issue completely. In this section, we analyze the AFO range of the same report.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO = 256.

In RAN4#112, we presented total budgets and frequency error reporting range [2]. The text is partially repeated here and some arguments regarding range are added. The accuracy requirements for the delay offset reporting depend on the total phase error budget we can tolerate and how we allocate this total budget onto the terms on the budget. The total budget, in turn, depends on the use case for CJT. Examples of total budgets:

- only coherent combining total budget: 50 degrees

- nulling total budget, shallow and deep: 30 and 10 degrees.

If we further develop a budget with at least the following terms:

- UE measurement error.

- UE reporting resolution/quantization error.

- BS algorithmic compensation error.

- Inter TRP phase drift.

- Potential inter TRP mast sway.

- Potential UE mobility

If we divide a total budget of 50-, 30- or 10- degrees phase error evenly across the 6 terms in the budget we get 8.3, 5 or 1.7 degrees per term. We can now calculate the required number of bits for the frequency offset (FO) reporting. The frequency resolution error is ½ of a quantization step:

∆fmax⁡_resolution_error=0.5fcarrierAFOMFO

Where AFO is the maximum carrier frequency error in Hz and MFO is the number of quantization levels. Where AFO is the maximum carrier frequency error in Hz and MFO is the number of quantization levels. AFO depends on the carrier frequency and the accuracy of the oscillator clock.

If we assume fcarrier= 7.125 GHz, for a UE with clock drift of 200ppb, may drift 1425 Hz (7.125 *10^9 * 200*10^-9) per second.  If we assume 10ms reporting periodicity, maximum drift is given by 14.25Hz. with RAN1 agreed 256 levels, the frequency error that can be mapped to one level is

14.25/256 = 0.0556 Hz per level

As we know UE measures the phase and then calculate the frequency error, the phase error that is linked frequency error of 0.0556 Hz is equal to 360*0.0556=20.02 degrees. However, maximum error that can be tolerated to this component is 8.3 degrees as per the above analysis. The number of levels we need to achieve 8.3 degrees per reporting level is provided below

We also know that we can translate to phase error domain. If we multiply with maximum frequency error ∆fmax⁡_resolution_error with 360, then we get the phase rotation per second. A multiplication of the phase rotation per second with the report periodicity Treport will then give us the maximum phase error during the report period, i.e. 360Treport∆fmax⁡_resolution_error. This we can compare with the total allocation of phase error (depending on total budget 50, 30 and 10 degrees and the allocation of total budget to quantization error 8.3, 5 and 1.7 degrees):

360Treport∆fmax⁡_resolution_error<Allocation

Hence:

AFOMFO<Allocation360Treport0.5fcarrier⇒ MFO> AFO360Treport0.5fcarrierAllocation

If we assume fcarrier= 7.125 GHz, that is the highest FR1 band and a Treport= 10 ms or 30 ms we then get:

Table 1: Required number of bits for frequency offset (FO) reporting

In RAN1#118 it was agreed to support MFO = 256, for the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn. This is very good.

However, additionally, a reduced range is another possibility to achieve a high possible resolution, besides number of levels. From table 1 we may deduce that 8 bits suffice to handle even the strictest 10-degrees total budget.

- A reduced range is another possibility to achieve a high possible resolution, besides number of levels.

- With 8 bits of MFO = 256 levels (8-bits) and AFO = 20 ppb we can handle a strict 10 degree total budget- This is needed for use cases requiring a stricter total budget.

- Propose to RAN1 to add AFO = {20 ppb, 50 ppb} to the set of configurable ranges to frequency offset report. Please find an LS draft in appendix.

Furthermore, for another report, the DL/UL phase offset report, RAN1 has agreed 16 or 32 levels:

… Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, the resolution parameters for n, i.e. M, are NW-configured via higher-layer (RRC) signalling from the candidate values {16, 32}, where Φn,σ∈0,2πMΦ-1,4πMΦ-1,…,MΦ-22πMΦ-1,'invalid'. …

We then get a worst phase error when we calculate the quantization uncertainties of:

0.5⋅36016=11.25

0.5⋅36032=5.6

If we compare this with the 8.3, 5 or 1.7 degrees per term, in Table 1 above, we are short of required number of bits already for coherent combining and far away when it comes to the required number of levels for coherent combining and nulling.

- We observe that the required number of levels for DL/UL phase offset report reporting is not compatible with RAN1 agreements of MPO = {32,64}, for coherent combining. We require a higher number of levels (bits). This becomes even more apparent when we consider coherent combining and nulling.

In the analysis in this section, we considered equal error allocation to all the error terms. However, it may not be the case and RAN4 should investigate this.

- The required UE Resolution, i.e., more than the current 16 or 32 levels, MPO, for DL/UL phase offset report, from a RAN4 point of view, has to be feed back to RAN1. Please find an LS draft in appendix.

3TX UL

Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

As per our understanding objective 4 may have some RF impact and Demod impact but we do not foresee any RRM requirements impact.

- specification of 3-TX UL may not have RRM requirements impact.

Asymmetric DL sTRP/UL mTRP

Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

Based on the above RAN1 agreements, we can summarize the enhancements introduced or discussed in RAN1 for supporting UL only TRP as following.

- Two closed-loop PC adjustment states for SRS, both separate from PUSCH

- PL offset configuration and update

  - PRACH enhancements using one of the PL offset of the TCI states

  - TCI state enhancements using different PL offset

- Two TA enhancements

Two closed-loop PC adjustment states for SRS, both separate from PUSCH:

Typically, power control and power control offset are not discussed in RRM. Hence, RRM impact is not foreseen for the two CLPC for SRS.

- Two closed-loop PC adjustment states for SRS, both separate from PUSCH may not have RRM requirements impact.

PL offset configuration and update:

As per the RAN1 discussion we can see that PL offset is introduced per TCI state. The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission. However, the detailed design is FFS.

TCI state enhancements using different PL offset:

The offset within the TCI state is applicable for SRS, PUSCH/PUCCH. It is configured in the RRC message and can also be updated using the MAC CE at later stage. Corresponding RAN1 agreement can be found below.

For the association between PL offset and joint/UL TCI state, support the following

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

PL-RS configuration requirements:

When RRC configures the PL-RS and pathloss offset for the TCI state associated with UL only TRP, the PL-RS may be same as the PL-RS configured for the reference TRP or may be different. When a PL-RS is configured, the time required for PL estimation from PL-RS and the configured pathloss offset can depend on whether the PL-RS is maintained or not. RAN4 to study PL estimation requirements from the PL-RS and pathloss offset.

- RAN4 to study PL estimation requirements from PL-RS and pathloss offset.

Issue 2-5-4: RRM core impacts of Active uplink TCI state switching delay for unified TCI?

<Way forward>

- Option 1: The legacy requirements can be reused.

- Option 2: The legacy requirements cannot be reused.

- Option 3: RAN4 to discussion how to update R17/R18 requirement for asymmetric DL sTRP/UL mTRP scenarios

UL TCI state switch involves following steps:

- Pathloss estimation

- Timing acquisition

- QCL acquisition

Pathloss estimation: As per our understanding, RAN1 is discussing how to configure the PL-RS for the UL only TRP. One of the proposals under discussion is anchor TRP DL-RS can be used for pathloss computation along with PL-offset indicated in the TCI state configuration.

Timing acquisition: As per our understanding, RAN1 is discussing the Dl reference timing aspect. One of the proposals was to use anchor TRP DL reference timing with some DL offset.

QCL acquisition: We think SRS can be configured as a QCL source for deriving spatial info relation. With SRS being configured as QCL RS, next question that arises is how the UE acquires the spatial info for the SRS. As per RAN1 specification, we think the spatial relation info for the SRS can be obtained using the SRS resource set configured with beamManagement. When NW configures SRS resource set with usage set to beamManagement, UE transmits the SRS resources in the SRS resource set in different spatial direction. In the UL TCI state, NW can indicate SRS resource ID as the QCL source and UE uses the spatial relation acquired from SRS beam management for the UL TCI state spatial relation information.

We understand that SRS based beam management requirements are not defined in RAN4 at present. Hence, we suggest RAN4 to study and specify SRS based beam management requirements.

- RAN4 to study and specify SRS based beam management requirements.

- RAN4 to specify UL TCI state switching requirements for UL only TRP.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to define the requirements for the Frequency offset (FO), Delay offset (DO), Joint delay offset, and frequency offset (DO-FO), DL/UL phase offset (PO) reporting.

- RAN4 to define the measurement delay or measurement behavior (number of samples for measurement averaging) for the aperiodic standalone CJT reporting.

- RAN4 to define measurement accuracy requirements for CJT calibration reporting.

- Propose to RAN1 to add AFO = {20 ppb, 50 ppb} to the set of configurable ranges to frequency offset report. Please find an LS draft in appendix.

- The required UE Resolution, i.e., more than the current 16 or 32 levels, MPO, for DL/UL phase offset report, from a RAN4 point of view, has to be feed back to RAN1. Please find an LS draft in appendix.

- RAN4 to study PL estimation requirements from PL-RS and pathloss offset.

- RAN4 to study and specify SRS based beam management requirements.

- RAN4 to specify UL TCI state switching requirements for UL only TRP.

# References

[1] 	RP-234007, New WID: NR MIMO Phase 5
[2] 	R4-2414035, WF on RRM requirements for NR_MIMO_Ph5

# Annex

Aperiodic CSI reporting steps:

- Step 1: Configure CSI-Aperiodic Trigger states (maximum 128 trigger states)

  - Each trigger state can contain 16 resource sets

    - Each resource set contain 64 NZP CSI-RS resources per set

- Step 2: MAC CE activates up to 64 aperiodic trigger states to indicate using DCI

- Step 3: DCI indicating the MAC CE activated TCI states

=== Annex: LS draft ===

3GPP TSG-RAN WG4 Meeting #112bis	R4-XXXXX

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	LS on frequency offset report range AFO and phase offset number of levels MPO

Response to:	-

Release:	Release 19

Work Item:	NR_MIMO_Ph5-Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact person:	Magnus Larsson

Magnus dot k dot Larsson at Ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Overall Description:

Based on the following RAN1 agreements from RAN1#118, RAN4 further discussed the total budget for the AFO range and the MPO number of levels.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO = 256.

Agreement

For the Rel-19 aperiodic standalone CJT calibration reporting, the resolution parameters for n, i.e. M, are NW-configured via higher-layer (RRC) signalling from the candidate values {16, 32}, where Φn,σ∈0,2πMΦ-1,4πMΦ-1,…,MΦ-22πMΦ-1,'invalid'.

Following agreement is reached in RAN4 for the AFO range and the MPO number of levels.

- The current resolution of 16 or 32 levels for DL/UL phase offset report MPO has to be increased to 32 or 64.

- Add AFO = {20 ppb, 50 ppb} to the set of configurable ranges to frequency offset report.

# 2. Actions:

# To RAN1

RAN4 kindly requests RAN1 to consider the feedback in subsequent analysis and work.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN4 Meeting#113	 18th – 22nd November 2024	Orlando, US

TSG-RAN4 Meeting#114	 17th – 21st February 2025	Athens, Greece