3GPP TSG-RAN WG4 Meeting #112bis	R4-2416339

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Specification improvements in R19 timeframe

Agenda item:	8.1.2.2

Document for:	Discussion

Introduction

In the last couple of meetings, RAN4 has been discussing possible specification improvements. The outcome of the RRM part of the discussions is captured in the agreed WF’s [1, 2 and 3].

In RAN4#111 a number of concrete actions were agreed to implemented. These were further discussed in RAN4#112 meeting where following was agreed and captured in [3]:

- Interested companies are encouraged to collaborate on CR review and drafting before RAN4#112bis

  - CR will be based on TS38.133 v18.6.0

  - The updated CR will be submitted in RAN4#112bis

  - The proposed revision should be strictly limited to the following three issues

    - Issue1: [], “TBD” and “FFS” clean up

    - Issue2: Terminology/style inconsistency, incorrect notation/symbols/abbreviation, undefined abbreviations, redundant information/notes clean up

    - Issue3: Empty test cases clean up

A CR drafting and review plan was made. Interested companies has spent a great amount of time and effort on reviewing the CRs in the time between RAN#112 and RAN4#112bis.

Besides above agreement also following agreements were reached in RAN4#112 [3]:

- Modal verbs

  - Agreement: deprioritized

- Duplicated requirements

  - Agreement: deprioritized

- Unused test configurations

  - Agreement: deprioritized

- Suffix alignment

  - Agreement: deprioritized

- References or mapping tables in the core part requirements that point to the relevant test cases from Rel-19

  - Agreement: Deprioritized. Meanwhile, interested companies can bring examples on how to implement.

- Abbreviations

  - “PRB” or “RB”:

    - Current status: both are used, choose one of them? None of them is in Abbreviation of TS 38.133.

    - Agreement: “PRB” should be consistently used in all sections instead of “RB” and the abbreviation is to be included in section 3.

  - “BW”:

    - Current status: used, but not in the Abbreviation section in TS 383.133.

    - Agreement: Include the abbreviation in the Abbreviations section 3 of TS 38.133.

  - “TRS”: is used but not defined in Abbreviations section in TS 38.133.

    - Agreement: Include the abbreviation in the Abbreviations section 3 of TS 38.133.

Besides the specification clean-up exercise RAN4 also agreed looking at other quality issues in 38.133 and agreed to look at if it is possible to improve the current challenges we have seen with the use of indentation:

- Agreements:

  - Section 8.3.2 in TS38.133 v18.6.0 will be used as the example clause.

  - Other example section(s) are not precluded

  - Approach

    - Option 1: RAN2 pseudo-code

    - Other options, which can be reflected in the example provide by each interested company.

In Appendix A, we give our view on how RAN4 could potentially improve the use of hierarchy of indent by use of ‘pseudo-code’-style.

In this paper we discuss a few aspects identified during the process of drafting the example in the Appendix, which is a continuation of our earlier proposal in [4]. We have left the comments in the TP to make it easier to locate each identified issue.

Discussion

Issues identified during re-structuring of section 8.3.2

Initially, the principle we have taken in drafting the TP exemplifying how we see that 38.133 structure could look like, to improve the 38.133 specification quality for example in terms of:

- Readability

- Accuracy

- Consistency

- Complexity

is keeping most of the base structure, while applying use of numbered list. While doing this we obviously encountered some challenges. Most of these, if not all, should be rather straight forward to address except maybe one (review process). Following we go through the issues we would like to point out where RAN4 discussion would be needed.

‘Hanging’ UE requirement

During the drafting the CR we noticed one example where it was very unclear exactly when ‘no activation requirements will be applied’ applies:

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell

-	its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

This part is currently ‘hanging’ a bit in the specification. Especially as the condition are the same as another place – where it states that the activation requirement apply when the conditions are fulfilled (this was also pointed out in [5])

It is unclear how/when (conditions when) above applies. However, according to the logic of our proposed indentation, it would potentially be applicable if following clarifications are made:

- However, when the following conditions are not fulfilled, no activation requirement will be applied for this unknown Scell:

However, even if this addition of ‘not’ is added, it seems that still one more condition is missing related to:

- the side condition Ês/Iot < -2dB

Anyway, this clearly needs discussion even for the current NR specification. As this is more a legacy clarification, we do not need to solve in this discussion but can handle the discussion during maintenance in coming meeting.

Use of different terms

We also identified that in section 8.3.2 (at least) the SCell to be activated is referred to either by:

- ‘SCell to be activated’ or

- ‘target SCell’ or

- ‘SCell’.

Preference is using the same phrase. Hence either ‘target SCell’ or ‘SCell to be activated’ or ‘SCell’ (or something else).

Align the phrase used for referring to the ‘SCell’ being addressed in the requirements in section 8.3.2.

Use of ‘and/or’

During the drafting we also noticed the use of the ‘and/or’ phrase a couple of places. When trying to write the requirements clear the use of ‘and/or’ becomes even more problematic and will cause unclear requirements. Hence, RAN4 should not use ‘and/or’ – as discussed and commented earlier. The example from 8.3.2 is:

3> if the target SCell is unknown to UE:

4> if the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA, or

4> if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management:

5> if semi-persistent CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> if UE supports reduceForCellDetection, and/or

6> if UE supports reduceForSSB-L1-RSRP-Meas, and/or

6> if UE support shortMeasInterval-r18, and

Exactly what RAN4 agreed and what agreement is used for drafting the ‘and/or’ requirements should of course be used for clarifying how to clean this ‘and/or’. In this particular example our understanding is that using ‘or’ should be sufficient. Whether UE Support one of the three capabilities or only one still makes the condition true.

However, this of course needs more discussion.

Clarify and remove use of ‘and/or’ phrases.

Move of requirements not conditioned by ‘known/unknown’ SCell

The current structure of the requirements in section 8.3.2 is not fully logical. At least the way we read the current requirements, some of the requirements are not conditioned on whether the SCell is known or unknown. Anyway, such requirements are listed below requirements for known/unknown SCell requirements.

Logically, requirements not conditioned on whether the SCell is known or unknown should be defined before the requirements conditioned by the SCell known and unknown status.

One example is the activation time for an SCell which belongs to FR2 if there is at least one active serving cell on that FR2 band.

However, in our CR we have not moved such requirements, but we suggest doing so.

Requirements not conditioned on the SCell known or unknown status, should be defined prior to the requirements conditioned by the SCell known and unknown status.

Inconsistent known/unknown definition and applicable requirements

When reading carefully the known SCell conditions for an FR2 SCell, it sems that such SCell can only be known if the SCell is the first SCell to be activated in an FR2 bands.

And based on this the requirements for activation time under known or unknown FR2 SCell applies. However, for FR2 SCell (not being the first SCell in that band) they will be unknown. However, in this case the SCell activation time requirements for such SCell are not defined under activation time requirements for an unknown FR2 SCell. Instead, these are (with the current indentation) listed under requirements for unknown FR1 SCell and under condition ‘If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2’. This is in itself not very logical.

Hence, based on this our proposal 3 would also better align the known/unknown SCell definition and how the SCell activation time delay requirements are listed.

Specification template support

One aspect to address as well, is the need for a specification template which efficiently support the selected specification outline chosen. Hence, to ensure that whatever structure is selected by the group also leads to easy maintenance keeping the specification quality, there would need to be either one or more of:

- Clear drafting guidelines

- Support from the specification template (e.g. list handling)

Otherwise, we may end up facing the same quality problems in the future.

If a new outline is selected, these need to be supported by clear drafting guidelines and, if needed, template support.

Review process

Having provided a CR with one example how section 8.3.2 could be improved by applying new drafting rules for the RAN4 requirements, it became clear during the process, that reviewing such CR will be quite a complex procedure.

How to perform a proper review of such CR which aims at improving the specification quality needs discussion. Obviously, delegates do not have the time to review multiple of such CRs for each meeting. It is even doubtful if it is possible to perform review and agree during one meeting – especially if the section is complex (like section 8.3.2).

If RAN4 agrees to apply quality improvements and related CRs, a proper review process needs to be agreed.

A quality improvement CR should of course not change the technical content of the current requirements.

Second aspect is how to handle Cat-F and Cat-A CRs in future when baseline section is not the same. It may however be possible to use existing approach based on the act that the technical content of the Cat-F and Cat-A CRs shall be the same while how to capture the change in the specification would depend on whether change is made to ‘legacy’ specification text or ‘improved’ specification text.

However, one may assume that reviewing the CR’s will be more time consuming.

Future CR review time may increase if RAN4 agree to initiate specification quality improvement rewriting in NR timeline.

Conclusion

While drafting the example CR for how to improve section 8.3.2 by applying a pseudo-code style (or RAN2 style) approach, we identified several challenges and open issues which have discussed in this paper. Based on the discussion we propose:

- Align the phrase used for referring to the ‘SCell’ being addressed in the requirements in section 8.3.2.

Clarify and remove use of ‘and/or’ phrases.

Requirements not conditioned on the SCell known or unknown status, should be defined prior to the requirements conditioned by the SCell known and unknown status.

If a new outline is selected, these need to be supported by clear drafting guidelines and, if needed, template support.

If RAN4 agrees to apply quality improvements and related CRs, a proper review process needs to be agreed.

And we observe:

- Future CR review time may increase if RAN4 agree to initiate specification quality improvement rewriting in NR timeline.

- References

- R4-2406710, WF on [110bis][142] RRM_Spec_Improvement, Apple.

- R4-2410715, WF on [111][140] RRM_Spec_Improvement, Apple

- R4-2414103, WF on [112[229] RRM_Spec_Improvement, Apple

- R4-2404701, Discussion on RRM specification quality improvement, Nokia, Nokia Shanghai Bell

- R4-2411785, On improvements of Rel-19 RRM specifications, Qualcomm

Appendix

Example use of pseudo-code

Following is one TP example how to apply the pseudo-code in section 8.3.2 which is a school example of complex RAN4 specification writing.

<Start of Change #1>

### 8.3.2	SCell Activation Delay Requirement for Deactivated SCell

The requirements in this clause shall apply for the UE configured with at least one downlink SCell in EN-DC, or in standalone NR carrier aggregation or in NE-DC or in NR-DC and when one SCell is being activated.

The delay within which the UE shall be able to activate the deactivated SCell depends upon the specified conditions.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot:

Tactivation_delay = n+THARQ+Tactivation_time+TCSI_ReportingNR slot length

where:

- THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]

- Tactivation_time is the SCell activation delay in millisecond.

- TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2].

1> If the SCell is known and belongs to FR1:

2> if the measurement period of the SCell being activated is equal to or smaller than 2400ms:

3> Tactivation_time = TFirstSSB+ 5ms.

2> if the measurement period of the SCell being activated is larger than 2400ms:

3> Tactivation_time = TFirstSSB_MAX + Trs + 5ms.

1> If the SCell is unknown and belongs to FR1:

2> if ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

2> if ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

3> if the side condition Ês/Iot ≥ -2dB is fulfilled:

4> if UE supports shortMeasInterval-r18 and:

5> if the SCell is contiguous to an active serving cell in the same band, and

5> if the ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

5> if the SMTC offset of the sCell is same as the one of contiguous FR1 active serving cell, and

5> if the RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB.

6> Tactivation_time = TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms.

5> otherwise,

6> Tactivation_time = TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms.

4> Otherwise:

5> if the SCell is contiguous to an active serving cell in the same band, and

5> if the ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

5> if the SMTC offset is same as the one of contiguous FR1 active serving cell, and

5> if the RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB

6> Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms.

5>  otherwise:

6> Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms.

2> otherwise:

3> if the side condition Ês/Iot ≥ -2dB is fulfilled:

4> If UE supports shortMeasInterval-r18:

5> if semi-persistent CSI-RS is used for CSI reporting:

6> Tactivation_time = 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting:

6> Tactivation_time = 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay).

4> Otherwise:

5> if semi-persistent CSI-RS is used for CSI reporting:

6> Tactivation_time = 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting:

6> Tactivation_time = 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay).

-	However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell:

-	the SCell is contiguous to an active serving cell in the same band, and

-	A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell

-	its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB;

1> If the SCell being activated belongs to FR1:

2> if the UE is supporting scellWithoutSSB-InterBandCA-r18:

The reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured.

Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”.

2> if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL); and

2> if the UE is not provided with an SMTC configuration for the target SCell, and

2> if there is one collocated active reference serving cell on different FR1 band, and:

3> if the RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and

3> if the [EPRE] difference at the UE is smaller than or equal to [12] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and

3> if the RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell.

4> if aperiodic CSI-RS resources are not configured for SCell activation, or

4> if the UE do not support aperiodicCSI-RS-FastScellActivation-r17:

5> Tactivation_time = Tfirst_TRS + TTRS +5 ms.

4> if aperiodic CSI-RS resources are configured for Scell activation, and

4> if the UE supporting aperiodicCSI-RS-FastScellActivation-r17

5> Tactivation_time = Tfirst_ATRS + Tgap + TATRS + 5 ms.

1> else, if the SCell being activated belongs to FR1:

2> if the UE supporting scellWithoutSSB, and:

2> if there is at least one active serving cell contiguous to the SCell on that FR1 band, and

2> if the UE is not provided with SSB configuration (absoluteFrequencySSB), and

2> if the UE is not provided with SMTC configuration for the target SCell, and:

3> if the RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and

3> if he difference of the reception power with the contiguous active serving cell is <= 6dB, and

3> if the The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band:

4> Tactivation_time = 3 ms.

1> If the SCell belongs to FR2:

2> if there is at least one active serving cell on that FR2 band:

3> if the UE is provided with SMTC for the target SCell, and

3> if the SSBs in the serving cell(s) and

3> if the SSBs in the SCell fulfil the condition defined in clause 3.6.3, and

3> if the parameter ssb-PositionsInBurst is same for the serving cell(s) and the Scell, and

3> if the SSB is in the same half-frame on the SCell and the contiguous FR2 active serving cell.

4> Tactivation_time is TFirstSSB+ 5ms.

3> if the UE supporting scellWithoutSSB:

4> if UE is not provided with any SMTC for the target SCell:

5> if the RS(s) of SCell being activated is (are) QCL-TypeD with RS(s) of one active serving cell on that FR2 band:

6> Tactivation_time = 3 ms.

4> if the UE is a FR2 power class 6:

5> if the UE supports [Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode], and

5> if the RS(s) of SCell being activated is (are) QCL-TypeD with RS(s) of one active serving cell on that FR2 band:

6> Tactivation_time = 3 ms.

2> if there is no active serving cell on that FR2 band:

3> if the target SCell is known to UE:

4> if semi-persistent CSI-RS is used for CSI reporting:

5> if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time:

6> Tactivation_time = 3ms + TFineTiming + 2ms.

5> otherwise:

6> Tactivation_time = 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP)

4> if periodic CSI-RS is used for CSI reporting:

5> if UE receives the SCell activation command and TCI state activation commands at the same time:

6> Tactivation_time = max(5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ).

5> otherwise:

6> Tactivation_time = max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ).

3> if the target SCell is unknown to UE:

4> if the PCell/PSCell and the target SCell are configured as FR1-FR2-1 CA, or

4> if the PCell/PSCell and the target SCell are in a FR2-1 band pair with independent beam management:

5> if semi-persistent CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> if UE supports reduceForCellDetection, and/or

6> if UE supports reduceForSSB-L1-RSRP-Meas, and/or

6> if UE support shortMeasInterval-r18, and

7> if SCell activation triggered L3 report is not configured, or

7> if SCell activation triggered L3 report is configured but not reported:

8> Tactivation_time = 6ms + TFirstSSB_MAX,enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced  + TL1-RSRP, enhanced_measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

6> otherwise:

7> Tactivation_time = 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> if UE supports reduceForCellDetection, and/or

6> if UE supports reduceForSSB-L1-RSRP-Meas, and/or

6> if UE support shortMeasInterval-r18, and

7> if SCell activation triggered L3 report is not configured, or

7> if SCell activation triggered L3 report is configured but not reported:

8> Tactivation_time = 3ms + TFirstSSB_MAX, enhanced + 15*TSMTC_MAX, enhanced + X1*Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

6> otherwise:

7> Tactivation_time = 3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

4> else, if the PCell/PSCell and the target SCell are configured as FR1-FR2-2 CA, or

4> if the PCell/PSCell and the target SCell are in a FR2-2 band pair with independent beam management:

5> if semi-persistent CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> Tactivation_time = 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

5> if periodic CSI-RS is used for CSI reporting, and

5> if the side condition Ês/Iot ≥ -2dB:

6> 3ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + max {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Where:

- TSMTC_MAX:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18 or UE is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided or in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated. TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA; in case of FR2 inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

- TSMTC_MAX, enhanced:

-	In FR1 or FR2-1, a UE supporting shortMeasInterval-r18 if the SMTC for SCell being activated is only configured in measObjectNR, TSMTC_MAX, enhanced is the SSB periodicity of SCell being activated. Otherwise, TSMTC_MAX, enhanced = TSMTC_MAX.

- Trs:

  - is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms.

- Trs, enhanced:

  - is the SSB periodicity of the SCell being activated for a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR. Otherwise, Trs, enhanced = Trs

- TFirstSSB:

  - is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

- TFirstSSB_MAX:

  - Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

- TFirstSSB_MAX, enhanced:

- For a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR, TFirstSSB_MAX, enhanced is the time to the end of the first complete SSB burst indicated by the SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length. Otherwise, TFirstSSB_MAX, enhanced = TFirstSSB_MAX

- TFineTiming

  - is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

- X1:

  - is equal to the reported value in reduceForCellDetection in FR2-1. Otherwise, if reduceForCellDetection is absent, X1 is 8.

- TL1-RSRP, measure:

  - is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

- TL1-RSRP, enhanced_measure is:

-	SSB based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0; N is equal to the value reported by the UE in  reduceForSSB-L1-RSRP-Meas. Otherwise, if  reduceForSSB-L1-RSRP-Meas is absent, N= 8. Or,

-	CSI-RS based L1-RSRP measurement delay TL1-RSRP_Measurement_Period_CSI-RS ms based on applicability as defined in clause 9.5 assuming M=1 and TReport=0.

-	If UE supports shortMeasInterval-r18 capability, L1-RSRP measurement for TL1-RSRP, enhanced_measure can be performed based on non-DRX mode even if DRX is configured.

- TL1-RSRP, report:

  - is delay of acquiring CSI reporting resources.

- Tuncertainty_MAC:

- is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- Tuncertainty_RRC:

- is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- Tuncertainty_SP:

- is the time period between reception of the activation command for semi-persistent CSI-RS resource set for CQI reporting relative to:

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

- TRRC_delay:

- is the RRC procedure delay as specified in TS38.331 [2].

  - Longer delays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

  - When absoluteFrequencySSB is not configured in DownlinkConfigCommon for target SCell but SMTC for target SCell is configured, no requirement would be applied.

- TFirstTRS:

- is the time to the end of the first complete periodic CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length.

- TTRS:

- is the periodicity of periodic CSI-RS burst for SCell activation.

- TFirstATRS:

- is the time to the end of the first complete CSI-RS burst for SCell activation after slot n + THARQ+3msNR slot length, where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

- TATRS:

- is the CSI-RS burst for SCell activation where the CSI-RS burst is defined as four CSI-RS resources in two consecutive slots.

- Tgap:

- is a gap length between two aperiodic CSI-RS bursts,

-	at least 2 slots for 15kHz and 30kHz

-	at least 3 slots for 60kHz

SCell in FR1 is known if it has been meeting the following conditions:

1> if during the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

2> the UE has sent a valid measurement report for the SCell being activated and

2> the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3.

1> if the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

SCell in FR2 is known if it has been meeting the following conditions:

1> If the SCell is the first SCell to be activated in an FR2 bands, and following conditions:

2> if, during the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

3> if the UE has sent a valid L3-RSRP measurement report with SSB index, and

3> the SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

2> if during the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown.

The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [2] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The starting point of an interruption window on spCell or any activated SCell, as specified in clause 8.2, shall not occur before slot n+1+THARQNR slot length  and not occur after slot slot n+1+THARQ+3ms+TXNR slot length, where NR slot length is with respect to the numerology used in the SCell being activated, and TX is:

-	0, if Tactivation_time is 3ms;

-	TFirstSSB, for any scenario where Tactivation_time  includes TFirstSSB;

-	TFirstSSB_MAX, for any scenario where Tactivation_time  includes TFirstSSB_MAX;

-	Tuncertainty_MAC +TFineTiming, for any scenario where Tactivation_time  includes only TFineTiming and no TFirstSSB_MAX;

-	Tfirst_TRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_TRS;

-	Tfirst_ATRS, for FR1 inter-band SSB-less SCell activation scenario where Tactivation_time   includes Tfirst_ATRS.

The length of the interruption window may be different for different victim cells, and depends on the applicable scenario and on the frequency band relation between the aggressor cell and the victim cell.

The requirements in this clause and requriements on interruption due to SCell activation in clause 8.2 apply provided that the SSB of the to-be-activated SCell is within the first active DL BWP of the Scell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

<End of Changes>