# 3GPP TSG-RAN WG4 Meeting # 112bis                                 R4-2416343

# Hefei, China, 14 – 18 October, 2024

# Title: 	Discussion on spatial channel model for demodulation performance requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.14.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of study on spatial channel model for demodulation performance was approved in [1], wherein the study item includes the following objectives :

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we give some initial discussion on the practical spatial channel modelling methodology and other analysis based on the last meeting conclusion [2].

# 2 Discussion

During the last meeting, RAN4 discussed how to construct the SCM channel model and some simulation cases. Regarding the construction of the SCM channel model, there are essentially three approaches, CDL channel model based on the TR 38.827, extending current TDL channel model and TX-RX beam steering approach, and the simulation scenarios predominantly revolve whether MU-MIMO test case needs to be considered and the selection of current TDL channel model.

Currently, in RAN4 demod testing, SU-MIMO and MU-MIMO are the primary scenarios being tested. RAN4 has researched numerous test cases related to these two systems, such as interference cancellation and SU-MIMO with multiple antennas. Meanwhile, based on feedback from operators, MU-MIMO is a very typical scenario in practical network scheduling. Concerning the SCM channel model currently studied by RAN4, our motivation is to verify that spatial channel model posses spatial gains over TDL channel model, which different layers show various performance among one code word or two code words. From a practical deployment perspective, the scenario for MU-MIMO is vert necessary. Meanwhile, in the WID, it is explicitly stated that RAN4 also needs to study the demodulation requirements for MU-MIMO.

During the last discussion period, we heard some comments indicating that the MU-MIMO scenario is ralatively complex due to factors requiring consideration such as precoding and the processing of multiple co-schedule UEs. On the other hand, the current RAN4 definition for MU-MIMO test cases only include 2 and 4 receiver antennas, as well as the number of co-schedule UE being 1. For spatial channel model, we need to consider multi-layer and multiple code word scheduling scenarios. Therefore, we think that further discussion is needed regarding the introduction of MU-MIMO test cases.

Observation 1. MU-MIMO scenario is a very typical scenario in practical network scheduling.

Proposal 1. MU-MIMO scenario could be considered from practical deployment perspective.

In the previous meeting, several companies proposed modeling approaches that differ from the CDL channel model, such as TX-RX beam steering and extension TDL channel model. As we all know, the biggest difference between TDL and CDL lies in that the spatial characteristics of the CDL channel model can be represented through AOA and AOD. Our motivation precisely revolves around exploring the gains imparted by spatial properties during signal transmission. It is evident that, compared to the complexity modeling approach of CDL, incorporating spatial angles into the TDL channel model presents a certain level of feasibility in theoretically. However, we think that this approach still needs to verify the feasibility in implementation aspect for TE vendor.

Proposal 2. RAN4 needs to further study the testability of TX-RX beam steering and extension TDL channel model.

Another issue is regarding the mapping between CSI-RS antenna ports and physical antenna arrays. The gNB beam pattern including the assumptions for gNB antenna for definitions in TR 38.827 as shown in following table.

Table 7.2-7: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |  |
| --- | --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz | FR2 |
| Antenna panels in vertical dimension | Mg | 1 | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 | 16 |
| Number of polarizations per panel | P | 2 | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 | 0.5 |

Since we chose (Mg,Ng,Me,Ne,P) = (1,1,8,8,2) as BS antenna element radiation pattern for 8 port transmission in last meeting. For 8 port CSI-RS transmission, there are currently two specified CSI antenna port arrays defined by the specification as shown in following, (N1,N2) = (2,2) or (N1,N2) = (4,1). The main difference caused by this different structure is the shape of the beam in 3D space.

In following Fig 1, which is the process of mapping each of CSI-RS antenna port to antenna elements in the array to physical antenna array. From (a) to (b) is the process of mapping each of CSI-RS antenna port to 1D or 2D array of cross-polarized virtual antenna ports. (b) to (c) is the process of mapping each of antenna elements in the array to a physical antenna array. The array size of (c) should be equal to or larger than the size of (b). In case of beam forming, the size of (c) would be much greater than the size of (b). In this example, we use 4×1 array for 8 CSI-RS port, one polarized antenna in (b) is mapped to sixteen polarized antenna in (c). However, this mapping is just an example and the mapping depending on the implementation of vendor. Typically, we need to employ analog beam forming to realize this mapping approach. Consequently, from RAN4 demod perspective, different analog beam forming can result in substantial variations in beam direction, thereby leading to significance differences in the final results. From our understanding, RAN4 should consider further specifying this implementation of potentially reducing the number of antenna arrays to ensure one-to-one correspondence between CSI-RS ports and physical antenna array.

+-

Fig 1. CSI-RS mapping to antenna array

Observation 2. The process of mapping each of CSI-RS antenna port to antenna elements in the array to physical antenna array based on the implementation if the size of physical antenna array larger than CSI-RS antenna port.

Proposal 3. The process of mapping between CSI-RS antenna port and physical antenna array needs more further discussion.

# 3 Conclusion

In this contribution, we give some discussions on spatial channel model for demodulation performance requirements , The conclusions are:

Observation 1. MU-MIMO scenario is a very typical scenario in practical network scheduling.

Observation 2. The process of mapping each of CSI-RS antenna port to antenna elements in the array to physical antenna array based on the implementation if the size of physical antenna array larger than CSI-RS antenna port.

Proposal 1. MU-MIMO scenario could be considered from practical deployment perspective.

Proposal 2. RAN4 needs to further study the testability of TX-RX beam steering and extension TDL channel model.

Proposal 3. The process of mapping between CSI-RS antenna port and physical antenna array needs more further discussion.

# 4 References

- RP-241610, New WID: Study on spatial channel model for demodulation performance requirements, Nokia.

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM. Nokia.