# 3GPP TSG-RAN WG4 Meeting # 112bis                                 R4-2416344

# Hefei, China, 14 – 18 October, 2024

# Title: 	Discussion on spatial channel model for demodulation performance requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.14.2

# Document for:	Discussion

# 1 Introduction

In RAN #104 meeting, the new WID of study on spatial channel model for demodulation performance was approved in [1], wherein the study item includes the following objectives :

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we give some initial simulation results on the practical spatial channel modelling methodology.

# 2 Discussion

During the last meeting, RAN4 discussed how to construct the SCM channel model, and agreed some simulation parameters to do link level simulation. Regarding the methodology of SCM model, RAN4 prioritized 38.827 UMa-CDL-C for initial comparison and alignment. Based on this agreement [2], we give some initial simulation results for TDL and CDL channel model.

Regarding simulation assumptions, we reused R-18 8Rx assumptions as baseline expect channel model parameters of CDL-C. And for CDL-C channel model, we reused TR38.827 as baseline.

In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in following Tables.

Table 2-1. SNR corresponding to 70% maximum throughput under Rank 2

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel moel | Baseline (SNR@70% maxTP) | Simplified (SNR@70% maxTP) |
| --- | --- | --- | --- | --- | --- |
| TDD 40 MHz/30 kHz (106) | 2T8R | Rank 2, MCS 19 (Table 1) | TDLA 30-10 ULA Low | 7.3 | 8.5 |
|  |  |  | TDLC 300-100 ULA Low | 8.6 | 9.2 |
|  |  |  | CDL-C UMa (38.827) | 11.1 | 15.0 |

Table 2-2. Throughput curves for Rank 2

Table 2-3. SNR corresponding to 70% maximum throughput under Rank 4

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel moel | Baseline (SNR@70% maxTP) | Simplified (SNR@70% maxTP) |
| --- | --- | --- | --- | --- | --- |
| TDD 40 MHz/30 kHz (106) | 4T8R | Rank 4, MCS 17 (Table 1) | TDLA 30-10 ULA Low | 10.1 | 14.4 |
|  |  |  | TDLC 300-100 ULA Low | 11.5 | 16.7 |
|  |  |  | CDL-C UMa (38.827) | 18.4 | 25.6 |

Table 2-4. Throughput curves for Rank 4

Table 2-5. SNR corresponding to 70% maximum throughput under Rank 8

| Duplex CBW/SCS (RB allocation) | Antenna configuration | Rank, MCS | Channel moel | Baseline (SNR@70% maxTP) |
| --- | --- | --- | --- | --- |
| TDD 40 MHz/30 kHz (106) | 8T8R | Rank 8, MCS 17 (Table 1) | TDLA 30-10 ULA Low | 14.5 |
|  |  |  | TDLC 300-100 ULA Low | 18.1 |

Table 2-6. Throughput curves for Rank 8

# 3 Conclusion

In this contribution, we give some initial simulation results on spatial channel model for demodulation performance requirements.

# 4 References

- RP-241610, New WID: Study on spatial channel model for demodulation performance requirements, Nokia.

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM. Nokia.