# 3GPP TSG-RAN WG4 Meeting #112bis                               R4-2416347

# Hefei, Anhui, China, 14th – 18th October, 2024

# Title: 	Discussion on BS demodulation performance requirements MMSE-IRC

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.16.3

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the revised WID of NR demodulation performance requirements for NR BS with MMSE-IRC receiver was approved in [1], wherein the objectives related to NR BS MMSE-IRC receiver are listed as below:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we give some initial discussion on the NR BS MMSE-IRC receiver.

- Discussion

Based on the conclusion in RAN4 #112 meeting, some issues for BS demodulation performance requirements MMSE-IRC need further discussion.

With regard to the test scenarios, 2 proposals are proposed as below.

Test scenarios - Proposals: - Proposal 1: Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104 - Proposal 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile

As the requirements for HomNet and HetNet are defined in LTE, it is straightforward to cover these 2 scenarios for NR BS MMSE-IRC receiver. And these scenarios are reflected by DIP set1 and DIP set2 in TS 36.104.

With regard to proposal 2, such interference exists as different gNB may adopt different TDD pattern. If this type of interference is involved the interference profile needs to be discussed. Therefore, the above proposal 1 is prioritized and proposal 2 needs further discussion since additional interference source is to be reflected.

Proposal 1. Proposal 1 is prioritized and proposal 2 needs further discussion.

With regard to interference modelling criteria, 2 criteria are identified.

Interference modelling criteria - Candidate options on interference modelling criteria: - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value - Option 2: DIP-based modelling - Candidate options on DIP definition: - Option 1: Reuse definition of DIP in sub-clause B.6.1 of TS 36.104 ...

For the 2 options, option 2 uses the system level simulation derived DIP to define BS MMSE-IRC requirements. Option 1 is proposed by company that INR-based modelling + SNR-based definition can be considered.

For option 1, the requirements is defined by SNR, and the DIP is transferred into interference power in INR. The relationship between DIPi, I and noise power is as below.

,

From the above formula, each interference power can be calculated and added to the receiver. Since this method is used from LTE requirement definition. It is better to reuse this method to define the BS MMSE-IRC requirements.

Proposal 2. To adopt option 2: DIP-based modelling for interference modeling.

With regard to interference profile, the following is reached last meeting.

Interference profile - ... - On interference power level: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - FFS how to account for HPUE in existing LTE profiles.

The description of the interference profile in the revised WI is as follows.

Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

It cannot be determined whether there will be a significant change in the gain of MMSE-IRC relative to MMSE when HPUE is present as an interference source. From our initial simulation results which reuse LTE DIP sets, it can be seen that the MMSE-IRC has significant gain compared to MMSE. The gain varies when different parameters such as the number of Rx, selection of DIP sets and TDL channel settings are selected. If HPUE is to be reflected in the interference profile, the parameters of system level simulation need to be reached.

Proposal 3. Reusing LTE based interference profile is preferred. We also hold an open view on reflecting HPUE in the interference profile.

With regard to channel bandwidth, it is necessary to check all bandwidth configurations supported by an SCS.

Channel bandwidth - Candidate options for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose - Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz - Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz

Figure 2-1 below shows the MMSE vs MMSE-IRC for {15kHz SCS, 5MHz BW} and {30kHz SCS, 10MHz BW}

Figure 2-1 Performance comparison between MMSE and MMSE-IRC with 15kHz 5MHz and 30kHz 10MHz

From figure 2-1, we can find that the operating SNR for 5MHz and 10MHz with same receiving method is similar which is also pointed by other company. Therefore, it is preferred to select one channel bandwidth for each SCS, such 5MHz for 15kHz and 10MHz for 30kHz.

Proposal 4. To select one channel bandwidth for each SCS, for example, 5MHz BW for 15kHz SCS and 10MHz BW for 30kHz SCS.

With regard to waveform, it is pointed by company that DFT-s-OFDM is rare in the real deployment. If DFT-s-OFDM is rarely used in real deployment we can only define performance requirements for CP-OFDM.

Proposal 5. If DFT-s-OFDM is rarely used in real deployment, we can only define the performance requirements for CP-OFDM.

With regard to the antenna configuration, since BS side can support up to 8Rx, lager performance gain can be verified. 2/4Rx also need to be considered for different BS types. 1Tx can be prioritized for requirement definition as 1Tx is also used for LTE.

Antenna configuration - Candidate options: - Option 1: Cover 2/4/8 Rx with 1Tx - Option 2: Consider only 4RX and 8RX gNB - Option 3: Cover 2/4/8 Rx with 1/2/4 Tx - Option 4: Prioritize 4Rx case especially in the case of 2 explicit interferers - Option 5: Prioritize 1Tx

Proposal 6. Option 3: Cover 2/4/8 Rx with 1/2/4 Tx.

With regard to the channel model, some options are identified as below.

Propagation condition - Candidate options: - Option 1: - For the serving PUSCH, use TDLA30-10 and TDLC300-100 propagation channel model for HetNet and HomNet respectively - Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000. - Option 2: Select among legacy TDLA30, TDLB100, TDLC300 - Option 2A: - Cover TDLA30-10, TDLB100-400 and TDLC300-100 - Same propagation condition for serving and interference PUSCH - Option 2B: - For the serving PUSCH, use TDLA30-10 for HetNet and TDLB100-100 for HomNet - For the interference PUSCH, use TDLC300-100 - Option 2C: - Cover TDLB100-400, TDLC300-100 and TDLA30-10 low for study purpose, select one for requirement definition - For the interference PUSCH, use TDLC300-100

From table 8.2.6.1-1 to table 8.2.6.1-6 in TS 36.104[3] we can see that tested signal experiences one channel model(EPA 5 Low or EVA 70 Low), while the interferers (interferer 1 and interferer 2) experience another channel model(ETU 5 Low or ETU 70 Low). Channel models such as TDLA30-10 Low, TDLC300-100 Low and TDLB100-400 Low are used for NR BS requirement definition. These TDL channel models can be considered for requirement definition. From LTE requirement, it can be seen that the tested signal and the interferer signal experience different channel. If more than one MCS are considered, then different channel model can be selected for different MCS.

Proposal 7. To select one channel model, such as TDLB, as the channel model for interferer for NR BS MMSE-IRC requirement definition.

Proposal 8. Multiple channel models for the tested signal can be considered.

With regard to MCS for requirement definition, the following options are identified last meeting.

MCS - Candidate options: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20

From TS 36.104[3] we see the MCS used for LTE requirement definition which is summarized in table 2-1.

Table 2-1 LTE MCS and the close MCS in NR table 1

| Index | Modulation order | Code rate | Close MCS |
| --- | --- | --- | --- |
| 1 | QPSK | 0.36 | 5(0.3700) |
| 2 | 16QAM | 0.51 | 13(0.4785) 14(0.5400) |
| 3 | 16QAM | 3/4 | No close code rate for 16QAM 24 for 64QAM(0.7539) |

Due to the conclusion that QPSK/16QAM/64QAM need to be covered, it is recommended to at least use the MCS value given in option 1 for QPSK and 16QAM.

Proposal 9. To consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) for BS MMSE-IRC requirement definition and FFS the MCS for 64QAM.

# 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements of MMSE-IRC receiver for NR BS, The conclusions are:

Proposal 1. Proposal 1 is prioritized and proposal 2 needs further discussion.

Proposal 2. To adopt option 2: DIP-based modelling for interference modeling.

Proposal 3. Reusing LTE based interference profile is preferred. We also hold an open view on reflecting HPUE in the interference profile.

Proposal 4. To select one channel bandwidth for each SCS, for example, 5MHz BW for 15kHz SCS and 10MHz BW for 30kHz SCS.

Proposal 5. If DFT-s-OFDM is rarely used in real deployment, we can only define the performance requirements for CP-OFDM.

Proposal 6. Option 3: Cover 2/4/8 Rx with 1/2/4 Tx.

Proposal 7. To select one channel model, such as TDLB, as the channel model for interferer for NR BS MMSE-IRC requirement definition.

Proposal 8. Multiple channel models for the tested signal can be considered.

Proposal 9. To consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) for BS MMSE-IRC requirement definition and FFS the MCS for 64QAM.

# 4	Reference

- RP-241297, Revised WID on NR demodulation performance: Phase 5, .

- R4-2413570, WF on NR demodulation performance: Phase 5, China Telcom

- 3GPP TS 36.104 V18.5.0