# 3GPP TSG-RAN WG4 Meeting # 112bis                                 R4-2416350

# Hefei, China, 14 – 18 October, 2024

# Title: 	Discussion on AI/ML two-side model logistical issues

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.17.2.2

# Document for:	Approval

# 1 Introduction

In RAN #105 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

In RAN #105 plenary meeting, some conclusion has been reached for two-side model logistical issues

- RAN Plenary agrees to formally start implementing the infrastructure/platform needed by RAN4 for data sharing . - MCC to work out the details of the data sharing platform with input from RAN4 experts - Discussion on the platform setup with MCC to be triggered as soon as possible - Inputs to MCC based on previous RAN4 discussions (data size, etc) - RAN4 to discuss the technical aspects (data formats, model formats etc. for purposes of studying the two-sided CSI compression) in the next RAN4 meeting

During the last meeting, RAN4 had some offline discussions on AI/ML two-side model logistical issues. In the following section, we will provide the detailed discussions for logistical issues.

# 2 Discussion

As RAN4 discussed before, in order to achieve testability and interoperability, datasets or AI/ML models need to be collected or shared. Especially for CSI compression, both the TE vendor and the DUT vendor need to obtain sufficient information to train the decoder or encoder. Therefore, RAN4 discussed the issues related to data size and data formats, among others in previous offline meetings. Currently, the main technical issues involve data formats, model formats and the storage size, etc, which caused by different companies use different platforms to generate training data (Matlab, Python), and also use different platforms to achieve AI/ML model (PyTorch, TensorFlow). In order to be uploaded to the platform, standardized format management is essential.

In previous discussion, RAN4 gave some candidate size for training data and AI/ML model storage as follows.

- Training data - Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance) - Total space needed would depend on the number of contributing companies and meetings - Upper bound should be ~60GB per company per year, possible ~1TB per year to accommodate all companies - Data sharing might not be needed in every meeting

- AI/ML Model Storage/Sharing - Model size: ~7MB – 100MB - Total space needed would depend on the number of companies contributing and meetings - Upper bound should be 600MB per company per year -> ~10GB per year

In previous meeting, based on current research on CSI compression, RAN4 expects have roughly calculated the storage size required for training data and AI/ML model as shown in above. Generally, for the storage of models, the storage size can be accommodated from MCC’s feedback. Given the large storage space required for training data, this poses a challenge for uploading, storing and downloading. Based on our understanding, RAN4 should confirm with MCC whether the large storage can be supported and ensure no technical issues during upload and download.

Observation 1. For training data, a substantial amount of storage space is required, which poses a challenge for uploading, storing and downloading.

Meanwhile, another crucial aspect related to formatting is the implementation of AI/ML model code. Common platforms for AI/ML algorithms include PyTorch and TensorFlow. It has been clearly pointed out that there are differences in the implementation platforms among various companies. In previous discussion, some companies proposed using the open source ONNX for conversion between different platforms, which appears to be the only tool capable of achieving conversion. ONNX supports a wide range of tools and frameworks for both training and inference, including PyTorch, TensorFlow, Keras, and others. This interoperability is crucial for moving models form one environment to another without having to rewrite them from scratch. In summary, we also believe that the ONNX can meet the expectations of RAN4. However, we believe there are additional aspect that required consideration.

Firstly, while ONNX supports a wide range of framework, there might be some specific features or proprietary functions in certain frameworks that are fully covered by ONNX. And secondly, ONNX could introduce conversion overhead during the import and export stages, potentially affecting the performance of deep learning models when moving between different frameworks. Other considerations include challenges in maintaining synchronization with updates across frameworks and the potential for high resource consumption during the model conversion process using ONNX.

As a consequence, RAN4 should at least assess the feasibility of ONNX during the WI phase to ensure that conversions between different framework platforms can be fully and accurately realized.

Observation 2. ONNX still have some significant disadvantages, which include interoperability limitations, performance issues, update synchronization and resource consumption.

Proposal 1. RAN4 at least assess the feasibility of ONNX during the WI phase to ensure that conversions between different framework platforms can be fully and accurately realized.

Regarding data formats, theoretically, both .mat files and .npy files can be read by Python platform, which means both file formats are viable options for us. Here, we would like to emphasize that for training data, it would be preferable to have a description of the dataset files. All uploaded data needs to have the same pre-processing procedure or detailed description about pre-processing and follow the assumptions of RAN4.

Observation 3. Python is capable of reading .mat files and .npy files.

Proposal 2. All uploaded data needs to have the same pre-processing procedure or detailed description about pre-processing and follow the assumptions of RAN4.

On the other hand, since the datasets and models involve multiple companies and regions, from a policy and regulatory perspective, standardized or uploaded datasets must comply with legal restrictions. 3GPP also needs to further discuss how to ensure the security of the datasets or models and compliance with regulatory constraints in different regions.

Observation 4. The datasets and models may be subject to regulatory constraints in different regions during the upload and utilization process.

Observation 5. 3GPP may need to further discuss how to ensure the security of the datasets or models and compliance with regulatory constraints in different regions.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for CSI prediction , The conclusions are:

Observation 1. For training data, a substantial amount of storage space is required, which poses a challenge for uploading, storing and downloading.

Observation 2. ONNX still have some significant disadvantages, which include interoperability limitations, performance issues, update synchronization and resource consumption.

Observation 3. Python is capable of reading .mat files and .npy files.

Observation 4. The datasets and models may be subject to regulatory constraints in different regions during the upload and utilization process.

Observation 5. 3GPP may need to further discuss how to ensure the security of the datasets or models and compliance with regulatory constraints in different regions.

Proposal 1. RAN4 at least assess the feasibility of ONNX during the WI phase to ensure that conversions between different framework platforms can be fully and accurately realized.

Proposal 2. All uploaded data needs to have the same pre-processing procedure or detailed description about pre-processing and follow the assumptions of RAN4.

# 4 References

- RP-242350, WF on AI/ML Logistical Issues, CAICT, Qualcomm.