3GPP TSG-RAN WG4 Meeting #112bis								                          R4-2416361

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:			6.1.2

Source:	Ericsson

Title:	Discussions on RRM impact of Rel-19 WI on UE RF enhancements WI

Document for:	Discussions

# Introduction

In 3GPP Release 18, RAN4 finalized the standardization efforts for enabling 8Rx for CPE/FWA/vehicle/industrial type of UEs. Although 6Rx was proposed and deliberated upon, it was given lower priority during the Rel-18 discussion. To enhance DL coverage and throughput for handheld UEs with larger sizes, RAN4 has approved a new WI to specify requirements for both handheld and FWA UEs [1].

High power UE (HPUE) for CA in terrestrial network (TN) - Specify the generic requirements of high power UE (HPUE) for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO - Example band combinations: - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO - Example band combinations: - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs - PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA - Focus on the SAR solution - Enable power class 2 (PC2) and PC1.5 of two band inter-band uplink CA and EN-DC with 3Tx for handheld UE - Identify and update the requirements if necessary - Only PC3 is considered for LTE FDD in EN-DC - NOTE: leave the band combination specific requirements, e.g., MSD to the corresponding Rel-19 basket WIs - Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified - Introduce the signaling to support the above objectives, if needed. - Consider release independency, if needed Power boosting and/or MPR reduction - Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives 6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

The RRM impact of this new WI was discussed at last meeting and good progress was reached which is summarized in [2].  This way forward contains one open issue about interruption requirements at SRS antenna switching which are discussed in this paper.

# Discussions

Interruptions at SRS antenna port switching was discussed and following was captured in [2]:

<Way Forward> Issue 1-4: Interruption requirements at SRS antenna switching for 6Rx capable UE - Proposals - Option 1(Nokia): To wait for RAN1 conclusion on the 6Rx SRS antenna switching pattern to make sure that the legacy requirements can be reused. - Option 2(Huawei, Ericsson): The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed.

The legacy UE interruption requirements at SRS antenna port switching are defined as follows [3]:

##### 8.2.2.2.16 Interruptions at NR SRS antenna port switching The requirements in this clause are applicable to SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot. For interruption caused by SRS antenna port switching, the victim cell is based on the entry number of the band indicated by txSwitchImpactToRx and/or txSwitchWithAnotherBand regardless of per-FR MG capability. An UL interruption is allowed on any of the serving cells as indicated in txSwitchWithAnotherBand, and a DL interruption is allowed on any of the serving cells as indicated in txSwitchImpactToRx. The UE shall perform SRS antenna port switching only if the below conditions are met. - the SRS switching is not colliding with any other UL transmission with higher priority defined in TS 38.214 [26] if the serving cell on which the higher priority transmission is performed is a victim cell based on txSwitchWithAnotherBand or is the same carrier on which SRS is transmitted. - the SRS switching is not colliding with any NR measurements (i.e. SSB/CSI-RS based L1/L3 measurements) and the measurements for RLM/BFD/CBD if the serving cell on which the NR measurements and the measurements for RLM/BFD/CBD is performed is a victim cell based on txSwitchImpactToRx or is the same carrier on which SRS is transmitted. No requirements are defined for SRS antenna port switching if aperiodic SRS switching is colliding with aperiodic L1-RSRP/L1-SINR measurements and the serving cell on which the aperiodic L1-RSRP/L1-SINR measurement is configured is indicated in txSwitchImpactToRx or is the same carrier on which aperiodic SRS is scheduled/configured. When 1 SRS symbol is configured in a slot for SRS antenna switching and the aggressor and victim cells are synchronized, the interruption requirement in Table 8.2.2.2.16-1 applies. When 1 SRS symbol is configured in a slot for SRS antenna switching and the aggressor and victim cells are asynchronized, the interruption requirement in Table 8.2.2.2.16-2 applies. For the rest of SRS configurations, the interruption requirement in Table 8.2.2.2.16-3 applies. Table 8.2.2.2.16-1: Interruption length in symbols of victim CC when 1 SRS symbol is configured, and aggressor and victim cells are synchronized Victim cell SCS(kHz); Aggressor cell SCS (kHz). 15; 30; 60. 15; 3; 2; 2. 30; 4; 3; 3. 60; 8; 6; 5. 120; 14; 10; 8. Table 8.2.2.2.16.2: Interruption length in slots of victim CC when 1 SRS symbol is configured, and aggressor and victim cells are asynchronized Victim cell SCS(kHz); Aggressor cell SCS (kHz). 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 2; 2; 2. 120; 2; 2; 2. Table 8.2.2.2.16-3: Interruption length in slots of victim CC for rest of the SRS configurations for synchronised and asynchronized scenarios Victim cell SCS(kHz); Aggressor cell SCS (kHz). 15; 30; 60. 15; 2; 2; 2. 30; 2; 2; 2. 60; 3; 2; 2. 120; 5; 3; 3.

It was brought to attention at last meeting that the above requirements apply only when the SRS resources are configured within the last 6 symbols of a slot in Rel-17 (see the highlighted text), and RAN4 needs to confirm whether that is also the case in Rel-19.

In our understanding, beside the condition on SRS resource allocation, the interruption requirements at SRS antenna port switching are quite generic and do not depend on the number of Rx in the UE. From this perspective, the existing interruption requirements should also be applicable for 6Rx capable UEs.

From the RAN1 specification point of view, a single SRS resource can occupy more than 4 symbols per slot. More specifically, in NR Rel-17 the number of symbols was extended to include 8, 10, 12 and 14. However, the RAN4 requirements apply only when the SRS resources are configured within the last 6 symbols of a slot. In our understanding, no further discussions related to SRS resource design change is expected in Rel-19 in RAN1. Therefore, we propose to reuse the existing interruption requirements for Rel-19 4Rx capable UEs.

- The legacy interruption requirements for SRS antenna port switching are reused for 4 Rx capable UEs.

Regarding the impact on SNR levels used in RLM and BFD tests, following was captured in [2]:

<Way Forward> Issue 1-6: SNR level for RLM and BFD testing for 6Rx capable UE - Proposals - Option 1(Huawei): Reuse SNR levels specified for 4Rx capable UE - Option 2 (Ericsson): RAN4 to discuss impact on RLM/BFD tests when operating with 6 Rx UEs

SNR levels for testing belongs to the performance part of the work item, and we therefore propose to postpone the discussions.

- Impact on SNR levels for RLM and BFD tests is discussed in the performance part of the WI.

# Conclusion

In this contribution we have discussed the interruption requirements at SRS antenna port switching and also provided our view on SNR levels for RLM/BFD tests. Based on the discussions, we have made following proposals:

- The legacy interruption requirements for SRS antenna port switching are reused for 4 Rx capable UEs.

- Impact on SNR levels for RLM and BFD tests is discussed in the performance part of the WI.

# References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- R4-2413972, WF on RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC, Huawei, HiSilicon

- TS 38.133 V18.5.0