3GPP TSG-RAN WG4 Meeting #112bis	R4-2416382

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	ZTE Corporation

Title:	Discussion on the Interoperability and testability aspects

Agenda Item:	6.18.3

Document for:	Approval

- Introduction

In RAN plenary #105 meeting, a revised SID[1] for Rel-19 was approved to study the AI/ML for mobility in NR. The objectives for RAN4 of this SID are listed as follows.

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In this document, we will provide some initial views on the interoperability and testability of AI/ML for mobility from the RRM perspective.

- Discussion

According to the WID, we should discuss the testability and interoperability based on RAN2 framework. It’s the first meeting for AI mob in RAN4, we can start the study based on the RAN2 agreemrnts. Meanwhile, the discussion in R18 AI Phy also can be used as baseline. Due to testability and interoperability are discussing under work from “AI/ML for NR air interface”, we should wait for their conclusion in some issues.

2.1 Interoperability

In this part, we should figure out what interoperability is. In R18 AI Phy, interoperability has been discussed and RAN1 proposed three levels for network-UE collaboration levels.

1.	Level x: No collaboration

2.	Level y: Signaling-based collaboration without model transfer

3.	Level z: Signaling-based collaboration with model transfer

To facilitate the discussion, consider at least the following Cases for model delivery/transfer to UE, training location, and model delivery/transfer format combinations for UE-side models and UE-part of two-sided models.

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top | Outside 3gpp Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format | 3GPP Network | NW-side |
| z3 | model transfer in open format | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE | 3GPP Network | NW-side |

In our understanding, no need to discuss the interoperability for level x, which is not involved network-UE collaboration. For level y and level z, based on signaling, some information need transmit between UE and network whether with transfer model or not, so it’s necessary to discuss the interoperability on network-UE collaboration here.

In R18 AI, RAN4 discussed one side model and two side model on AI/ML framework [2]. According to the description on TR 38.843, one-sided (AI/ML) model could be a UE-side (AI/ML) model or a Network-side (AI/ML) model. UE-side (AI/ML) model is an AI/ML Model whose inference is performed entirely at the UE. Network-side (AI/ML) model is an AI/ML Model whose inference is performed entirely at the network. Two-sided (AI/ML) model is a paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e., the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa.

Fig 1. Reference block diagram for one-sided AI/ML model

Fig 2. Reference block diagram for 2-sided AI/ML model

For AI mobility, we can observe from revised WID, only one side model should be studied. For measurement prediction, UE sided and NW sided model should be consiodered for cell-level measurement prediction and beam-level measurement prediction. For HO failure/RLF prediction and measurement events prediction, just need to consider UE side model.

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

In our understanding, for one side mode, the inference is performed entirely on one side (UE side or NW side), so there should be no interoperability issues involved in the inference process. However, the interaction maybe happen before or after inference. Take UE side model as example, before inference, some information about measurement and prediction objects should be provided by network, and after inference, some results should be reported to NW for further discision. So, interoperability should be studied for one side model.

As mentioned above, the collaboration between UE and NW mainly depended on signalling. There is no conclusion on RAN2, so whether or which information should be deliver is not clear. In our view, it’s premature to discuss interoperability and we can start the discussion after RAN1/RAN2 have reach concensus on signalling design.

Observation 1: One side model is considered in AI mobility. For one side mode, the inference is performed entirely on one side (UE side or NW side), so there should be no interoperability issues involved in the inference process. However, the interaction maybe happened between UE and NW mainly depended on signalling.

Proposal 1：RAN4 to consider interoperability for one side model, including

- UE sided and NW sided model for cell-level and beam-level measurement prediction

- UE sided model for HO failure/RLF prediction

- UE sided model for Measurement events prediction

And detail should be further discussed based on RAN2’s input.

2.2 Testability

For testability part, we should study the issue about test feasibility. Above all, we should have a common understanding on testing goals. In AI/ML for NR air interface, RAN4 have discussed the similar issue for AI/ML model and have the agreement last meeting

Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be teste

In our understanding, we can reuse the conclusion on testing goals for R19 AI/ML monility. From RRM perspective, R19 AI/ML mobility is a machine to use AI/ML model to reduce measurement samples, predicting future measurements based on previous measurement results to achieve the goal of skipping future measurements, then reduce the delay requirements. In legacy measurement, the performance, i.e. accuracy, will be impacted if measurement samples are reduced. Even the measurement results of skipped samples are predicted, the network performance based on predicted results is highly likely to differ from the performance based on actual measurements due to the vary channel condition. So,it’s necessary to ensure the performance compared to legacy measurement.

Observation 2: Even the measurement results of skipped samples are predicted, the network performance based on predicted results is highly likely to differ from the performance based on actual measurements due to the vary channel condition.

Proposal 2: it’s necessary to ensure the performance compared to legacy measurement and the testing goal is to verify whether the minimum performance of AI/ML Mobility can be achieved.

Same as AI/ML model, RAN4 tests are executed by TE emulating the network and environment by generating signals based on abstract models, e.g, TDL or CDL channels. However, the channel conditions of real environment are complex and diversified, and the channel state is not stationary and constantly changing for mobility, which can easily lead to measurement results used for inference not being applicable to the inference stage, resulting in subsequent processes such as predicting measurement events and handover. The performance would degrade and UE may fail the test. So, we need to consider whether it is reliable to use the measurement results from the previous moment to infer the results from another moment under constantly changing channel conditions. That is to say, we need to discuss the uncertainty brought about by environmental changes. Furthermore, it needs further discussion whether the performance in practical network can be guaranteed even if the UE passes the defined tests.

Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Observation 3: the channel conditions of real environment are complex and diversified, and the channel state is not stationary and constantly changing for mobility, which can easily lead to measurement results used for inference not being applicable to the inference stage, resulting performance would degrade.

Proposal 3: RAN4 should further discussion whether the performance in practical network can be guaranteed based on prediction mesurement results.

Determine the configuration of the relationship between the observation set and the prediction set as part of the side conditions. In our understanding, due to environmental changes, some side conditions are needed to ensure the prediction can be based on existing measurements. For example, limiting the time interval between measurement and prediction, during which channel conditions do not change significantly. Similar to legacy measurement requirements, we can set a time restriction for the time-domain prediction model to minimize changes in channel conditions. That is to say, ensuring the freshness of the actual measurement results used for prediction.

Meanwhile, genralization performance is important for model evalution. From the test perspective, there is trade-off between supported scenarios and generalization performance verification. RAN4 is not able to test all identified scenarios and instead should try to cover more typical scenarios as much as possible.

For RRM measurement, it can be divided into FR1 and FR2, including intra-frequency intra cell measurement, intra-frequency inter cell measurement, inter-frequency intra cell measurement, and inter-frequency inter cell measurement. Additionally, it may be necessary to consider whether a gap is needed. According to the current assumption of RAN2, evaluation scenarios for RRM measurement prediction can be observed from blew table.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain caseA | 2ndgoal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain caseB | 1stgoal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1stgoal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2ndgoal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1stgoal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1stgoal | Intra-cell |

Meanwhile, RAN2 has the agreement that AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found. Thus, RAN4 should discuss whether and how to define and test the generalization performance of AI/ML model.

- AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found.

Observation 4: Genralization performance is important for prediction, RAN2 thinks that AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found, and RAN4 should discuss whether and how to define and test the generalization performance of AI/ML model.

When defining the core part and performance part requirements, the testability should also be considered. For AI/ML mobility, since it is a module based enhancement which has a bit difference from legacy RRM procedure, and the details within the module would be algorithm implementations, so maybe some new types of test should be considered. At least, to ensure the performance of prediction measurement, delay requirement and measurement accuracy should be verified. Under the condition that the measurement accuracy is met, the measurement delay should be smaller than the actual measurement.

Proposal 4: From the perspective of test, core part and performance part requirements should be considered, i.e. measurement delay and measurement accuracy.

As mentioned in the WID, RAN4 should study the testability and interoperability based on RAN2 framework, e.g., number of cells to measure, beams etc. In RRM measurement, number of cells and number of SSB are clearly stated in TS 38.133 clause 9 for traditional measurement, including intra-frequency measurement, inter-frequency measurement, and LTM measurement for neighbor cell.

|  | FR1 | FR2 |
| --- | --- | --- |
| 9.2.3 Intra-frequency measurements | For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and - 14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources. | For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PC |
| 9.3.3 Inter-frequency measurements | For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 4 identified cells, and - 7 SSBs with different SSB index and/or PCI on the inter-frequency layer. | For each inter-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 4 identified cells, and - 10 SSBs with different SSB index and/or PCI on the inter-frequency layer, and - 1 SSB per identified cell. |
| 9.10.2.3 CSI-RS based intra-frequency measurements | For each intra-frequency CSI-RS layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least: - 32 CSI-RSs with different CSI-RS index and/or PCI on the intra-frequency layer, and - the cells to be monitored based on CSI-RS are the same set or a subset of the cells monitored based on the layer of the associated SSB | For one single intra-frequency CSI-RS layer in a band, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least: - 32 CSI-RSs with different CSI-RS index and/or PCI, and - the cells to be monitored based on CSI-RS are the same set or a subset of the cells monitored based on the layer of the associated SSB. |
| 9.10.3.2 CSI-RS based Inter-frequency measurements | For each inter-frequency CSI-RS layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least: - 14 CSI-RSs with different CSI-RS index and/or PCI , and - The cells to be monitored based on CSI-RS are the same set or a subset of the cells monitored based on the layer of the associated SSB. | For each inter-frequency CSI-RS layer, during each layer 1 measurement period, the UE shall be capable of performing CSI-RSRP, CSI-RSRQ, and CSI-SINR measurements for at least: - 24 CSI-RSs with different CSI-RS index and/or PCI, and - The cells to be monitored based on CSI-RS are the same set or a subset of the cells monitored based on the layer the associated SSB. |
| 9.14.4 Intra-frequency LTM | The number of intra-frequency layers, number of cells and number of SSBs that UE shall be capable of performing L1-RSRP measurements on are reported by [TBD]. For L1-RSRP measurement on neighbour cell, UE measures only one intra-frequency layer on each FR2 band in CA scenario, where this single intra-frequency layer shall be the same frequency layer that UE performs L3 intra-frequency layer in that FR2 band. |  |
| 9.15.4 inter-frequency LTM | The number of inter-frequency layers, number of cells and number of SSBs that UE shall be capable of performing L1-RSRP measurements on are reported in [TBD]. |  |

In our view, if we want to use the AI/ML model to enhance RRM measurement, all of above should be considered for the integrity of the specfication. We can start the discussion on L3 SSB based measurement, and the number of cells and SSBs used for legacy can serve as a starting point for discussion.

Proposal 5: RAN4 to discuss the number of cell to measurement for AI/ML mob, and the number of cells and SSBs used for legacy intra-f/inter-f measurement can serve as a starting point for discussion.

- Conclusion

In this contribution, we have the following observations and proposals on the Interoperability and testability aspects of AI/ML mobility:

Observation 1: One side model is considered in AI mobility. For one side mode, the inference is performed entirely on one side (UE side or NW side), so there should be no interoperability issues involved in the inference process. However, the interaction maybe happened between UE and NW mainly depended on signalling.

Proposal 1：RAN4 to consider interoperability for one side model, including

- UE sided and NW sided model for cell-level and beam-level measurement prediction

- UE sided model for HO failure/RLF prediction

- UE sided model for Measurement events prediction

And detail should be further discussed based on RAN2’s input.

Observation 2: Even the measurement results of skipped samples are predicted, the network performance based on predicted results is highly likely to differ from the performance based on actual measurements due to the vary channel condition.

Proposal 2: it’s necessary to ensure the performance compared to legacy measurement and the testing goal is to verify whether the minimum performance of AI/ML Mobility can be achieved.

Observation 3: the channel conditions of real environment are complex and diversified, and the channel state is not stationary and constantly changing for mobility, which can easily lead to measurement results used for inference not being applicable to the inference stage, resulting performance would degrade.

Proposal 3: RAN4 should further discussion whether the performance in practical network can be guaranteed based on prediction mesurement results.

Observation 4: Genralization performance is important for prediction, RAN2 thinks that AI/ML model generalization could be addressed after sufficient performance gains for different use cases are found, and RAN4 should discuss whether and how to define and test the generalization performance of AI/ML model.

Proposal 4: From the perspective of test, core part and performance part requirements should be considered, i.e. measurement delay and measurement accuracy.

Proposal 5: RAN4 to discuss the number of cell to measurement for AI/ML mob, and the number of cells and SSBs used for legacy intra-f/inter-f measurement can serve as a starting point for discussion.

- Reference

- RP-242393, Revised SID on AIML for mobility in NR, 3GPP RAN#105 meeting.

- R4-2307730, Further discussion on interoperability and testability aspects for AI/ML, vivo.