3GPP TSG RAN WG4 Meeting #112bis	R4-2416391

Hefei, P.R. China, 14th – 18th October 2024

Source: 		Rohde & Schwarz

Title: 			Discussion on test setup for AI/ML based beam management

Agenda Item: 	6.17.4.2

Document for: 	Discussion

# Introduction

In the previous RAN4 meetings, testability aspects of AI/ML based beam management were discussed, with the final goal, the definition of a baseline test system. Given that this discussion is quite complex and time demanding, in the last RAN plenary it was proposed to outsource the testability discussion from the AI/ML Air WID to the FR2 OTA Testability SID. This proposal was not approved. Instead the AI/ML Air WID was revised ([4]) and is stating now explicitly, that the testability aspects need to be discussed along with AI/ML based BM core requirements.

In this paper we provide our view on the need of clear core requirements and the respective impact on the testability aspects.

# Discussion

The inter-dependencies between AI/ML based BM core requirements and the respective test setup introduce a “chicken and egg” challenge. As a test equipment provider, we do understand the need of a test system which can accommodate a broad set of capabilities for testing the various and multi-dimensional aspects of the AI/ML based BM. However, given the OTA environment required, because of FR2, such a system can become very easily extremely complex, hardly maintainable and commercially unfeasible.

Observation 1: Because of the OTA environment and multi-dimensional aspects of AI/ML BM, the test system can become extremely complex, hardly maintainable and commercially unfeasible.

Also, the past experience has shown that only the convergence to the least possible number of system types, with a broader coverage across the features, ensures a feasible commercialization of systems, and at the end of the day the actual testing of the features. The introduction of numerous test system types driven by the specific needs of dedicated features, or the introduction of single full-blown test system driven by the needs of a very complex feature, would rather be a wrong (and a bit “naive”) decision at this stage. We need to define a system with broader coverage in terms of features and time, but also with a reasonable complexity, which makes it affordable also to vendors not targeting very complex features.

Observation 2: AI/ML BM testing needs to be covered in a test system with broader coverage in terms of other features and lifetime, with reasonable complexity, which makes it affordable also to vendors not targeting AI/ML BM testing.

Our impression from the last meetings, is that the discussion on AI/ML BM testability was initiated and progressed quickly based on the assumption of CDL channel models and the availability of a certain system type (MIMO-OTA) to support such testing. However detailed discussions (example [2]) showed that upscaling of such systems for the AI/ML BM scenarios (keywords: multiple beams and clusters) are not feasible. Other detailed discussion, showed that there is not yet a common understanding among vendors about the necessity of such a channel, but above all about the overall scope (including all aspects) of the AI/ML BM testing.

Observation 3: AI/ML BM testability discussion started based on the assumption that CDL channel model are relevant for AI/ML BM core requirement and that the MIMO-OTA system supports such testing.

Observation 4: Detailed testability discussions showed that upscaling the MIMO OTA system for the AI/ML BM scenarios is not feasible.

Observation 5: Detailed requirement discussions showed that there is not yet a common understanding among vendors about the necessity of CDL channel and about the overall (including all aspects) scope of the AI/ML BM testing.

Given that the consequences of such an uncertainty on the test setup complexity are huge in this case, it is of a crucial importance to first converge to a common understanding and expectation on the core requirements. Only then, a testability discussion can be conducted efficiently and goal-oriented.

Proposal 1: RAN4 to first converge to a common understanding and expectation about the AI/ML BM core requirements. Only then, a testability discussion can be conducted efficiently and goal-oriented.

In our view there are many aspects which need to be discussed and concluded whether they are relevant and need to be included in the AI/ML BM requirements or not.

Observation 6: There are many aspects which need to be discussed and concluded whether they are relevant and need to be included in the AI/ML BM requirements or not.

As per our understanding, the AI/ML model in case of BM, predicts Set A L1-RSRP based on input of Set B L1-RSRP. At this stage, only the input and output of L1-RSRP is relevant and no other aspects of L1-RSRP measurements are involved. If the AI/ML BM requirements are targeting only the AI/ML model performance, the requirement for the test system are the less complex ones.

Question 1: Do the AI/ML BM requirements focus only on the performance of the AI/ML model, which deals only with L1-RSRP values (input: Set B beams L1-RSRP, output: predicted Set A beams L1-RSRP), or go beyond that, including the performance of other aspects?

Observation 7: If AI/ML BM requirements focus only on AI/ML model performance, the requirement for the test system are the less complex ones.

If as part of AI/ML performance we include also the L1-RSRP measurement performance, then we need to deal with the main challenge of FR2 OTA RRM requirements, the very big tolerances of the UE RSRP measurement accuracy. We need to check the numbers exactly, but such big tolerances might loosen the results of AI/ML testing, in extreme case to meaningless results. Probably the agreement of a reference antenna gain, would reduce the RSRP accuracies and increase the significance of AI/ML testing in case of RSRP measurement inclusion. On the other side the TE uncertainty need to be minimized and this is ensured only by CATR based systems.

Question 2: Do the AI/ML BM requirements include the L1-RSRP UE measurement performance?

Observation 8: If L1-RSRP UE measurement performance is included, large accuracy tolerances might loosen the AI/ML performance, reference antenna gain might be required and CATR technology in unavoidable.

At the next step the UE Rx beam sweeping might be included in the AI/ML BM performance. Especially for this aspect the ambiguity seems to be bigger, since as per some references only Tx Beams are targeted in the current scope of the WI. The consideration of the different UE Rx beams is probably the first trigger for a multi AoA system, since in case of a locked UE Rx beams, inputing the UE with DL signal is also possible with a single AoA.

Question 3: Do the AI/ML BM requirements include UE Rx beam sweeping performance?

Observation 9: If UE Rx beam sweeping performance is not included, single AoA testing might be sufficient.

The other aspect of the core requirement is the number of Tx Beams. Here several questions arise. How many Set B beams are required? Are the prediction results validated against emulated Set A beams? If yes, how many Set A beams are emulated? From a testability perspective the number of beams become very critical, if combined with directivity aspect and spatial (multi AoA) channel models, as discussed below.

Question 4: How many Set B beams are required? Are the prediction results validated against emulated Set A beams? If yes, how many Set A beams are emulated?

Observation 10: From a testability perspective the number of Tx beams become very critical, if combined with directivity aspects and spatial (multi AoA) channel models.

The channel model seems in fact to be the last aspect of the discussion (so far it was somehow treated as the first). The first question with this regard, is whether dealing with multipath fading channel models is part of the AI/ML scope, and if yes, of which type? At this stage, from the TE perspective, the channel is simply SW signal processing effort. The question is more crucial for core requirement, since it is not clear how to test real performance AI/ML models with a testing-oriented (synthetic) channel models.

Question 5: Do the AI/ML BM requirements include the performance under multipath fading channel model? If yes, of which type?

Observation 11: It is not clear how to test real performance AI/ML models with testing-oriented (synthetic) channel models.

From a testability perspective inclusion of the channel becomes crucial if combined with directivity. Thus, the question for the core requirement is, is directivity performance (Tx / Rx) in the scope of AI/ML BM performance?  As discussed in the meetings before, spatial channel models (CDL based) are a complexity multiplier for the TE HW complexity, implying huge OTA chamber with very large number of RF channels. The current impression is that support of spatial (multi AoA) channels goes on the cost of reducing extremely the number of Set B/A beams, which in fact underperform the intended AI/ML model, which is the main test objective.

Question 6: Do the AI/ML BM requirements include UE directivity performance and performance under spatial (multi AoA) multipath fading channel models?

Observation 12: Support of spatial (multi AoA) channels goes on the cost of reducing extremely the number of Set B/A beams, which in fact underperform the intended AI/ML model, which is the main test objective.

On a last note, considering the complex cross-dependencies of the aspects above, for both requirements as well as for testing, we think the requirements should focus on the essential KPIs impacting the performance of AI/ML model. Including too many aspects, which are tested also elsewhere separately, diverge the testing scope to an end-to-end test, risking to leave the key component, AI/ML model, not thoroughly tested.

Proposal 2: The AI/ML BM requirements should focus on the essential KPIs impacting the performance of AI/ML model.

Observation 13: Including too many aspects in the AI/ML BM requirement, which are tested also elsewhere separately, diverge the testing scope to an end-to-end test, risking to leave the key component, AI/ML model, not thoroughly tested.

# Conclusions

In this contribution, the following observations, proposals and questions were made.

Observation 1: Because of the OTA environment and multi-dimensional aspects of AI/ML BM, the test system can become extremely complex, hardly maintainable and commercially unfeasible.

Observation 2: AI/ML BM testing needs to be covered in a test system with broader coverage in terms of other features and lifetime, with reasonable complexity, which makes it affordable also to vendors not targeting AI/ML BM testing.

Observation 3: AI/ML BM testability discussion started based on the assumption that CDL channel model are relevant for AI/ML BM core requirement and that the MIMO-OTA system supports such testing.

Observation 4: Detailed testability discussions showed that upscaling the MIMO OTA system for the AI/ML BM scenarios is not feasible.

Observation 5: Detailed requirement discussions showed that there is not yet a common understanding among vendors about the necessity of CDL channel and about the overall (including all aspects) scope of the AI/ML BM testing.

Proposal 1: RAN4 to first converge to a common understanding and expectation about the AI/ML BM core requirements. Only then, a testability discussion can be conducted efficiently and goal-oriented.

Observation 6: There are many aspects which need to be discussed and concluded whether they are relevant and need to be included in the AI/ML BM requirements or not.

Question 1: Do the AI/ML BM requirements focus only on the performance of the AI/ML model, which deals only with L1-RSRP values (input: Set B beams L1-RSRP, output: predicted Set A beams L1-RSRP), or go beyond that, including the performance of other aspects?

Observation 7: If AI/ML BM requirements focus only on AI/ML model performance, the requirement for the test system are the less complex ones.

Question 2: Do the AI/ML BM requirements include the L1-RSRP UE measurement performance?

Observation 8: If L1-RSRP UE measurement performance is included, large accuracy tolerances might loosen the AI/ML performance, reference antenna gain might be required and CATR technology in unavoidable.

Question 3: Do the AI/ML BM requirements include UE Rx beam sweeping performance?

Observation 9: If UE Rx beam sweeping performance is not included, single AoA testing might be sufficient.

Question 4: How many Set B beams are required? Are the prediction results validated against emulated Set A beams? If yes, how many Set A beams are emulated?

Observation 10: From a testability perspective the number of Tx beams become very critical, if combined with directivity aspects and spatial (multi AoA) channel models.

Question 5: Do the AI/ML BM requirements include the performance under multipath fading channel model? If yes, of which type?

Observation 11: It is not clear how to test real performance AI/ML models with testing-oriented (synthetic) channel models.

Question 6: Do the AI/ML BM requirements include UE directivity performance and performance under spatial (multi AoA) multipath fading channel models?

Observation 12: Support of spatial (multi AoA) channels goes on the cost of reducing extremely the number of Set B/A beams, which in fact underperform the intended AI/ML model, which is the main test objective.

Proposal 2: The AI/ML BM requirements should focus on the essential KPIs impacting the performance of AI/ML model.

Observation 13: Including too many aspects in the AI/ML BM requirement, which are tested also elsewhere separately, diverge the testing scope to an end-to-end test, risking to leave the key component, AI/ML model, not thoroughly tested.

# References

- R4-2409739, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #111, May 2024

- R4-2411587, “Discussion on test setup for AI/ML based beam management”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #112, August 2024

- R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, 3GPP TSG RAN WG4 Meeting #112, August 2024

- RP-242399, “Revised WID: AI/ML for NR air interface”, Qualcomm, 3GPP TSG RAN Meeting #105, September 2024

- RP-242400, “Revised SID: Study on NR frequency range 2 (FR2) OTA (Over the Air) testing Phase 3”, Qualcomm, 3GPP TSG RAN Meeting #105, September 2024