3GPP TSG RAN WG4 Meeting #112bis	R4-2416395

Hefei, China, 14th – 18th October 2024

Source:		Rohde & Schwarz

Title:	Discussion on test decoder for CSI compression

Agenda Item:	6.17.2.1

Document for:	Approval

# Introduction

In this contribution, we analyze the reconstruction accuracy of the CNN-based model proposed for the Option 3 feasibility study for the CSI compression use case. Following the recent 3GPP RAN4#112 meeting, we made several adjustments to the data set generation and training process based on the alignments described in [1]. We present our updated simulation results in this contribution.

# Discussion

In the following, we investigate the reconstruction accuracy of the CNN-based model proposed for the Option 3 feasibility study for the CSI compression use case.

## Model Architecture

Following the 3GPP RAN4#111 meeting, after two rounds of e-mail discussion the following CNN-based model was proposed for the Option 3 feasibility study:

| Figure 1 CNN-based (Resnet) Encoder | Figure 2 CNN-based (Resnet) Decoder |
| --- | --- |

Model implementation details:

- We use a fixed uniform scalar quantization with equal ranges (the quantizer and dequantizer do not contain trainable neurons)

- During backpropagation, the quantization layer is bypassed, i.e., the gradient is passed through

- We use a sigmoid activation function after the last fully-connected (FC) layer of the encoder

## Dataset Parameters

Our evaluation is based on the following parameters:

| Parameter | Value |
| --- | --- |
| Scenario | Urban Macro (UMa) |
| Inter-BS distance | 200 m |
| Channel model | According to TR 38.901 |
| Antenna Setup gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna Setup UE | 4 RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ Ideal omni-directional antenna elements |
| BS Antenna Height | 25 m |
| UE Antenna Height | According to TR36.873 (In [2] we used a fixed height of 1.5 m) |
| UE Distribution | 80% indoor, 20% outdoor |
| UE Velocity | Indoor: 3 km/h, Outdoor: 30 km/h (In [2] we used a fixed velocity of 0 km/h) |
| Carrier Frequency | 2 GHz |
| Subcarrier Spacing | 15 kHz |
| Bandwidth | 10 MHz |
| No. of PRBs | 52 |
| No. of Subbands | 13 |
| Subband Channel Averaging | Average channel matrices per PRBs in the subband before SVD; 1 sample per PRB is selected |
| Channel Estimation | Ideal |
| MIMO Layers | 1 |
| Any Other Pre-Processing | Phase normalization is applied such that the phase of the first value of each eigenvector is 0; the L2 norm of each eigenvector is 1 |
| Data Collection Assumptions | Each channel realization is collected from independent UE drop |
| Dataset Quantization | Float32 |

Table 1 Simulation parameters

## Dataset Size

The training/validation/test data sets generated according to the simulation parameters listed in Section 2.2 contain the following number of samples:

- Training Set: 614,440 samples

- Validation Set: 81,920 samples

- Test Set: 81,920 samples

## Training Parameters

The following table lists the training parameters, which have been adjusted based on the alignment described in [1]:

| Parameter | Value |
| --- | --- |
| Optimizer | Adam |
| Learning Rate | Fixed learning rate of 1e-4 |
| Loss Function | 1-SGCS |
| Batch Size | 256 |
| Max. Epochs | 150 |
| Stopping Criteria | No improvement of validation loss over 25 epochs |
| Drop-Outs | None |
| Weight Initialization | Default |
| Kernel Parameters | Default |

Table 2 Training Parameters

## Simulation Results

Our simulations reveal the following results:

| No. of conv filters in decoder | No. of Resnet blocks in decoder | SGCS | MFLOPs |
| --- | --- | --- | --- |
| 128 | 6 | 0.7728 | 1,475.4 |

Table 3 Simulation results for CNN-based (Resnet) model

Our updates to data set generation and training parameters impact the observed reconstruction accuracy results only marginally (in [2] we reported a SGCS of 0.7650 for the CNN-based model). Since no adjustments were made to the neural network architecture, the computational complexity (i.e., number of MFLOPs) remains unchanged.

Observation 1: For the CNN-based (Resnet) model selected for the Option 3 feasibility study, we observe a SGCS of 0.7728 after considering the alignment decisions described in R4-2414447. The deviation from our previously reported reconstruction accuracy results is marginal.

Observation 2: The computational complexity of the CNN-based (Resnet) test decoder remains unchanged in comparison to our previous evaluation (1,475 MFLOPs).

# Summary

In this contribution, we reevaluated the reconstruction accuracy of the CNN-based (Resnet) model selected for the Option 3 feasibility study with data set and training parameters adjusted according to the alignment described in [1]. The reconstruction accuracy deviates only marginally from our previously reported results. The computational complexity of the test decoder remains unchanged.

Observation 1: For the CNN-based (Resnet) model selected for the Option 3 feasibility study, we observe a SGCS of 0.7728 after considering the alignment decisions described in R4-2414447. The deviation from our previously reported reconstruction accuracy results is marginal.

Observation 2: The computational complexity of the CNN-based (Resnet) test decoder remains unchanged in comparison to our previous evaluation (1,475 MFLOPs).

# References

- R4-2414447; “Alignment of model training and data set parameters for AI/ML-based CSI compression”; Nokia; RAN4#112 August 2024

- R4-2411534; “Discussion on test decoder for CSI compression”; Rohde & Schwarz; RAN4#112 August 2024