# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416416

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Draft running LS Reply on Parameters for 14800 to 15350 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27

Response to:	LS R4-2400333 on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

Release:	Rel-19

Work Item:	FS_NR_IMT_4400_7125_14800MHz

Source:	TSG RAN WG4

To:	ITU-R WP 5D

Cc:	RAN

Contact person:	Michal Szydelko

michal.szydelko@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN WG4 received the incoming LS from ITU-R Working Party 5D on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27 (Att. 7.4 to 5D/134) and would like to thank for the opportunity to give input on this topic.

RAN WG4 has already replied for the 4400-4800 MHz frequency range (R4-2410576) and for the 7125 to 8400 MHz frequency range (R4-2414449).

The recommended IMT technology related parameters for the frequency range 14800 to 15350 MHz are given in Annex 1 of this LS.

The following should be noted:

- non-AAS BS architecture is not considered for 14800 to 15350 MHz frequency range.

- AAS limits always apply Over-the-Air (OTA).

- In the BS specification TS 38.104, AAS BS are identified as BS type 1-H or BS type 1-O for FR1 bands, and as BS type 2-O for FR2 bands.

- Parameters for the frequency range 14800 to 15350 MHz were derived based on [FR1/FR2/mix of FR1 and FR2] RF requirements.

The recommended IMT antenna characteristics are given in Annex 2 of this LS. The following should be noted:

- Parameters are interdependent and derived as a package, based on deployment scenarios and other requirements.

- .

# 2	Actions

To ITU-R WP5D

ACTION: 	3GPP RAN WG4 asks ITU-R WP 5D to consider the following information provided for 14800 to 15350 MHz frequency range in Annex 1 and Annex 2.

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #113		Orlando, US					18-22 November, 2024

TSG-RAN4 Meeting #114		Athens, Greece				17-21 February, 2025

ANNEX 1

IMT technology-related and deployment-related parameters for bands between 14800 and 15350 MHz

Table 1: IMT-2030 specification related parameters in 14800 to 15350 MHz

| No. | Parameter | Base station (non-AAS) (Note 1) | Base station (AAS) | Mobile station |
| --- | --- | --- | --- | --- |
| 1 | Duplex Method | TDD |  |  |
| 2 | Channel bandwidth (MHz) | Typical 100 MHz (200 MHz not precluded) |  |  |
| 3 | Signal bandwidth (MHz) | Signal bandwidth = NRB × SCS × 12 Will be derived from Channel bandwidth, see [1], § 5.3.2. |  |  |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | Not applicable | 0 dB | TBD |
| 4.2 | Spectral mask (dB) | Not applicable | TBD | TBD |
| 4.3 | ACLR (dB) | Not applicable | TBD: Subject to co-ex simulations | TBD: Subject to co-ex simulations |
| 4.4 | Spurious emissions /out of band emissions | Not applicable | Based on SM.329, and aligned with [1], § 9.7.5. | Based on SM.329 |
| 4.5 | Maximum output power (dBm) | Not applicable | See Item No. 1.13 in Table 4 for typical values | 23 dBm typical (Note 2) |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | Not applicable | 8 dB (WA BS) 13 dB (MR BS) 16 dB (LA BS) For BS class definitions, see [1], § 4.4 | TBD |
| 5.2 | Sensitivity (dBm) | Not applicable | To be specified | To be specified |
| 5.3 | Blocking response | Not applicable | TBD | TBD |
| 5.4 | ACS | Not applicable | TBD: Subject to co-ex simulations | TBD: Subject to co-ex simulations |
| 5.5 | SINR operating range (dB) | Not applicable | See below “SINR operating range and mapping function” |  |

Note 1:	Non-AAS is not expected to be considered for 14800 to 15350 MHz frequency range.

Note 2:	Refer to TR 38.922 [3] for more information on other values for channel bandwidth and maximum output power.

Note 3:	Base station Operating band unwanted emissions define all unwanted emissions in the supported downlink operating band plus the frequency ranges extending ΔfOBUE above and ΔfOBUE below each band. Base station Unwanted emissions outside of this frequency range are limited by the spurious emissions requirement.

Note 4:	Base Station In-band blocking applies in the supported uplink operating band plus the frequency ranges extending ΔfOOB above and ΔfOOB below each band, excluding the downlink frequency range in case of an FDD operating band. Out-of-band blocking applies from 1 MHz to 12.75 GHz, excluding the in-band blocking frequency range, but including the downlink frequency range in case of an FDD operating band. Requirements are defined assuming a receiver desensitization of 6 dB.

References used in the Table:

[1]	3GPP TS 38.104 v.18.6.0, “NR; Base Station (BS) radio transmission and reception”

[2]	3GPP TS 38.101-1 v.18.6.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[3]	3GPP TR 38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”.

SINR operating range and mapping function

The following equations approximate the throughput over a channel with a given SINR, when using link adaptation:

Throughput SINR [bps/Hz] =0                       for SINR≤SINRMIN   α∙SSINR                     for SINRMIN<SINR<SINRMAX α∙SSINRMAX               for SINR ≥ SINRMAX

Where:

- SSINR: Shannon bound, S(SINR) =log2(1+SINR) [bps/Hz];

- α: Attenuation factor, representing implementation losses;

- SINRMIN: Minimum SINR of the code set [dB];

- SINRMax: Maximum SINR of the code set [dB];

- The SINR values specified in Table 2 below are in dB but must be converted to linear scale in the formula above.

The parameters, SINRMIN and SINRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 2 represent a baseline case, which assumes:

- 1:1 antenna configurations

- AWGN channel model

- Link Adaptation (see Table 2 for details of the highest and lowest rate codes)

- No HARQ

Table 2: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN [dB] | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMax [dB] | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

_____________

ANNEX 2

Antenna characteristics for IMT AAS base stations

An extended version of the AAS array antenna model is created to support vertical sub-array geometries with fixed sub-array down-tilt. The model equations are summarized in Table 3.

Table 3: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan Where M and N is corresponding to (Row × Column) in Table 4, row 1.6. |
| Composite array radiation pattern | AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ Where M and N is corresponding to (Row × Column) in Table 4, row 1.6. |

Considering base stations are optimized for various factors including performance, cost, and coverage, it is expected that sub array configurations are relevant where a set of physical antenna elements are combined to form a logical element. The model comprises of a basic element pattern which is then combined appropriately based on the equations to form the sub array pattern and the composite pattern. Since dual polarized elements are used in typical base stations, each polarization separately is considered in the models. The models are selected so that they are simple and representative to model BS performance with sufficient confidence. The element pattern is based on a simple gaussian beam which has a flat sidelobe level. The Gaussian pattern is sufficiently wide and cover most of the regions of interest, especially in the elevation domain. Thus, the extended antenna model with sub arrays is recommended to represent the beamforming capability of IMT base stations in considered frequency ranges.

Table 4: Beamforming antenna characteristics for IMT-2030 in MHz

|  |  | Rural macro (If it’s available) | Suburban macro | Urban macro | Urban small cell (outdoor)/Micro cell | Indoor (small cell) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Base station antenna characteristics |  |  |  |  |  |
| 1.1 | Antenna pattern |  |  |  |  |  |
| 1.2 | Element gain (dBi) |  |  |  |  |  |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) |  |  |  |  |  |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) |  |  |  |  |  |
| 1.5 | Antenna polarization |  |  |  |  |  |
| 1.6 | Antenna array configuration (Row × Column) |  |  |  |  |  |
| 1.7 | Horizontal/Vertical radiating element/sub-array spacing, dh /dv |  |  |  |  |  |
| 1.7a | Number of element rows in sub-array, Msub |  |  |  |  |  |
| 1.7b | Vertical radiating element spacing in sub-array, dv,sub |  |  |  |  |  |
| 1.7c | Pre-set sub-array down-tilt, θsubtilt (degrees) |  |  |  |  |  |
| 1.8 | Array Ohmic loss (dB) |  |  |  |  |  |
| 1.9 | Conducted power (before Ohmic loss) per antenna element/sub-array (dBm) |  |  |  |  |  |
| 1.10 | Base station horizontal coverage range (degrees) |  |  |  |  |  |
| 1.11 | Base station vertical coverage range (degrees) |  |  |  |  |  |
| 1.12 | Mechanical downtilt (degrees) |  |  |  |  |  |
| 1.13 | Maximum base station output power/sector (e.i.r.p.) (dBm) |  |  |  |  |  |

_____________