3GPP TSG-RAN WG4 Meeting #112bis	R4- 2416434

Hefei, China, October 14 – 18, 2024

Source:	Nokia

Title:	Simulation results for PRS Bandwidth Aggregation

Agenda item:	4.2.2.5

Document for:	Discussion

Introduction

The Rel-18 WID [1] for NR positioning contains several enhancements, including PRS/SRS bandwidth aggregation:

- Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4].

PRS/SRS bandwidth aggregation has been studied in the FS_NR_pos_enh2 study item by RAN4 with company contributions and outcome of the study being captured in TR 38.859 [2].  Based on Rel-16 and Rel-17 link level simulation assumptions for RSTD and PRS-RSRP, respectively, in [3] and [4], RAN4 has agreed link level simulation assumptions for PRS bandwidth aggregation in [5].

We have provided initial preliminary simulation results on PRS RSTD with bandwidth aggregation in [6] and [7].  In this contribution we provide further updated simulation results for PRS RSTD and UE Rx-Tx time difference versus [9].

Simulation Assumptions

Updated simulation assumptions for PRS RSTD are captured in [8]. The general parameters and PRS configurations as listed in Table-1 and Table-2 of [8] are used in the simulation study. No RF/BB impairments are included in the simulation.

Simulation Results

In this section we provide the simulation results for the above listed simulation assumptions. The results are reported for 5-percentile and 95-percentile CDF of the PRS RSTD errors in Tc, and 90-percentile CDF of the UE Rx-Tx time difference errors in Tc for given Es/Iot for three cells [5].

FR1, AWGN Channel

RSTD results

The PRS RSTD error in Tc for AWGN channel in FR1, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells are listed in Table 1, Table 2, and Table 3.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 272 | 16 | 1 | 4 | 3 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 4 | -1 | 1 | -1 | 1 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -1 | 1 | -2 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 132 | 16 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 1: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 272 | 16 | 1 | 4 | 3 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 4 | -1 | 1 | -1 | 1 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 132 | 16 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 2: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-3,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -2 | 2 | -2 | 2 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 272 | 16 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 132 | 16 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 3: PRS RSTD error for AWGN channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for AWGN channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 4, Table 5, and Table 6.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 2 | 2 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 272 | 16 | 1 | 4 | 3 | 1 | 1 | 1 |
|  |  |  |  |  |  | 4 | 1 | 1 | 1 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 132 | 16 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 4: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 2 | 2 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 272 | 16 | 1 | 4 | 3 | 1 | 1 | 1 |
|  |  |  |  |  |  | 4 | 1 | 1 | 1 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 132 | 16 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 5: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-3,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 2 | 2 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 272 | 16 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 132 | 16 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 6: UE Rx-Tx error for AWGN channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

FR1, TDL-A Channel

RSTD results

The PRS RSTD error in Tc for TDL-A channel in FR1, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells are listed in Table 7, Table 8, and Table 9.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -9 | 10 | -11 | 9 |
|  |  |  |  |  |  | 3 | -16 | 15 | -19 | 17 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -16 | 15 | -15 | 15 |
|  |  |  |  |  |  | 3 | -7 | 7 | -7 | 7 |
|  |  | 272 | 16 | 1 | 4 | 2 | -7 | 7 | -7 | 6 |
|  |  |  |  |  |  | 3 | -6 | 6 | -6 | 6 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -16 | 14 | -TBD | 15 |
|  |  |  |  |  |  | 3 | -7 | 7 | -10 | 7 |
|  |  | 132 | 16 | 1 | 4 | 2 | -7 | 7 | -7 | 7 |
|  |  |  |  |  |  | 3 | -6 | 5 | -6 | 6 |

Table 7: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | -9 | 10 | -11 | 9 |
|  |  |  |  |  |  | 3 | -16 | 15 | -18 | 17 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | -16 | 15 | -15 | 15 |
|  |  |  |  |  |  | 3 | -7 | 7 | -7 | 7 |
|  |  | 272 | 16 | 1 | 4 | 2 | -7 | 7 | -7 | 6 |
|  |  |  |  |  |  | 3 | -6 | 6 | -6 | 6 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | -17 | 14 | -TBD | 15 |
|  |  |  |  |  |  | 3 | -7 | 7 | -10 | 7 |
|  |  | 132 | 16 | 1 | 4 | 2 | -7 | 7 | -8 | 7 |
|  |  |  |  |  |  | 3 | -6 | 5 | -6 | 6 |

Table 8: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | -9 | 9 | -9 | 9 |
|  |  |  |  |  |  | 3 | -15 | 16 | -16 | 17 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | -17 | 15 | -16 | 16 |
|  |  |  |  |  |  | 3 | -7 | 7 | -7 | 7 |
|  |  | 272 | 16 | 1 | 2 | 2 | -7 | 7 | -7 | 7 |
|  |  |  |  |  |  | 3 | -6 | 6 | -6 | 6 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | -17 | 15 | -16 | 16 |
|  |  |  |  |  |  | 3 | -7 | 7 | -8 | 7 |
|  |  | 132 | 16 | 1 | 2 | 2 | -7 | 7 | -7 | 7 |
|  |  |  |  |  |  | 3 | -6 | 6 | -6 | 6 |

Table 9: PRS RSTD error for TDL-A channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for TDL-A channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 10, Table 11, and Table 12.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 36 | 37 | 37 |
|  |  |  |  |  |  | 3 | 41 | 42 | 43 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 30 | 30 | 31 |
|  |  |  |  |  |  | 3 | 30 | 30 | 30 |
|  |  | 272 | 16 | 1 | 4 | 2 | 30 | 30 | 29 |
|  |  |  |  |  |  | 3 | 27 | 27 | 28 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 30 | 29 | 35 |
|  |  |  |  |  |  | 3 | 30 | 30 | 31 |
|  |  | 132 | 16 | 1 | 4 | 2 | 29 | 29 | 30 |
|  |  |  |  |  |  | 3 | 27 | 27 | 28 |

Table 10: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-6,-13,-13).

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 4 | 2 | 36 | 37 | 37 |
|  |  |  |  |  |  | 3 | 41 | 42 | 43 |
| 30 | 4 | 132 | 32 | 1 | 4 | 2 | 30 | 30 | 31 |
|  |  |  |  |  |  | 3 | 30 | 30 | 30 |
|  |  | 272 | 16 | 1 | 4 | 2 | 30 | 30 | 29 |
|  |  |  |  |  |  | 3 | 27 | 27 | 28 |
| 60 | 4 | 64 | 32 | 1 | 4 | 2 | 30 | 29 | 35 |
|  |  |  |  |  |  | 3 | 30 | 30 | 31 |
|  |  | 132 | 16 | 1 | 4 | 2 | 29 | 29 | 30 |
|  |  |  |  |  |  | 3 | 27 | 27 | 28 |

Table 11: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 15 | 2 | 104 | 64 | 1 | 2 | 2 | 36 | 37 | 36 |
|  |  |  |  |  |  | 3 | 41 | 42 | 42 |
| 30 | 4 | 132 | 32 | 1 | 2 | 2 | 31 | 30 | 31 |
|  |  |  |  |  |  | 3 | 30 | 30 | 30 |
|  |  | 272 | 16 | 1 | 2 | 2 | 30 | 30 | 30 |
|  |  |  |  |  |  | 3 | 27 | 27 | 27 |
| 60 | 4 | 64 | 32 | 1 | 2 | 2 | 31 | 30 | 31 |
|  |  |  |  |  |  | 3 | 30 | 30 | 30 |
|  |  | 132 | 16 | 1 | 2 | 2 | 29 | 30 | 30 |
|  |  |  |  |  |  | 3 | 27 | 27 | 27 |

Table 12: UE Rx-Tx error for TDL-A channel, FR1, in Tc for Es/IoT (-3, -6, -6) dB.

FR2, AWGN channel

RSTD results

The PRS RSTD error in Tc for AWGN channel in FR2, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells, respective, are listed in Table 13, Table 14, and Table 15.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -1 | 1 | -2 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 132 | 16 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 128 | 8 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 13: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 132 | 16 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 128 | 8 | 1 | 4 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 14: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 132 | 16 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |
|  |  | 128 | 8 | 1 | 2 | 2 | -1 | 1 | -1 | 1 |
|  |  |  |  |  |  | 3 | -1 | 1 | -1 | 1 |

Table 15: PRS RSTD error for AWGN channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for AWGN channel in FR2, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 16, Table 17, and Table 18.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 132 | 16 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 128 | 8 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 16: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 132 | 16 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 128 | 8 | 1 | 4 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 17: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 132 | 16 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |
|  |  | 128 | 8 | 1 | 2 | 2 | 1 | 1 | 1 |
|  |  |  |  |  |  | 3 | 1 | 1 | 1 |

Table 18: UE Rx-Tx error for AWGN channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

FR2, TDL-C channel

RSTD results

The PRS RSTD error in Tc for TDL-C channel in FR2, derived from the 5-percentile and 95-percentile of the CDF, for Es/IoT of three cells, respective, are listed Table 19, Table 20, and Table 21.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-6, -13) dB |  | RSTD (cell-1, cell-3) (-6, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -38 | 29 | -TBD | 28 |
|  |  |  |  |  |  | 3 | -34 | 32 | -45 | 30 |
|  |  | 132 | 16 | 1 | 4 | 2 | -34 | 30 | -39 | 31 |
|  |  |  |  |  |  | 3 | -31 | 28 | -32 | 27 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -47 | 37 | -TBD | 23 |
|  |  |  |  |  |  | 3 | -39 | 38 | - TBD | 33 |
|  |  | 128 | 8 | 1 | 4 | 2 | -38 | 38 | - TBD | 33 |
|  |  |  |  |  |  | 3 | -39 | 38 | -44 | 38 |

Table 19: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -13) dB |  | RSTD (cell-1, cell-3) (-3, -13) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | -38 | 28 | - TBD | 28 |
|  |  |  |  |  |  | 3 | -34 | 31 | -45 | 29 |
|  |  | 132 | 16 | 1 | 4 | 2 | -34 | 29 | -40 | 30 |
|  |  |  |  |  |  | 3 | -31 | 28 | -32 | 27 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | -49 | 36 | - TBD | 23 |
|  |  |  |  |  |  | 3 | -39 | 38 | - TBD | 32 |
|  |  | 128 | 8 | 1 | 4 | 2 | -38 | 38 | - TBD | 33 |
|  |  |  |  |  |  | 3 | -39 | 38 | -44 | 38 |

Table 20: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | RSTD (cell-1, cell-2) (-3, -6) dB |  | RSTD (cell-1, cell-3) (-3, -6) dB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 5%-tile | 95%-tile | 5%-tile | 95%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | -48 | 48 | -49 | 49 |
|  |  |  |  |  |  | 3 | -51 | 50 | -51 | 50 |
|  |  | 132 | 16 | 1 | 2 | 2 | -51 | 50 | -51 | 51 |
|  |  |  |  |  |  | 3 | -49 | 49 | -49 | 39 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | -53 | 53 | -54 | 52 |
|  |  |  |  |  |  | 3 | -52 | 51 | -51 | 51 |
|  |  | 128 | 8 | 1 | 2 | 2 | -51 | 51 | -51 | 51 |
|  |  |  |  |  |  | 3 | -52 | 52 | -52 | 51 |

Table 21: PRS RSTD error for TDL-C channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

UE Rx-Tx time difference results

The UE Rx-Tx time difference error in Tc for TDL-C channel in FR1, derived from the 90-percentile of the CDF, for Es/IoT of three cells are listed in Table 22, Table 23, and Table 24.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 82 | 86 | TBD |
|  |  |  |  |  |  | 3 | 38 | 38 | 39 |
|  |  | 132 | 16 | 1 | 4 | 2 | 33 | 33 | 38 |
|  |  |  |  |  |  | 3 | 24 | 24 | 24 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 33 | 39 | TBD |
|  |  |  |  |  |  | 3 | 35 | 35 | TBD |
|  |  | 128 | 8 | 1 | 4 | 2 | 29 | 24 | TBD |
|  |  |  |  |  |  | 3 | 50 | 50 | 50 |

Table 22: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-6,-13,-13) dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 4 | 2 | 82 | 86 | TBD |
|  |  |  |  |  |  | 3 | 38 | 38 | 39 |
|  |  | 132 | 16 | 1 | 4 | 2 | 33 | 33 | 38 |
|  |  |  |  |  |  | 3 | 24 | 24 | 24 |
| 120 | 40 | 64 | 16 | 1 | 4 | 2 | 33 | 39 | TBD |
|  |  |  |  |  |  | 3 | 35 | 35 | TBD |
|  |  | 128 | 8 | 1 | 4 | 2 | 29 | 24 | TBD |
|  |  |  |  |  |  | 3 | 50 | 50 | 50 |

Table 23: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-3,-13,-13)dB.

| SCS (kHz) | Carrier frequency (GHz) | PRS BW in RB per PFL | Sampling rate (Tc) | Rep | Number of samples | Number of PFLs | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 90%-tile | 90%-tile | 90%-tile |
| 60 | 40 | 64 | 32 | 1 | 2 | 2 | 83 | 83 | 83 |
|  |  |  |  |  |  | 3 | 33 | 44 | 44 |
|  |  | 132 | 16 | 1 | 2 | 2 | 33 | 33 | 33 |
|  |  |  |  |  |  | 3 | 24 | 25 | 24 |
| 120 | 40 | 64 | 16 | 1 | 2 | 2 | 33 | 33 | 33 |
|  |  |  |  |  |  | 3 | 48 | 48 | 48 |
|  |  | 128 | 8 | 1 | 2 | 2 | 48 | 48 | 48 |
|  |  |  |  |  |  | 3 | 63 | 63 | 63 |

Table 24: UE Rx-Tx error for TDL-C channel, FR2, in Tc for Es/IoT (-3, -6, -6) dB.

Discussion

From the PRS BW aggregated RSTD and UE Rx-Tx time difference simulation results reported above, we observed:

- in general, increasing the PRS BW by aggregating PFLs, improves the accuracy of the positioning measurements.

- accuracy of RSTD measurement increases with the increase in the number of measurement samples (i.e., using 4 measurement samples rather than 2 measurement samples) for TDL channels.

- in general Es/IoT (-6, -13, -13) dB and (-3, -13, -13) dB yields the same results for the current simulation setup. These results imply that Es/IoT (-6, -13, -13) dB and (-3, -13, -13) dB both detect the same first path.

Thus, following observations are made:

PRS BW aggregation improves the measurement accuracy.

The RSTD/UE Rx-Tx time difference measurement accuracy improves with the increase in the number of measurement samples.

Following proposals are made.

To define the performance requirements for PRS BW aggregation, use 4 measurement samples as a baseline.

Conclusion

In this contribution we have provided revised simulation results for PRS RSTD error and also UE Rx-Tx time difference error without RF/BB impairments for PRS BW aggregation based on simulation assumptions in [8].

Following observations are made.

- PRS BW aggregation improves the measurement accuracy.

- The RSTD/UE Rx-Tx time difference measurement accuracy improves with the increase in number of measurement samples.

- Following proposals are made.

- To define the performance requirements for PRS BW aggregation, use 4 measurement samples as a baseline.

- References

| [1] | RP-233382, “Revised WID on Expanded and Improved NR Positioning,” 3GPP TSG RAN#102. |
| --- | --- |
| [2] | T. 38.859, “Study on Expanded and Improved NR Positioning (Release 18),” V18.0.0. |
| [3] | R4-2002287, “Updated link level simulation assumption for RSTD and RSRP,” Huawei. |
| [4] | R4-2211061, “Simulation assumption for PRS-RSRPP,” Huawei. |
| [5] | R4-2321461, “Simulation assumptions for PRS measurement accuracy for PRS/SRS BW aggregation,” Huawei, HiSilicon. |
| [6] | R4-2401874, “Simulation results for PRS Bandwidth Aggregation,” Nokia. |
| [7] | R4-2405886, “Simulation results for PRS Bandwidth Aggregation,” Nokia. |
| [8] | R4-2405673, “Simulation assumptions for PRS measurement accuracy for PRS BW aggregation,” Qualcomm Incorporated. |

[9] R4-2409165, “Simulation results for PRS Bandwidth Aggregation,” Nokia.