3GPP TSG-RAN WG4 Meeting #112bis	R4-2416439

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	RRM core requirements for AI/ML Based Beam Management

Agenda item:	6.17.4.1

Document for:	Discussion

Introduction

The discussions on AI/ML enabled BM use case were continued in RAN4#112 in the scope of Rel-19 WI based. Agreements achieved during this meeting are summarized in the WF [1] and the ad hoc minutes [2].

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion, In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement error impact and the approximated ground truth

- Indicated/activated TCI states requirements

- Impacts on requirements related to candidate beam detection

- KPIs for beam prediction

- LCM related requirements

Generalization, test coverage and testing setup related aspects are discussed in our accompanying paper [3].

Discussion

Measurement error impact and the ground truth

During RAN4#112 meeting, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreement is copied below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

In this section, the impact of measurement error for inference operation is proposed. We evaluate the performance for AI/ML and non-AI/ML methods with and without measurements error, where the RSRP difference between Top-1 measured strongest beam and Top-1 predicted beam are compared. We consider BM-Case1 for Set B is subset of Set A, where Set A and Set B are configured with codebook size 32 beams and 8 beams, respectively. The details of simulation assumptions are provided in TR 38.843 - Table 6.3.1-1.

- Methodology:

The measurement error is defined in terms of the maximum error margin. This is typically specified as a maximum absolute error (in dB) that the measurement can deviate from the actual RSRP. We model the measurement error with a Normal distribution having standard deviation that changes the variability of the measurement error, e.g. ±6dB, ±4dB, ±3dB, ±2dB, such that 95% of the measurement errors fall within the error margin.

For the AI/ML method in the inference phase, Set B is considered with measurement error. On the other hand, the model is trained using RSRP measurements without measurement error at input.

The baseline approach of non-AI/ML method for performance comparisons is as follows. For the non-AI/ML (exhaustive search), the best beam is selected from Set A codebook beams where the measurement of all RS resources or all codebook beams in Set A are considered. When there is no measurement error, non-AI/ML method (exhaustive search) would select the best beam with highest RSRP value. When there is measurement error, non-AI/ML method may select a sub-optimal beam without the highest RSRP value.

In both ML and non-ML cases ΔeRSRP is calculated as follows: ΔeRSRP = eRSRP (with error) – eRSRP_(without error). In this analysis it is key to understand the impact of the measurements error on the prediction. Therefore, we subtract the prediction error obtained with the AI/ML method (without measurement error) from the result of the AI/ML method obtained with measurement error. For non-ML exhaustive search eRSRP (without error) = 0 at any percentile of CDF. Therefore, the ΔeRSRP = eRSRP (with error).

- Results:

In Figure 1, we evaluate the impact of measurement error using the CDF of RSRP difference, i.e. the difference between RSRP value of Top-1 predicted beam ID and the RSRP value of Top-1 measured strongest beam.  The RSRP difference of AI/ML method and RSRP difference of non-AI/ML method are provided and compared. In both AI/ML and non-AI/ML approaches, the selection of Top-1 beam is among the Tx beams (32 codebook beams) and assuming that the Rx beam is corresponding to the highest RSRP values.

The CDF of RSRP difference of non-AI/ML method with measurement error ±6dB is considered as baseline. At 90-th percentile, the RSRP difference between measurement with error (±6dB) and measurement without error is 3.26 dB.

- For AI/ML method, by comparing the black solid line (measurement error with ±6dB) and grey solid line (without measurement error), at 90-th CDF percentile, the RSRP difference between measurement with error and measurement without error (ΔeRSRP ) is 8.3 dB. When considering AI/ML method with measurement error  ±4dB, by comparing the black dash line (measurement error with ±4dB) and grey solid line, at 90-th CDF percentile, the RSRP difference between measurement with error and measurement without error (ΔeRSRP) is 3.68 dB. Similar to measurement error ±3dB, by comparing the black dotted line (measurement error with ±3dB) and grey solid line, at 90-th CDF percentile, the ΔeRSRP is 2.07 dB.  Lastly, for measurement error with sigma ±2dB the ΔeRSRP is 0.98 dB.

- Accordingly, we provide the values of RSRP difference between the measurement with error and measurement without error for AI/ML method at 90-th percentile in Table 1.

Figure 1: Performance comparison between AI/ML and non-AI/ML methods with and without measurement error.

Table 1. RSRP Difference (ΔeRSRP ) at 90-th CDF percentile W and W/O measurement error  [dB]

| Methods | ΔeRSRP |
| --- | --- |
| non-AI/ML (exhaustive search with input error ±6dB) | 3.26 |
| AI/ML method (with input error ±6dB) | 8.3 |
| AI/ML method (with input error ±4dB) | 3.68 |
| AI/ML method (with input error ±3dB) | 2.07 |
| AI/ML method (with input error ±2dB) | 0.98 |

In Table 1, at 90-th percentile of the CDF curves shown in Figure 1, the ΔeRSRP of the AI/ML method with measurement error ±6dB, ±4dB, ±3dB, and ±2dB are 8.3 dB, 3.68 dB, 2.07 dB and 0.98 dB, respectively. While the non-AI/ML method with input error ±6 dB has ΔeRSRP of 3.26 dB.  It can be observed that the ΔeRSRP of AI/ML method with measurement error ±6dB is quite high compared with the baseline (non-AI/ML with measurement error ±6dB).

The AI/ML method is very sensitive to measurement error. The RSRP difference between measurement with error (±6dB) and measurement without error is 5 dB higher than non-ML, suggesting that the performance of the AI/ML method degrades significantly with legacy RSRP measurement error requirement.

To align the AI/ML method's performance with the non-AI/ML approach, it is reasonable to consider reducing the measurement error margins. Specifically, reducing the measurement error to ±4 dB or ±3 dB result in ΔeRSRP values of 3.68 dB and 2.07 dB, respectively, which are more comparable to the non-AI/ML performance.

These observations highlight the need for addressing the sensitivity of the AI/ML method to measurement errors and the importance of discussing the measurement error margins to ensure performance alignment with legacy non-AI/ML beam management.

Given the significant degradation in prediction performance observed with the current ±6 dB measurement error margin, RAN4 shall discuss: 
i) whether such degradation (around 5 dB in simulations) with respect to legacy performance is acceptable or not.

- ii) whether impacts on other performance metrics should be studied or not

To ensure that the AI/ML method performs comparably to the non-AI/ML method, RAN4 shall discuss: 
i) whether a tightening in the legacy measurements’ requirement is feasible/acceptable.

- ii) If there are other ways for reducing the gap between AI/ML and non-AI/ML prediction performance in presence of measurements error

Filtering design for measurements

For an AI/ML functionality, if the input distribution has been changed, e.g., if there is measurement error happening, the AI/ML model might experience measurement error during inference, it leads to low accuracy of prediction output as shown in Figure 1.

One solution for this performance degradation may be tightening of measurement error, as discussed above. However, another possible solution may be to consider filtered L1-RSRP measurements. By applying L1-filtering, it means that we consider an averaging of the measurements over time period instead of relying on instantaneous measurement. The error in measurement may be reduced by applying L1-filtering, since higher number of samples are averaged.

RAN4 should discuss if filtered L1-RSRP measurements can be alternative to improve prediction accuracy and can be used to avoid any tightening in legacy measurement accuracy requirements.

Indicated/activated TCI state requirements

The issue of indicated/activated TCI states requirements was discussed during RAN4#112 meeting. Although, no agreement was reached but initial discussions were held on the options listed below:

- Option 1: Tx beam prediction does not imply that UE knows the best UE Rx beam to receive the predicted beam - TCI state for the predicted beam is unknown unless it has been explicitly measured within 1280ms - Option 2: UE knows the Rx beam paired with the predicted Tx beam - FFS whether the TCI state associated with a predicted Tx beam is known or unknown - Option 3: UE knows the Rx beam paired with the predicted Tx beam - TCI state should be considered known , FFS whether side conditions for known TCI state should be modified (relaxed) for this case - Option 4: others

In this section, we further provide our view on above listed options. In Rel-19 AI/ML BM, the measurements of the Set B beams are used as input to the ML model, which outputs the best K beams (beam indices and/or L1-RSRP values) from Set A beams. For a UE-sided model, Set A and B can be provided with the CSI reporting configurations.

When the UE predicts the best Tx beams out of Set A, while using only Set B Tx beams as input to the AI/ML functionality, and since UE has the information about complete set of beams (i.e. Set A), hence it is reasonable to assume that UE may know already the Rx beam to be used for receiving the predicted Tx beam. Furthermore, without corresponding Rx beams, the prediction for Tx beams may not be correct and may change with a change in corresponding Rx beams. Therefore, among Option 1 and Option 2 above, in our view, Option 2 is the right option.

It is reasonable to assume that UE may know the Rx beam to be used for receiving the predicted Tx beam, therefore Option 2 should be selected.

However, Option 2 only helps in identifying a problem, but it does not provide a solution. Therefore, it is important to consider Option 3 as well. In the following, we highlight the problem where we may have much more unknown TCI states due to the existence of predicted beams in case of AI/ML BM use case in comparison of legacy.

The NW can determine whether any changes are required in the set of active TCI states, and if yes, the NW can adapt the set of active TCI states based on the prediction report received from the UE. Here, the NW needs to ensure that the RS resource corresponding to the activated/indicated beams which can be measured by the UE prior any beam indication takes place corresponding to the predicted RS resource.

In the case of UE uses subset of beam measurements (Set B) to predict beam(s) from Set A, the UE may not have measured the RS resource included in the indicated/activated TCI state. Since the predicted beam(s) is not measured by the UE for longer period, the predicted beam(s) will be corresponding to unknown TCI state. The delay for switching the beam to an unknown TCI state is higher than switching to a known TCI state. Therefore, in our view, for a UE that supports beam prediction, there is a need to relax the known conditions for TCI state, specified in Clause 8.10.2 of TS 38.133:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

If this condition is relaxed, then the predicated beam (which may not be part of Set B beams) corresponding to a Set A/subset of set A, may be measured with a longer delay than 1280 ms and still fulfil TCI known state condition. In other words, a higher value of the delay requirements should be used for known TCI state conditions in Clause 8.10.2 of TS 38.133 for predicted beams.

RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2.

Impacts on requirements related to candidate beam detection

In current RAN4 requirements, as mentioned in Clause 8.5.6.2 of TS 38.133, the UE is supposed to evaluate during a certain time period the L1-RSRP measured on the configured CSI-RS resource in the candidate beam set (called q1) containing CSI-RS resources configured for a serving cell. The objective is to verify if the L1-RSRP measured on the configured CSI-RS resource has not been degraded too much and is still better than a certain threshold (Qin_LR). The certain period used to evaluate L1-RSRP is known as TEvaluate_CBD_CSI-RS and is specified in Table 8.5.6.2-2 of clause 8.5.6.2 in TS 38.133.

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2 Configuration; TEvaluate_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD ´ P ´ N ´ PCBD) ´ TCSI-RS). DRX cycle > 320ms; Ceil(MCBD ´ P ´ N ´ PCBD) ´ TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length..

- However, with the addition of AI/ML BM feature, the UE is supposed to predict best beams from the Set A beams and the UE is configured to measure only a few beams (i.e. Set B). Therefore, the network will often configure only CSI-RS of the Set B beams, therefore the CSI-RS periodicity of Set A beams may be increased. If the NW uses same CSI-RS periodicity also for candidate RS resources, it may lead to a higher value of TCSI-RS used for the calculation of TEvaluate_CBD_CSI-RS. This will eventually lead to a higher value of  TEvaluate_CBD_CSI-RS, which consequently may cause a delayed beam recovery operation and may cause frequent beam failures A higher value of TCSI-RS in Table 8.5.6.2-2 may lead to a higher value of TEvaluate_CBD_CSI-RS, which consequently may be the cause of frequent beam failures.

RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

KPIs for beam prediction

RAN4 studied different test metrics for performance requirements for AI/ML enabled BM use case.  Here is an extract from 3GPP TR 38.843 V18.0.0 [4]:

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics/KPIs are identified and could be considered, - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams", where K >1 and values can be reported. - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

During RAN4#112, although no agreement was reached on this issue but following options were discussed during the meeting:

- Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use Option 1,2,3 depending on use case - Option 4: Neither Option 1, 2, 3 is appropriate, a new metric is needed - Option 5: Consider reliability metric based on relative difference in between the predictions due to change in the conditions/inputs. - consider defining multiple tests under different conditions but with the same outcome (same predicted beam) - Option 6: Other

In this section, we provide our view on all the options discussed in previous meetings.

Option1 (RSRP accuracy):

As mentioned in the TR 3GPP TR 38.843 V18.0.0, RAN4 agreed to consider RSRP accuracy as performance metric/KPI for AI/ML BM use case. However, there is no agreement on the definition of this metric in RAN4. Here, we describe our view on the definition of RSRP accuracy.

If AI/ML functionality outputs L1-RSRP prediction and beam id prediction of Top-K beams, the predicted L1-RSRP accuracy metric can be defined as “The accuracy for the predicted L1-RSRP of Top-K predicted beams should be within a specified margin with respect to the ground truth for corresponding beams. The approximated ground truth can be the measurements reported by the UE for the corresponding beams.”

- Although RAN4 agreed to consider RSRP accuracy as the performance metric, there is no agreement in RAN4 on the formal definition of predicted L1-RSRP accuracy.

RAN4 should define RSRP accuracy metric per beam as, “The accuracy for the predicted L1-RSRP of Top-K predicted beams should be within a specified margin with respect to the ground truth for corresponding beams”.

Option2 (Beam IDs prediction accuracy):

In this option, when AI/ML functionality outputs Top-K beam IDs prediction, RAN4 discussed following metrics:

Top-1(%): This is the success rate, based on a number of occurrences, when Top-1 predicted beam is also the actual Top-1 strongest beam. The number of occurrences required for this metric needs yet to be discussed in RAN4. In our view, this is the strictest requirement, and the test may be failed due to even smallest prediction errors. Therefore, either RAN4 should have a relaxed performance target for this metric, or this metric should not be considered in RAN4.

Top-1(%) metric of Option2 is the strictest requirement and RAN4 should not consider it as a performance metric.

Top-K/1(%): This is the success rate, based on a number of occurrences, when Top-1 actual strongest beam is one of the Top-K predicted beams, where K>1. The number of occurrences required for this metric needs yet to be discussed in RAN4. This metric only guarantees that Top-1 actual strongest beam is one of the predicted beams. It doesn’t verify the accuracy of prediction by the AI/ML functionality for other beams. Therefore, this metric should not be considered alone. Although, one solution may be to use this metric in combination with the Top-1/K(%) metric.

Top-K/1(%) metric of Option2 only verifies if actual strongest beam is among Top-K predicted beams and it doesn’t verify the accuracy of prediction for other beams.

Top-K/1(%) metric of Option 2 should be considered in the combination of Top-1/K(%).

Top-1/K (%) : This is the success rate, based on a number of occurrences, when Top-1 predicted beam is one of the Top-K actual strongest beams, where K>1. The number of occurrences required for this metric needs yet to be discussed in RAN4. This metric only guarantees if the prediction of Top-1 predicted beam has a reasonable accuracy. It doesn’t verify the accuracy of other predictions. Since, network may indicate to the UE to use any of the predicted beams in the field, and if the prediction accuracy of other beams is not verified then it may cause a degraded performance in the field. Therefore, this metric should not be considered alone. Although, one solution may be to use this metric in combination with the Top-K/1(%) metric.

Top-1/K(%) metric of Option2 only verifies if Top-1 predicted beam is among Top-K actual strongest beams and it doesn’t verify the accuracy of other predictions.

Option 3:

Option 3 is defined as, “The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam x dB”.

This metric compares the measured L1-RSRP of the strongest predicted beam with the measured L1-RSRP of the actual strongest beam. In our view, this is a reasonable metric and should be considered by RAN4. However, this metric doesn’t verify the quality of prediction for other predicted beams. Since, network may indicate to the UE to use any of the predicted beams in the field, and if the prediction accuracy of other beams is not verified then it may cause a degraded performance in the field. Therefore, in our view, only Option 3 will not be sufficient, and it must be considered in combination with other Options (such as Option 1 and Option 2).

Option 3 only compares the measured L1-RSRP of the strongest predicted beam with the measured L1-RSRP of the actual strongest beam. It does not verify the quality of prediction for other predicted beams.

RAN4 should consider Option 3 but in combination of other Options (such as Option 1 and Option 2).

Option 5:

We introduced Option 5 as another option, which is to define the requirements not based on UE measurement as a reference, but to verify the expected behaviour of AI/ML-enabled functionality based on the change in the configurations/inputs of the functionality. For example, the best predicted beam should not change, if the power of all measured beams is scaled in the same way or when one of the measured beams cannot be received anymore, etc. In other words, the AI/ML functionality can be tested without explicit definition of the ground truth.

The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Option 5 should be considered in the combination of other approximated ground truth-based options (such as Option 1).

LCM related requirements

LCM related core requirements are further discussed in this section.

RAN2 is currently discussing LCM for the UE-side model for beam management.  One aspect is related to the activation/deactivation of the functionality after inference configuration. We recall that an active functionality can refer to functionalities that the UE starts to use to predict beam via inference.

In RAN4#111, RAN4 reached the following agreement:

Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - Model identification - Selection - Activation - Deactivation - Switching - fallback to non-AI operation - performance monitoring - Others are not precluded

As it is clear from the agreement above that RAN4 agreed to consider defining RRM core requirements for different LCM procedures. In this section, we further analyse how RAN4 may define core requirements for different LCM procedure including activation, deactivation, switching and fall to legacy etc.

Delay components for LCM related operations

The LCM operations for AI/ML BM, e.g., activation, deactivation, and fall-back (e.g., fall-back from AI/ML beam prediction to legacy mode) could be supported by CSI report configuration. These operations can be triggered, for example, based on the performance monitoring output. Reusing the existing capability framework, NW can configure CSI report configurations to enable BM-Case1 or BM-Case2 beam prediction. The CSI reports for both BM-Case1 and BM-Case2 can be based on periodic, semi-persistent, or aperiodic CSI-reporting. The latency of these operations will depend on multiple factors such as,

- Whether CSI configuration (ResourceSet) to be used is already available in the device or not,

- The type of the CSI reporting (periodic, semi-persistent, aperiodic),

- The availability of (previous) measurements that shall be used as an input to the model/functionality inference/prediction,

- Availability of the necessary model(s) in the device,

- The time it takes to substitute one model/functionality with another (the UE may have specialized AI-hardware accelerators to run the inference, which may require loading of the model),

However, it should be clearly specified how much time it takes for the UE to report the new predictions/inference results following the changes indicated by the LCM operation. Without such information NW will not be able to decide when to schedule grants for the reporting of measurements/predictions or data and taking too much time in these operations may also degrade the performance.

In LCM operation, the functionality activation operation is the first step when the UE receives from NW the necessary inference configuration needed for running the inference with the AI/ML functionality available at the UE. Then, other operations, e.g., deactivation, switching and fallback, come after functionality activation operation.

For AI/ML beam prediction reporting enabled by periodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

- delay related to RRC Reconfiguration,

- delay from UE processing time: This may be related to the size of the inference report, model-related pre-processing time or memory limitations on storing models (associated with beam prediction),

- delay related to associated ID: This delay will depend on if the associated ID is used or not used in the past, there might be delay corresponding to configure/indicate the associate ID to UE,

- delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related to RRC Reconfiguration,

  - delay from UE processing time

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by semi-persistent CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via semi-persistent CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from activating semi-persistent CSI resource set and delay for selecting of semipersistent CSI resource set.

  - delay from UE processing time

  - delay related to associated ID

  - delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from NW selecting of semi-persistent CSI resource set.

  - delay from UE processing time related

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by aperiodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related activating aperiodic CSI resource set,

  - delay from UE processing time

  - delay related to associated ID,

  - delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related NW selecting of aperiodic CSI resource set,

  - delay from UE processing time related

  - delay due to CSI-RS measurement.

In summary, the delay components related to LCM operations are provided in Table 2.

Table 2: Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Switching | Fall-back (From AI/ML BM to legacy) |
| --- | --- | --- | --- |
| Periodic CSI-reporting | - Delay from RRC Reconfiguration - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay from RRC Reconfiguration - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay from RRC Reconfiguration - Delay from UE processing - Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | - Delay for activating semi-persistent CSI resource set - Delay for selecting of semi-persistent CSI resource set - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay for selecting of semi-persistent CSI resource set - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay for activating semi-persistent CSI resource set - Delay for selecting of semi-persistent CSI resource set - Delay from UE processing - Delay due to CSI-RS measurement period |
| Aperiodic CSI reporting | - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay from UE processing - Delay due to CSI-RS measurement period |

The delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

RAN4 should consider Table 2 as a baseline to start discussions for LCM operations related RRM core requirements.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to RRM core requirements for AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

- Measurement error impact and the approximated ground truth

- Indicated/activated TCI states requirements

- Impacts on requirements related to candidate beam detection

- KPIs for beam prediction

- LCM related requirements

In the paper, the following Observations and Proposals were made:

Observation 1: The AI/ML method is very sensitive to measurement error. The RSRP difference between measurement with error (±6dB) and measurement without error is 5 dB higher than non-ML, suggesting that the performance of the AI/ML method degrades significantly with legacy RSRP measurement error requirement.

Observation 2: To align the AI/ML method's performance with the non-AI/ML approach, it is reasonable to consider reducing the measurement error margins. Specifically, reducing the measurement error to ±4 dB or ±3 dB result in ΔeRSRP values of 3.68 dB and 2.07 dB, respectively, which are more comparable to the non-AI/ML performance.

Proposal 1: Given the significant degradation in prediction performance observed with the current ±6 dB measurement error margin, RAN4 shall discuss:

i) whether such degradation (around 5 dB in simulations) with respect to legacy performance is acceptable or not.

ii) whether impacts on other performance metrics should be studied or not

Proposal 2: To ensure that the AI/ML method performs comparably to the non-AI/ML method, RAN4 shall discuss:

i) whether a tightening in the legacy measurements’ requirement is feasible/acceptable.

ii) If there are other ways for reducing the gap between AI/ML and non-AI/ML prediction performance in presence of measurements error

Proposal 3: RAN4 should discuss if filtered L1-RSRP measurements can be alternative to improve prediction accuracy and can be used to avoid any tightening in legacy measurement accuracy requirements.

Proposal 4: It is reasonable to assume that UE may know the Rx beam to be used for receiving the predicted Tx beam, therefore Option 2 should be selected.

Proposal 5: RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2.

Proposal 6: RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

Proposal 7: RAN4 should define RSRP accuracy metric per beam as, “The accuracy for the predicted L1-RSRP of Top-K predicted beams should be within a specified margin with respect to the ground truth for corresponding beams”.

Observation 3: Top-1(%) metric of Option2 is the strictest requirement and RAN4 should not consider it as a performance metric.

Observation 4: Top-K/1(%) metric of Option2 only verifies if actual strongest beam is among Top-K predicted beams and it doesn’t verify the accuracy of prediction for other beams.

Proposal 8: Top-K/1(%) metric of Option 2 should be considered in the combination of Top-1/K(%).

Observation 5: Top-1/K(%) metric of Option2 only verifies if Top-1 predicted beam is among Top-K actual strongest beams and it doesn’t verify the accuracy of other predictions.

Observation 6: Option 3 only compares the measured L1-RSRP of the strongest predicted beam with the measured L1-RSRP of the actual strongest beam. It does not verify the quality of prediction for other predicted beams.

Proposal 9: RAN4 should consider Option 3 but in combination of other Options (such as Option 1 and Option 2).

Observation 7: The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

Proposal 10: RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Proposal 11: Option 5 should be considered in the combination of other approximated ground truth-based options (such as Option 1).

Proposal 12: The delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

Proposal 13: RAN4 should consider Table below as a baseline to start discussions for LCM operations related RRM core requirements.

Table: Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Switching | Fall-back (From AI/ML BM to legacy) |
| --- | --- | --- | --- |
| Periodic CSI-reporting | - Delay from RRC Reconfiguration - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay from RRC Reconfiguration - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay from RRC Reconfiguration - Delay from UE processing - Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | - Delay for activating semi-persistent CSI resource set - Delay for selecting of semi-persistent CSI resource set - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay for selecting of semi-persistent CSI resource set - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay for activating semi-persistent CSI resource set - Delay for selecting of semi-persistent CSI resource set - Delay from UE processing - Delay due to CSI-RS measurement period |
| Aperiodic CSI reporting | - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay from UE processing - Delay related to associated ID - Delay due to CSI-RS measurement period | - Delay from UE processing - Delay due to CSI-RS measurement period |

- References

- R4-2414323, WF on AI/ML, Qualcomm, RAN4#112, Maastricht, Netherlands, 19th – 23rd August 2024.

- R4-2414446, AI/ML ad-hoc meeting minutes, Qualcomm, RAN4#112, Maastricht, Netherlands, 19th – 23rd August 2024.

- R4-2416440, On Testability and Interoperability aspects for AI/ML Based Beam Management, Nokia, RAN4#112-bis, Hefei, China, 14th –18th October 2024.

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Rel-18, ver. 18.0.0.