3GPP TSG-RAN WG4 Meeting #112bis	R4-2416448

# Hefei, China, October 14th – 18th, 2024

Source:	Nokia

Title:	Testability and interoperability issues for positioning accuracy enhancement

Agenda item:	6.17.5.2

Document for:	Discussion

Introduction

In RAN#102, the Release-18 study on artificial intelligence (AI)/machine learning (ML) for NR air interface (FS_NR_AIML_Air) [1] was completed [2] and the findings (with agreements and open issues) were documented in the outcome technical report 3GPP TR 38.843 V18.0.0 [3]. Furthermore, a new Release-19 work item on artificial intelligence (AI)/machine learning (ML) for NR air interface (NR_AIML_Air) [4] was also approved in RAN#102 to start the normative work for the general AI/ML framework for air interface and to enable the recommended use cases in the preceding study.

During the SI, as mentioned in the extract from 3GPP TR 38.843 V18.0.0 [3] below, following sub use cases for positioning accuracy enhancement are considered:

The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement More specifically, the following Cases are considered for the study: - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning One-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

During SI, RAN4 studied different test metrics for performance requirements for AI/ML enabled positioning use case.  Here is an extract from 3GPP TR 38.843 V18.0.0 [3] covering these aspects:

Both direct AI/ML positioning and AI/ML assisted positioning are considered. For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above The feasibility and testability of different options should be further justified in WI.

In RAN4#110, it has been agreed on the testability and interoperability issue for AIML positioning [5].

2.3 Testability and interoperability issues for positioning accuracy enhancement 2.3.1 Agreements in ad-hoc session (R4-240xxxx) Issue 3-2: Requirements for case 3a/3b RAN4 will not define positioning accuracy requirements for case 3a/3b Issue 3-6: Requirements for case 2a/2b RAN4 to come back to case 2a/2b based on progress in the other working groups

In RAN4#110bis, it has been agreed on the testability and interoperability issue for AIML positioning [6].

Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme(if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced

In RAN4#111, it has been agreed on the testability and interoperability issue for AIML positioning [7].

Issue 3-2: Requirements for case 2a Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups Issue 3-3: Requirements for case 2b Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups

In RAN4#112, it has been agreed on the testability and interoperability issue for AIML positioning [8].

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

The issues on AI/ML based positioning require further discussion on selected sub use cases. In this paper, the performance requirements for case 1 and case 2b are discussed in the aspects of performance test metric, measurement accuracy requirements, testability, and LCM-related latency requirements. This paper also discusses the requirements for LMF-side model in Case 2b.

Discussion

Requirements for UE-side model: Case1, Case 3a

This section discusses the requirements in the use cases with a UE-side model or gNB-side model. Considering that RAN#105 decided that RAN1 will not focus on 2nd priority cases anymore since RAN1#118bis, we exclude Case 2a here.

Testability aspects and Performance test metric in Case 1

In RAN4#110, we discussed the testability of Case 1 where the test metric in Case 1 can be the positioning accuracy.  Therefore, in this paper, we further discuss the testability aspects of positioning accuracy test metric. To define the feasible requirements for Case 1, it is useful to consider a possible test framework with the four essential steps of identification, setting, running, and evaluation as shown in Figure 1.

Figure 1. Test framework in Case 1

When inference results of AI/ML functionality are the positioning coordinate, the AI/ML functionality inference consisting of the positioning co-ordinates can be verified based on the ground truth. The ground truth may consist of the location point(s) with known positioning co-ordinates, for example, in form of a test area/grid as shown in Figure 2. To know the coordinate of each location point, a PRU (i.e., Positioning Reference Unit location is known to gNB/LMF) is placed, or the position coordinate of each location point is a known reference location derived by, e.g., using GNSS based positioning. In that case, while UE moves from one location point to another location, the inference result with AI/ML functionality at each location is reported to the test equipment and compared to the ground-truth information for verification of positioning accuracy.

Figure 2. Grid map providing a ground-truth information for inference evaluation

The test setup can be implemented for a field test. If testing is performed in a limited indoor space, the same test setup can be scaled down to be fitted into the laboratory space. For instance, the test map granularity may be further reduced to a certain level depending on the achievable precision of an AIML-based positioning technique, and configured bandwidth size, and channel conditions.

Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based).

RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1.

Testability aspects in Case 3a and Case 3b

- Considering RAN#105 decided that RAN1 will focus on 1st priority cases, such as Case 1, Case 3a and Case 3b, testability for Case 3a and Case 3b is subject to be investigated. Based on the earlier RAN4 #110 agreement not to specify positioning accuracy requirements for Case 3a and Case 3b, there are no testability aspects for Case 3a and Case 3b, except if RAN1 specifies new measurement types for AIML-based positioning or enhances existing measurement types. In those cases, testability for Case 3a (gNB-side model) and Case 3b (direct positioning, related to gNB measurement output) should be investigated in greater detail.

Based on the earlier RAN4 #110 agreement not to specify positioning accuracy requirements for Case 3a and Case 3b, there are no testability aspects for Case 3a and Case 3b, except if RAN1 specifies new measurement types for AIML-based positioning or enhances existing measurement types for AIML-based positioning.

RAN4 to consider testability aspects for Case 3a and Case 3b only, if RAN1 specifies new measurement types for AIML-based positioning or enhances existing measurement types for AIML-based positioning.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML based positioning. In the paper, the following observations and proposals were made:

- Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based).

- RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1.

- Based on the earlier RAN4 #110 agreement not to specify positioning accuracy requirements for Case 3a and Case 3b, there are no testability aspects for Case 3a and Case 3b, except if RAN1 specifies new measurement types for AIML-based positioning or enhances existing measurement types for AIML-based positioning.

RAN4 to consider testability aspects for Case 3a and Case 3b only, if RAN1 specifies new measurement types for AIML-based positioning or enhances existing measurement types for AIML-based positioning.

- References

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96, Budapest, Hungary, June 06 – June 09, 2022.

- RP-233132, Status report for SI on AI/ML for NR air interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110bis, Changsha, China, April 15 - 19, 2024.

- R4-2403712, WF on AI/ML, Qualcomm, RAN4#110, Athens, Greece, February 26 – March 1, 2024.

- R4-2410570, WF on NR ALML air interface, Qualcomm, RAN4#111, Fukuoka, Japan, May 20 – 24, 2024.

- R4-2404323, WF on NR ALML air interface, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug. 19 – 23, 2024.