3GPP TSG-RAN WG4 Meeting #112bis	R4-2416449

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On AIML Mobility Scenarios Evaluation and RRM Requirements Impacts

Agenda item:	6.18.2

Document for:	Discussion

Introduction

RAN4 discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) starts from RAN4#112bis. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

During this meeting, it is expected the following aspects to be discussed.

- Work Plan for RAN4 Discussions on AI/ML for Mobility in NR

- AI/ML Mobility Scenarios Evaluation and RRM Requirements

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution, we provide our views and discuss different aspects of AI/ML Mobility Scenarios, Evaluation and RRM Requirements based on the RAN2 framework.

Discussion

Based on the current RAN2 agreements, following scenarios and use cases have been initially considered for AI/ML in mobility ‎[2]:

- Measurement Prediction,

- Handover and Radio Link Failure Prediction, and

- Measurement Event Prediction.

However, to ensure a clearer understanding and appropriate definition of RRM requirements, RAN4 must wait for further progress from RAN2 as well as to agree on the RAN4 work plan. RAN2 has begun its study by discussing simulation assumptions and the RRM measurement prediction use case, making more progress on this compared to the other scenarios. Additionally, RAN4’s RRM requirements for measurement prediction can serve as a baseline for the other scenarios.

RAN2 has initiated its work by focusing on measurement prediction, leading to more advanced progress in this area compared to other use cases.

Therefore, we believe that RAN4 should start with the RRM measurement prediction use case and related scenarios, and, as RAN2 advances, proceed with measurement event prediction and Handover/Radio Link Failure Prediction. Our focus is thus on measurement prediction, and we offer our perspectives on the different RRM requirements and key KPIs.

RRM Measurement Prediction

In RAN2#125bis the following sub-use cases were agreed for RRM measurement prediction:

- Sub-case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

In the draft of TR 38.744 [4], the two main identified study goals are:

- First Goal; Measurement reduction. Achieve the same mobility performance, but with reduced DL radio measurements that the UE needs to take and report, which improves UE energy efficiency, reduces the required signalling, and network overhead of transmitting reference signals (RSs).

- Second Goal; Mobility optimization. Predict future UE radio measurements and use them to optimize HO performance and mobility decisions.

Three domains (or a combination them) have been identified for mobility predictions:

- Spatial: when UE is not measuring every cell and/or beam

- Frequency domain: The UE is not measuring every frequency carrier/bands

- Temporal: when UE may skip and predict some of the measurement time steps

The primary focus of the spatial domain prediction is making a decision about a beam/cell based on the measurement of another cell/beam. Based on the current RAN2 framework, spatial domain prediction is used for FR2 intra-cell intra-frequency scenario targeting to reduce the measurement effort.

In the case of frequency domain prediction, the UE measures a specific frequency carrier or band and predicts the measurements for another carrier or band. This approach helps minimize the number of measurements and reduces the required measurement gap, thereby decreasing communication interruptions.

Two variants of temporal domain prediction are considered in RAN2, namely case A and case B.

As exampled in Figure 2, in RRM temporal prediction case A, there is no measurement reduction within observation window (OW). When sliding forward, to produce new predictions, the UE measures at least part of the samples of the previous predicted window (PW). There is no measurement samples reduction in Case A and it is used for achieving the second goal (i.e., mobility optimization such as handover prediction).

Figure 2: Illustration of temporal domain prediction – case A

Two examples of temporal domain case B are provided in Figure 3. Case B design target is for the first goal of measured reduction, where the measurement of the previously predicted samples in previous PW is skipped (either by sliding window or applying sample and hold method) for the new predicted window, enabling measurement reduction (e.g., 50% Figure 3).

Figure 3: Illustration of temporal domain prediction – case B

It is expected that the performance of different approaches (spatial, temporal case A, temporal case B) to be dependent on several factors such as input samples, reductions patterns, predicted window length, etc. Some of the factors need standardization and some need RAN2 signaling for coordination between UE and the network.

With regard to scenario prioritization, the following prioritization was agreed upon during RAN2#126, as outlined in Table 1:

Table 1: Prioritization of evaluation scenarios (Table 5.2.1-1 in TR 38.744 draft)

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

RAN4 initially focus on conducting a detailed analysis of the key RAN2 scenarios #2, #3, #4, and #6 (i.e. FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, frequency domain prediction, FR2 to FR2 intra-frequency temporal domain case A, and FR2 to FR2 intra-frequency spatial domain).

AI/ML Measurement Prediction Based Measurement Reduction (UE-Side, Study goal 1)

With regards to scenario#2 (i.e. FR1 to FR1 intra-frequency temporal domain case B), the goal is to reduce measurement overhead by reducing the number of measured samples. To avoid additional complexity, i.e., in the design of new reporting mechanisms, in the 1st goal of measurement reduction, the network is expected to use the predicted measurements reported by a UE, or measurements derived from predicted and measured values, as it uses the current measurements reports. As shown in Figure 3, the reported predicted measurements or measurements derived from predictions may include:

- Beam-level layer-1 filtered measurements (A1)

- Cell-level layer-3 filtered measurements (C)

- Beam-level layer-3 filtered measurements (E)

Figure 3: 3GPP TS 38.300 Measurement Model

For predicted measurement, one of the key issues which needs to be investigated is predicted measurement accuracy. Based on the current RAN2 progress on AI/ML for Mobility, for measurement prediction accuracy Mean Absolute Error (MAE) which is the average difference between the predicted values and the actual values (e.g., predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement) has been agreed to be used as the performance metric.

However, in RAN4 AI/ML for Air Interface, single or a combination of options listed in Table 3 are under investigation for accuracy.

Table 3: RAN4 metric options for beam management in AI/ML Air Interface WI

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

For measurement prediction, different accuracy metrics are considered in RAN2 SI for AI/ML mobility and in RAN4 for AI/ML Air Interface WI for beam prediction.

In TS 38.133, RAN4 has defined measurement accuracy as a fixed limit (e.g. +/-X dB). As an example, absolute SS-RSRP, and relative SS-RSRP accuracy requirements for intra-frequency FR1 band in some scenarios are provided in Table 2. Though, the specified measurement accuracy requirements are expressed as a fixed limit (e.g. +/-X dB), during the tests assuming a Gaussian distribution of the error, the limits will have to be set at +/-3.29 if the probability of failing a "good device under test" in a single test is to be kept at 0.1% (i.e., 99.9% of the measurements based on the assumed Gaussian distribution of errors) will fall within the specified range.

Table 2: Examples of intra frequency SS-RSRP Intra frequency absolute and relative accuracy in requirements in FR1[38.133]

|  | Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
|  | dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| Absolute Accuracy |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  | 4.5 | 9 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
| Relative Accuracy |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
|  | 2 | 3 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |

Requirements for the accuracy of predicted measurements may follow the requirements for baseline measurements, but RAN4 should study if there are potentially new or updated requirements when AI/ML-based predicted measurements are reported for measurement reduction purpose.

RAN4 will need to study accuracy requirements for AI/ML-based predicted beam level L1/L3-filtered or cell-level L3-filtered measurements for measurement reduction purpose.

Additionally, as discussed, RAN4 may have different requirement KPIs (metrics) than RAN2, e.g., because of the availability of the ground truth/reference values and their accuracy in the tests. Evaluation of new requirements can be associated with a need to perform new simulations, such as errors/accuracy of the inputs/measurements, latencies of the procedures, new metrics, etc. RAN2 has progressed in defining simulation assumptions and setup for the RRM measurement prediction and RLF prediction. To ensure that these simulations accurately reflect RAN4 RRM requirements and KPIs, it is important to cross-check them with the RAN4 requirements.

To ensure compatibility, RAN2 simulation assumptions must align with the RRM constraints outlined by RAN4.

RAN4 to check and compare RAN2 simulation assumptions to usual RAN4 RRM requirements/test configurations to conclude about their compatibility

Scenario#3 (i.e., FR1 to FR1 inter-frequency in frequency domain) focuses on an additional use case in the inter-cell mobility measurement reduction which is inter-frequency measurement prediction. In this case, the set of measured or predicted cells or beams will include cell or beams from different frequency layers. It can be used to reduce the required inter-frequency measurements a UE needs to take, which in turn can reduce the number of measurement gaps that the UE needs to be configured with. Reducing measurements gaps can improve the data throughput and reduce the latency. The required accuracy of the inter-frequency measurement prediction needs to be studied based on how the predictions are used. Additionally, RAN4 to study any impacts due to coordinating the measurement gaps between the network and the UE.

Frequency domain measurement prediction may need different accuracy and measurement gap requirements compared to temporal measurement prediction.

RAN4 to study the required RSRP accuracy of inter-frequency (frequency domain) measurement prediction, and the impact of frequency domain prediction on measurement gap.

Moreover, scenarios#2 and #3 use FR1 as operational band. Though there is an ongoing discussion in RAN4 AI/ML for Air Interface for measurement prediction, there the target bands are in FR2. RRM requirements for AI/ML predicted measurement in FR1 and FR2 differ due to the distinct physical and operational characteristics of each frequency range which leads to different AI/ML system design, performance evaluation, test cases and KPIs.

RAN4 AI/ML for Air Interface focus on FR2 use cases. RAN4 will need to study and evaluate RRM requirements for RAN2 prioritized FR1 AI/ML mobility scenarios.

Impact of measurement error on AI/ML for Mobility

In mobility scenarios, errors in measurements which may be used as the input or ground truth for AI/ML systems is unavoidable, this can significantly impact network performance by leading to inaccurate predictions that may affect mobility management, handover optimization, and radio resource management (RRM).

Performance of AI/ML-based measurement prediction is highly dependent on the accuracy of the input data.

RAN4 to study the impact of measurement error used as the input data on the accuracy of the predicted measurements.

Time Domain Measurement Prediction for Mobility Optimization- Case A (UE-Side, study goal 2)

The other aspect of measurement prediction is the enhancement of mobility/HO performance in the case when the time/spatial density of measurements is kept the same, i.e., there is not measurement reduction. In this use-case, future measurements predicted with AI/ML are used to optimize mobility decisions. The measurements may be predicted by the UE and either reported to the network or used in the UE to derive further predictions, for example predicting handover or radio link failures or measurement events. The exact use case and decisions, where the predicted measurements are used to optimize mobility performance, and their signaling impact are still subject to further study in RAN2. However, ultimately, the interplay of achievable prediction accuracy and mobility performance would define how RAN4 requirements will be formulated because the main goal is to enhance the robustness of mobility procedures and to avoid failures.

Depending on the use cases considered by RAN2, RAN4 may study requirements for reported predicted future measurements or for other predictions derived from those measurements with the goal to optimize mobility performance/HO without reduction of measurements.

Life Cycle Management (LCM) Related Requirements

As AI/ML models are integrated with critical network functions, such as mobility and handover management as well as radio resource allocation, it is essential to ensure that these models operate effectively in real time under varying network conditions and the dynamic nature of wireless channels. This applies to both UE-side and network-side models. To maintain the effectiveness of AI/ML models, it is necessary to define and monitor key performance indicators (KPIs), including prediction accuracy and latency, which reflect the real-time performance of these models.

When the performance of an AI/ML model falls below predefined thresholds, it is crucial for the network or UE to adapt by updating the model, modifying its configuration, or adjusting the model inputs. In cases of significant performance degradation, it may be necessary to deactivate the model and revert to legacy operations, such as traditional measurement and reporting mechanisms, to ensure reliable network functioning. This adaptive approach ensures that the AI/ML integration does not compromise network performance during periods of model failure or suboptimal operation.

RAN4 will need to study LCM required actions and the impact on RRM.

Radio Link Failure Prediction

Handover and radio link failure prediction was agreed to be studied for UE-side AI/ML. In RAN2#126 the following agreements were made:

- Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR). - Study Direct: Directly RLF prediction by AI/ML models. - FR2 study will be prioritized for RLF prediction - The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction. - HOF prediction is down prioritized in our study. NO simulations/evaluations should be done/submitted - RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case. FFS on expected RLF time and indirect case.

Additionally, further progress on the RLF evaluation was achieved at RAN2#127:

- Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using - Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models. - Output for direct: probability of RLF within an window - Companies should report the prediction time window they have used in the simulations

Depending on the RAN2 study outcome and proposed use cases using the predicted RLFs, RAN4 will need to study requirements for the reported RLF predictions, e.g., the required accuracy and interoperability of those. RLF prediction might also have an impact on the interruption time associated with re-establishment procedures.

Depending on the RAN2 study outcome and proposed reporting formats and utilization of the predicted RLFs, RAN4 will need to study requirements for the reported RLF predictions, e.g., accuracy and interoperability of reports and latency of related procedures.

Measurement Event Prediction

Predicted measurement events may be used to optimize mobility performance by more pro-active reaction to handover situations. Depending on how the predicted measurement events are used, different accuracy is required for the predictions. RAN4 will need to study the required accuracy metrics and required accuracy in different measurement event prediction scenarios.

RAN4 will need to study the required accuracy metrics and core requirements for measurement event prediction depending on the progress of this use-case in RAN2.

Conclusion

In this paper we overview the Rel-19 AI/ML mobility scenario and RRM requirements to be studied in RAN4 working group.

The following Observations and Proposals were made:

Observation 1: RAN2 has initiated its work by focusing on measurement prediction, leading to more advanced progress in this area compared to other use cases.

Proposal 1: RAN4 initially focus on conducting a detailed analysis of the key RAN2 scenarios #2, #3, #4, and #6 (i.e. FR1 to FR1 intra-frequency temporal domain case B, FR1 to FR1 inter-frequency, frequency domain prediction, FR2 to FR2 intra-frequency temporal domain case A, and FR2 to FR2 intra-frequency spatial domain).

Observation 2: For measurement prediction, different accuracy metrics are considered in RAN2 SI for AI/ML mobility and in RAN4 for AI/ML Air Interface WI for beam prediction.

Proposal 2: RAN4 will need to study accuracy requirements for AI/ML-based predicted beam level L1/L3-filtered or cell-level L3-filtered measurements for measurement reduction purpose.

Observation 3: To ensure compatibility, RAN2 simulation assumptions must align with the RRM constraints outlined by RAN4.

Proposal 3: RAN4 to check and compare RAN2 simulation assumptions to usual RAN4 RRM requirements/test configurations to conclude about their compatibility

Observation 4: Frequency domain measurement prediction may need different accuracy and measurement gap requirements compared to temporal measurement prediction.

Proposal 4: RAN4 to study the required RSRP accuracy of inter-frequency (frequency domain) measurement prediction, and the impact of frequency domain prediction on measurement gap.

Observation 5: RAN4 AI/ML for Air Interface focus on FR2 use cases. RAN4 will need to study and evaluate RRM requirements for RAN2 prioritized FR1 AI/ML mobility scenarios.

Observation 6: Performance of AI/ML-based measurement prediction is highly dependent on the accuracy of the input data.

Proposal 5: RAN4 to study the impact of measurement error used as the input data on the accuracy of the predicted measurements.

Observation 7: Depending on the use cases considered by RAN2, RAN4 may study requirements for reported predicted future measurements or for other predictions derived from those measurements with the goal to optimize mobility performance/HO without reduction of measurements.

Proposal 6: RAN4 will need to study LCM required actions and the impact on RRM.

Observation 9: RAN4 will need to study the required accuracy metrics and core requirements for measurement event prediction depending on the progress of this use-case in RAN2.

- References

- RP-242393, Revised SID on AIML for mobility in NR , OPPO, 3GPP TSG RAN Plenary #105, Melbourne, Australia, September 9-12, 2024.

- R2-2406309, Text proposal of 38.744, OPPO, RAN2#127, Maastricht.