3GPP TSG-RAN WG4 #112bis			                       R4-2416451

Hefei, China, 14th – 18th October 2024

Source:	Keysight Technologies UK Ltd.

Title:	On Rel-19 spatial channel model for demodulation performance requirements

Agenda item:	6.14.2

Document for:	Discussion

# 1. Introduction

In RAN#104 a new study item on spatial channel model for demodulation performance requirements was approved ([1]).

The main goal of this study item is to study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements for both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1:

- Including the limitations of the channel models used up to Rel-18 regarding UE MIMO performance

- Considering both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches (using the tuned repeatable spatial channel model of TR38.827 as the starting point for CDL approach).

- Evaluating test methodology feasibility and achievable results uncertainty while test complexity is not significantly increased.

In RAN4#112, the agreed way forward for this study item was agreed in [2], including the work plan with the following objectives for RAN4#112bis meeting:

Several alternatives were discussed for the channel modelling needed in this study item:

- Option 1: RAN4 to consider the option of multi-cluster TX-RX beam steering with TDL model for spatial channel modelling in R4-2412328 [3](Mediatek)

- Option 2: Derive TDL antenna correlation per tap from angular information defined in CDL parameter table as noted in R4-2412762 [4](Huawei)

- Option 3: CDL channel model based in TR 38.827[6], especially CDLC Uma as in R4-24113000 [5](Nokia)

Agreed way forward for TDL based methodologies documented in [2] is that interested companies should review and provide views at next meeting.

This document provides comparison of the 3 different channel modelling alternatives proposed for Re-19 demodulation performance requirements and provides a proposal on how to move forward.

# 2. Comparison among the different channel model approaches proposed

Two different channel model approaches in 3GPP context are named as TDL and CDL. In the current discussion they are considered for specifying MIMO channel models and generating realizations of wideband MIMO radio channels. In TDL models the underlying MIMO modelling method is the correlation matrix-based modelling, while CDL models rely on the sum-of-sinusoids method. It is known that both methods can in principle yield channel realizations with equal statistics as far as the models are adequately parameterized. In that sense, one approach is not better than the other.

Parameterizations are different and often it is understood that a smaller number of parameters is needed for TDL models. This expectation might not be valid in the case of large MIMO arrays, especially if complex correlations and different correlation matrices for each tap are used, as suggested in [4]. Moreover, it is commonly assumed that CDL models are more difficult and complex to implement. This might be true to some extent, but sum-of-sinusoid models have been specified, implemented and used in 3GPP for more than 20 years, since the introduction of prestigious SCM in TR 25.996 [7] in Rel-6.

There are at least three main differences between the TDL and CDL approaches:

- Firstly, CDL approach has inherent consistency across spatial and temporal characteristics of the channel. Namely, in CDL both fading and Doppler power spectrum are determined by the angular distribution of path components (rays) and beam patterns realized by antenna array geometries and the pre-coding. On the other hand, in TDL approach the MIMO aspects and Doppler are independent and must/can be specified separately. The CDL approach is in this sense more consistent, realistic and physical. The TDL upgrade of [3] does not affect this, while the geometry-based parameterization suggested in  [4] can be extended to incorporate such consistency.

- Secondly, CDL approach has inherent consistency across spatial and frequency domain characteristics of the channel. In CDL models the pre-coding affects the resulting power delay profiles and, consequently, the frequency selective fading per beam. These domains are disconnected in TDL models which have same correlation matrix (e.g., low, medium, or high) for all taps. This means that the applied antenna radiation patterns or MIMO pre-coding/beamforming does not impact the frequency-domain fading characteristics in TDL, whereas in CDL the frequency-domain fading characteristics are antenna and beamforming dependent.  The CDL approach is in this sense more consistent, realistic and physical. If different correlation matrices per tap are specified, as proposed by [4], then the resulting PDP and frequency selective fading will depend on the applied pre-coding. The TDL upgrade of [3] does not help in this aspect since it keeps same PDP for all beams directions.

- Thirdly, CDL approach requires specification of involved antenna arrays, since the method relies on angular and polarimetric parameters of both propagation channel and antenna arrays. However, both TDL enhancements suggested in [3] and [4] require also definition of array geometries. The joint impact of antenna arrays and propagation channel parameters on, e.g., MIMO correlation matrices can be computed and analysed for CDL models without any simulations and trial-and-error.

Additionally and regarding the comparison table between CDL and TDL with Multi-cluster Tx-RX-BS in [3] Table 2, Keysight has a different view for the CDL aspects analysed in that table as per comments in italic in table 1 below:

Table 1: Comparison of advanced channel modelling approaches

|  | CDL | TDL with Multi-cluster TX-RX-BS |
| --- | --- | --- |
| RAN4 scope | Completely new for RAN4 tests RAN4 has specified CDL models for OTA testing in TR 38.827. | Combines and extends existing RAN4 models |
| Spatial properties | Properties are pre-defined and fixed Properties can be selected as desired (for both SU-MIMO and MU-MIMO) if not using pre-defined CDL parameter tables but only the CDL framework. In TR 38.901 the original CDL model parameter tables are generated by drawing random parameters as specified by the base-line model. | Properties can be selected as desired |
| Test creation | Trial-and-error to match test with channel model No need for trial-and-error. Just choose parameters (cluster powers and angles) similarly using same principles as in the model proposal [3]. | Choose channel properties to match tested feature |
| Performance alignment | Results difficult to align between companies Should be perfectly comparable by using the fixed randomness definitions of OTA CDL models. Such alignment has been performed in RAN4 between interested parties when specifying CDL models for OTA testing. | Should be easy to align between companies |
| Implementation | Complex to implement Not really. Similar channel model approach has been in use in 3GPP community for more than 20 years since specification of TR 25.996. Very large correlation matrices for TDL models can be complex to determine and use. | Simple to implement on top of TDL |
| AoD/AoA time-domain variation | Time-domain variation not supported Angles and other parameters can be made time variant as has been already specified in CTIA and is currently discussed in RAN4 for dynamic OTA testing. | Time-domain variation of cluster directions can be specified similarly as in existing BS functionality |

MU-MIMO and related interference can be modelled by the CDL approach in a straightforward manner by parameterizing UE angles properly. This can be done also for both of the proposed TDL model upgrades of [3] and [4]. For the basic TDL model is not suitable for MU-MIMO. Same real valued correlation matrix per tap (e.g., low, medium, high) without phasing cannot support different UE angles.

As a summary it can be said that the CDL model approach is good and the most future-proof approach for demodulation performance evaluation needs. However, both the multi-cluster TDL extension proposed in [3] and the per tap geometric determination of correlation matrices for TDL models suggested in [4] are adequate alternatives if beamforming dependent fading characteristics are not considered relevant.

Observation 1: CDL model approach is good and the most future-proof approach for demodulation performance evaluation needs. However, both the multi-cluster TDL extension proposed in [3] and the per tap geometric determination of correlation matrices for TDL models suggested in [4] are adequate alternatives, though they do not preserve the consistency between the angular domain beamforming, and temporal and frequency selective fading.

Hence, selection of the appropriate channel modelling approach depends on whether emphasizing the simpler implementation of TDL model upgrades of [3] and [4] or the consistency in angle, time and frequency domain and the future proof characteristics of CDL model suggested in [5].

Observation 2: Selection of the appropriate channel modelling approach should depend on whether emphasizing the easiness of model implementation or the consistency and future proof characteristics of the approach.

# 3. Conclusion

This document provides comparison of the 3 different channel modelling alternatives proposed for Re-19 demodulation performance requirements:

- Option 1: RAN4 to consider the option of multi-cluster TX-RX beam steering with TDL model for spatial channel modelling in R4-2412328 (Mediatek)

- Option 2: Derive TDL antenna correlation per tap from angular information defined in CDL parameter table as noted in R4-2412762 (Huawei)

- Option 3: CDL channel model based in TR 38.827, especially CDLC Uma in R4-24113000 (Nokia)

The following observations were made:

Observation 1: CDL model approach is good and the most future-proof approach for demodulation performance evaluation needs. However, both the multi-cluster TDL extension proposed in [3] and the per tap geometric determination of correlation matrices for TDL models suggested in [4] are adequate alternatives, though they do not preserve the consistency between the angular domain beamforming, and temporal and frequency selective fading.

Observation 2: Selection of the appropriate channel modelling approach should depend on whether emphasizing the easiness of model implementation or the consistency and future proof characteristics of the approach.

References

- RP-241610, “New SID: Study on spatial channel model for demodulation performance requirements”, Nokia, RAN#104, June 2024.

- R4-2413606, “Way Forward for [112][327] FS_NR_demod_SCM, Moderator” (Nokia), 3GPP TSG-RAN WG4 Meeting #112, August 2024

- R4-2412328, “Discussion on Spatial Channel Modelling”, MediaTek Inc., 3GPP TSG-RAN WG4 # 112, August 2024

- R4-2412762, “Views on spatial channel model”, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting # 112, August 2024

- R4-2411300, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, 3GPP TSG-RAN WG4 Meeting #112, August 2024

- 3GPP TR 38.827, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”, Rel-16

- 3GPP TR 25.996, “Spatial channel model for Multiple Input Multiple Output (MIMO) simulations”