# 3GPP TSG-RAN4 Meeting #112bis	R4-2416457

# Hefei, China, October 14 – 18, 2024

Agenda item:		8.2

Source: 		Qualcomm Incorporated

Title: 		Support for HPUE with DL CA with single UL CC configured

Document for:		Approval

## 1	Introduction

As of today, RAN4 specifications explicitly indicate when HPUE is applicable for a band, as well as when HPUE is applicable to be used in CA configuration.

UEs having HPUE(=>26dBm) hardware support need to reduce power back to PC3(23dBm) in CA operation in case the HPUE basket work has not been completed. This may happen in DL CA with single UL carrier when simply configuring a second DL carrier can result in an output power drop of at least 3dB. This is illustrated in Figure 1.

Figure 1: In case HPUE is not explicitly specified for CA, UE needs to reduce output power and leave it’s HW capabilities unused.

## 2	Discussion

Given that the issue exists already in closed releases, the best approach is to give an allowance for UE to exceed CA power class, and allow them to indicate higher output power up to the level supported for single carrier operation. When formulated as an allowance UE can take full advantage of the higher output power capability, but also, the divergence from existing RAN4 HPUE introduction as well as regular UE power class indication is minimized this way.

Network needs to be made aware of UE power capabilities. Existing UE capabilities PowerClass and ue-PowerClassPerBandPerBC can be used to indicate maximum power for each individual band in a CA configuration. Therefore, UE can use either of these capabilities to unambiguously inform network of the used maximum output power without impact to single carrier (capability ue-PowerClass).

Proposal 1: Allow UE to maintain HPUE maximum transmit power capability in inter-band and intra-band DL CA with single configured UL carrier even when HPUE requirements are not yet completed for the CA configuration.

The pre-condition for indicating support for higher power class is that single carrier requirements are specified. The same applies in case UL-MIMO is supported. This avoids situations where e.g. A-MPR or single carrier Refsens requirements would be missing.

Proposal 2: For inter-band CA and intra-band CA with single configured UL carrier

- UE can indicate any power class specified for single carrier for the UL band.

- If UL MIMO is supported, UE can indicate any power class specified for MIMO with single carrier.

One of the debated aspects in RAN4 have been the relationship of this allowance and MSD specifications, i.e. potential UE receiver desensitization due to higher output power. MSD requirements are based on an artificial worst case reference point to set minimum UE RF performance based on a specific channel bandwidth, center frequency and RB allocation, while in field operation different parameters can be used. It is rare that such worst case spectrum allocation is held by an operator and on top of that worst case scheduling occurs. The specifications should not penalize the performance of all the CA combinations due to some corner cases.

If there is still a concern on DL performance, network has means to limit the maximum output power used by configuring Pmax on the CC transmitting UL

Therefore, it is sufficient to maintain the existing MSD requirements as a reference point and specify that if HPUE requirements are not yet defined for the higher power classes, MSD shall be verified at default power class power level (e.g PC3).

Proposal 3: If HPUE requirements for single UL with DL CA are not specified, MSD is verified at default power class output power level.

There are also on-going discussion on how to handle the HPUE MSD framework overall, the options in most recent WF [1] lists options as follows

  - PC2/PC1.5 MSD requirements are not specified. UE shall meet the Rx requirements of default power class, with default power class transmission

  - Using some equations or look-up tables or other means to derive PC2/PC1.5 MSD from respective PC3 MSD

  - Other solutions are not precluded

    - NOTE: RAN4 continues normal procedure specifying MSD for all PC’s band combination supports until down-selection is done

Some concerns were raised in previous meeting on allowing HPUE operation when there is no certainty on MSD levels for the higher power classes. The first two options are free of this risk, as there either would be no PC2 or PC1.5 MSD at all, or there would be means to derive the MSD directly from PC3 MSD-value. Third option or keeping the current MSD framework involves the risk that HPUE MSD is lower than expected.

To avoid this risk, in case neither of the first two options is adopted in UE RF enhancements WI, limiting testing to only PC3 MSD for all DL CA combinations with single UL can be considered. This would mean that even if HPUE MSD is introduced later, testing would still be limited to PC3 only.

Proposal 4: RAN4 shall consider not testing HPUE MSD for 1 UL with DL CA even if MSD for higher power classes is introduced later

## 3	Conclusion

Based on current RAN4 specifications UEs are not allowed to utilize their real maximum output power capability but instead need to artificially reduce output power back to default power class even when only single UL carrier with DL CA is used. This results in reduced maximum output power and therefore lost coverage and UL performance. Following proposals are made to allow UEs to take advantage of their higher output power capability even if the 3GPP process for the CA configuration is not yet fully completed.

Proposal 1: Allow UE to maintain HPUE maximum transmit power capability in inter-band and intra-band DL CA with single configured UL carrier even when HPUE requirements are not yet completed for the CA configuration.

Proposal 2: For inter-band CA and intra-band CA with single configured UL carrier

- UE can indicate any power class specified for single carrier for the UL band.

- If UL MIMO is supported, UE can indicate any power class specified for MIMO with single carrier.

Proposal 3: If HPUE requirements for single UL with DL CA are not specified, MSD is verified at default power class output power level.

Proposal 4: RAN4 shall consider not testing HPUE MSD for 1 UL with DL CA even if MSD for higher power classes is introduced later

Changes to RAN4 specifications are provided in companion CR.

## 4	References

[1]	R4-2414277, WF on HPUE for CA in TN, Samsung