3GPP TSG-RAN WG4 Meeting #112bis	R4-2416458

# Hefei, China, 14 – 18 October 2024

Agenda item:		5.20.2

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for PC2 and 40 MHz

Document for:		Approval

## 1	Introduction

Requirements for PC2 for n28 were discussed lengthily in rel-18, but eventually postponed to rel-19. New WI including also 40 MHz CBW was approved in [1]. This document discussed UE RF requirements, many of which can be leveraged from rel-18 work, as well as new requirements for 40 MHz CBW.

## 2	Discussion

According to the WID there could be changes for up to and including 30 MHz CBW for PC3 only if any new regulatory requirements come up. Therefore, PC3 requirements are not discussed further.

For PC2, a lot of evaluations for both RSD and NS_17 and NS_18 A-MPR were done in rel-18. This work can now be taken use of. There was discussion in rel-18 on whether traditional split duplexer or full band duplexer should be assumed. Now when 40 MHz channel bandwidth is included in the scope, it is clear that traditional 2x30 MHz split duplexer cannot be the only architecture option. It would be possible to use a wider filter for the sub-band containing 40 MHz CBW, or use a full band filter.

From RAN4 specification point of view, it is better to avoid tight linkage to final implemented filter solution. It is possible to apply the same A-MPR and RSD values independent of implemented solution. For example, if some implementation needs less or no A-MPR, it can just apply smaller value even if specification allows larger A-MPR.

Proposal 1: Specify a single set of A-MPR and RSD values independent of implemented filter solution.

NS_17 A-MPR

We propose to adopt the A-MPR for NS_17 as shown in Table 3 and Table 4 for PC2 operation.

Proposal 2: Adopt the A-MPR for NS_17 as in Table 1 and Table 2 for n28 PC2

Importantly, the proposed values are slightly different from values present in agreed WF in [2], as the tentative values in some case had more than 3 dB increase from general MPR to PC2 A-MPR. No A-MPR is allowed for NS_17 for PC3, so therefore difference to general MPR is considered.

Table 1: A-MPR regions for NS_17 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 723 ≤ Fc ≤ 728 | ≤ 0.18 | ≤ 1.44 | A1 |
|  |  | ≥ 0 | > 5.4 | A2 |

Table 2: A-MPR for NS_17 for PC2

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 3 | ≤ 3.5 |
|  | QPSK | ≤ 3 | ≤ 4 |
|  | 16 QAM | ≤ 4 | ≤ 4.5 |
|  | 64 QAM | ≤ 4 | ≤ 5 |
|  | 256 QAM |  | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 6 |
|  | 16 QAM | ≤ 5 | ≤ 6 |
|  | 64 QAM | ≤ 5 | ≤ 6 |
|  | 256 QAM |  |  |

NS_18 A-MPR

A-MPR was agreed to be analyzed for NS_18 for 40 MHz channel bandwidth. A simulation campaign was done assuming

- PA calibrated to 22 dBm at antenna port with DFT-s-OFDM 100 RB and 1 dB MPR just meeting ACLR

- IQ-image and DC-leakage of 28 dBc

- CIM3 of -60 dBc

The results are summarized in tables 5 to 8. Based on the simulations nearly all of the same A-MPR values can be used for 40 MHz as simulated earlier for 30 MHz and narrower CBW. Slight adjustment to region A5 values has been done for PC2. In addition, new A-MPR region definitions are needed for 40 MHz. The proposed regions apply for both PC3 and PC2.

Table 3: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_18 | 6.5.3.3.3 | n28, n83 | 5 |  | Table 7, A1 for PC3; Table 8, A1 for PC2 |
|  |  |  | 10, 15, 20 |  | Table 7, A2 for PC3; Table 8, A2 for PC2 |
|  |  |  | 25, 30, 40 |  | Table 7, A3, A4, A5 for PC3; Table 8, A3, A4, A5, A6 for PC2 |

Table 4: Band n28 25MHz, 30MHz and 40 MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 715.5 – 730.54 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 30 | 718 – 728.04 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |
| 40 | 723 – 723.04 | >(LCRB*12*SCS)/2+8.46 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+8.46 | ≥5.4 | A4 |
|  |  | ≤11.16 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+8.46 ≤ (LCRB*12*SCS)/2+10.8 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥5.4 | A6 |

The A-MPR values for PC3 and PC2 are defined as in the table below.

Table 5: A-MPR for NS_18 (Power Class 3)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |

Table 6: A-MPR for NS_18 (Power Class 2)

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | N/A | ≤ 8 | 3.5 | 10 | 5 | 2 |
|  | QPSK | ≤ 3.5 |  | ≤ 8 | 3.5 | 10 | 5 | 2 |
|  | 16 QAM | ≤ 4 |  | ≤ 9 | 3.5 | 11 | 5 | 2 |
|  | 64 QAM | ≤ 4.5 |  | ≤ 10 | 3.5 | 11 | 6.5 | 2.5 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 11 | 3.5 | 11 | 6.5 |  |
| CP-OFDM | QPSK | ≤ 5.5 |  | ≤ 9.5 | 5 | 11.5 | 7 | 4 |
|  | 16 QAM | ≤ 5.5 |  | ≤ 10 | 5 | 11.5 | 7 | 4 |
|  | 64 QAM | ≤ 6 |  | ≤ 11.5 | 5 | 11.5 | 7 | 4 |
|  | 256 QAM | ≤ 9 |  | ≤ 11.5 | 5 | 11.5 | 7.5 |  |

Proposal 3: Adopt A-MPR for NS_18 for both PC3 and PC2 as shown in Table 3 to Table 6.

UE-to-UE coexistence requirements

There are multiple stringent UE-to-UE coexistence requirements for n28, as shown in Table 7.

Table 7: n28 UE coexistence requirements

| n28, n83 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |

Meeting these requirements without any A-MPR is more difficult with wider channel bandwidth, as the emission region is relative terms closer to the transmission bandwidth when the channel bandwidth gets wider. It would be necessary to thoroughly confirm these emission requirements can be met without A-MPR, and therefore we propose to continue the evaluation of these requirements until RAN4#113.

Proposal 4: Confirm until RAN4#113 whether UE-to-UE co-existence requirements can be met with 40 MHz CBW without A-MPR.

PC2 RSD

In previous meeting it was agreed to define NS_18 A-MPR for 40 MHz CBW.

In previous meeting we provided our evaluation on the PC3 RSD. As shown in [2] the average of all companies contributions resulted to -66.4 dBm refsens for 40 MHz bandwidth, using 25RB UL allocation for 15 kHz SCS and 10 RB UL allocation for 30 kHz SCS. We have now evaluated the PC2 RSD assuming PC3 refsens is the average from previous meeting. The result is shown in Table

Proposal 5: Adopt RSD for 40 MHz channel bandwidth as in Table 8 and Table 9.

Table 8: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for single Tx

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 0.6 | 0.6 | 0.7 | 0.8 | 1.3 | 2.4 | 2.9 |  | 3.0 |  |  |

Table 9: Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for dual Tx

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n28 | 1.1 | 1.1 | 1.1 | 1.3 | 3.0 | 6.6 | 7.9 |  | 7.9 |  |  |

Delta-MPR

One of the FFS points from previous meeting is delta-MPR. Delta-MPR helps to account for variations in frequency response over the channel bandwidth, when the channel bandwidth is wide compared to the center frequency. Based on our evaluation, these frequency variations are similar for 30 and 40 MHz channel bandwidths, and therefore the same 0.5 dB delta-MPR can be applied both for 30 MHz and 40 MHz channel bandwidths, for both PC3 and PC2.

Proposal 6: Delta-MPR of 0.5 dB is sufficient for 40 MHz channel bandwidth both for PC3 and PC2.

Center-frequency restrictions when UE supports 40 MHz CBW

Current gNB specifications already support 40 MHz CBW at the special raster point of 723.04 MHz in uplink. This point enables using 30 MHz CBW RB aligned within 40 MHz CBW and meeting minimum guard band, given that 30 MHz CBW minimum guard band is 40 kHz wider than 40 MHz CBW minimum guard band. It would be reasonable that also UE can be configured to the same center frequency to keep the smooth co-existence of different channel bandwidths.

If some deployments use only 40 MHz channel bandwidths, it would be possible to also place 40 MHz channel bandwidth immediately adjacent to low band edge. Given that the enhanced channel raster with 10 kHz channel step size is mandatory for n28 starting from rel-18, it would be possible to enable 40 MHz CBW channel raster points from 723.00 to 723.04 MHz.

Proposal 7: Enable channel raster points for 40 MHz channel bandwidth from 723.00 MHz to 723.04 MHz.

When UE supports 40 MHz channel bandwidth, it has an RF filter with at least 40 MHz passband. Therefore, narrower channel bandwidths can be flexible positioned within this passband, as they do not have the restriction from 2x30 MHz duplexer. To enable more flexibility to network operations, it is proposed to enable flexibility to narrower channel bandwidths within the 40 MHz channel bandwidth.

Currently NS_18 A-MPR is restricted to be valid within the lower 30 MHz duplexer frequencies only. The A-MPR applicability needs to be extended to cover also the newly applicable center frequencies.

Proposal 8: For UEs supporting 40 MHz channel bandwidth, narrower channel bandwidths can be freely placed within the 40 MHz channel bandwidth as long as they are fully contained within 703 to 743.04 MHz.

Proposal 9: NS_18 A-MPR is applicable whenever channel bandwidth fully is contained within 703 to 743.04 MHz

## 3	Conclusion

In this contributions RF requirements for PC2 operation and 40 MHz CBW for n28 were discussed. Following observations and proposals were made:

Proposal 1: Specify a single set of A-MPR and RSD values independent of implemented filter solution.

Proposal 2: Adopt the A-MPR for NS_17 as in Table 1 and Table 2 for n28 PC2

Proposal 3: Adopt A-MPR for NS_18 for both PC3 and PC2 as shown in Table 3 to Table 6.

Proposal 4: Confirm until RAN4#113 whether UE-to-UE co-existence requirements can be met with 40 MHz CBW without A-MPR.

Proposal 5: Adopt RSD for 40 MHz channel bandwidth as in Table 8 and Table 9.

Proposal 6: Delta-MPR of 0.5 dB is sufficient for 40 MHz channel bandwidth both for PC3 and PC2.

Proposal 7: Enable channel raster points for 40 MHz channel bandwidth from 723.00 MHz to 723.04 MHz.

Proposal 8: For UEs supporting 40 MHz channel bandwidth, narrower channel bandwidths can be freely placed within the 40 MHz channel bandwidth as long as they are fully contained within 703 to 743.04 MHz.

Proposal 9: NS_18 A-MPR is applicable whenever channel bandwidth fully is contained within 703 to 743.04 MHz

## 4	References

[1]  RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC CBN

[2]  R4-2414274, WF on introduction of PC2 and 40 MHz in NR band n28