3GPP TSG-RAN WG4 Meeting #112bis	R4-2416459

# Hefei, China, 14 – 18 October 2024

Agenda item:		5.17.2

Source: 		Qualcomm Inc.

Title: 		Co-existence considerations for the new S-band

Document for:		Discussion

## 1	Introduction

The WI on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) can be found in [1]. The objectives include co-existence analysis, for example UE-UE co-existence, for adjacent band per RAN4 scope. In this contribution these co-existence aspects are discussed.

## 2	Discussion

In previous meeting it was agreed to study UE-to-UE coexistence between n252 UL and n25/n2 DL more in detail. The origins of the -50 dBm/MHz limit can be tracked back to 2008 [2]. The analysis back then was largely based on approximately 1m separation between UEs and assumptions on human body and antenna losses. For terrestrial networks, this scenario can realize e.g. in indoor environments or in a bus where multiple people are using data intensive applications simultaneously. Given the penetration loss of walls, it is also possible that indoors UEs both would need to use high transmit power to reach the gNB outdoors, as well as operate near sensitivity.

For NTN outdoor environment is the typical condition. One could also expect outdoor rural conditions to be more appropriate, as one would expect urban environments to have TN coverage always available. As rural environment is very different from indoor environment, we did evalutions on how different UE-to-UE distances and what would need to be the emission level at UE antenna to provide same protection as -50 dBm/MHz requirement does for 1m separation. We looked at different distances and two different propagation (path loss) models, i.e. free space and urban micro. Urban micro was included as TR 38.863 has documented it to be used for UE-to-UE interference in Ka-band studies.

The summary of the results is provided in Table 1.

Table 1: impact of UE-to-UE separation distance to required UE-to-UE protection limit

2GHz center frequency was used in the path loss calculation. It can be seen that with the assumed parameters 1m separation distance sets the emission power to -49.7 dBm/MHz. At 10 m distance there is 20 dB more power allowed assuming free space propagation. Then every doubling of the distance allows 6 dB more emissions. For example, if 20 m would be used in deriving the coex-limit, -24 dBm/MHz could be used as the limit.

Observation 1: If UE-to-UE coexistence limit is based purely on distance, moving the assumption from 1m to 10m would mean 20 dB relaxation to the emission limit assuming free space propagation between UEs. Moving from 1m to 20m would mean 26 dB relaxation.

Secondly, we looked at the propability that two randomly dropped UEs are close to each other. For this purpose, we simulated dropping UEs from two operators to an area of 1 sqkm. We assumed that if aggressor UE is dropped less than 20m apart from a victim UE, interference is possible. This assumption is pessimistic, as a victim UE close to cell center could still perform well given the DL signal is strong. We did not this into account, but considered all cases where UE-to-UE distance is less than 20m to cause interference. An example of one drop with 27 aggressor UEs and 9 victim UEs is shown in Figure 1.

Figure 1: Example of UE positions within 1 sqkm in one drop

Several tens of thousands drops with different UE densities were done. The results are summarized in Table 2, where interference probability is calculated as follows:

Interference probability= Number of victim UEs separated less than 20m from aggressor UENumber of all victim operator UEs*100%

Table 2: Probability of UE-to-UE interference vs. number of UEs

| case # | # of aggressor | # of victim | interference probability |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 0.12% |
| 2 | 1 | 27 | 0.12% |
| 3 | 3 | 1 | 0.38% |
| 4 | 3 | 9 | 0.36% |
| 5 | 3 | 27 | 0.36% |
| 6 | 3 | 78 | 0.37% |
| 7 | 9 | 1 | 1.04% |
| 8 | 9 | 3 | 1.10% |
| 9 | 9 | 9 | 1.09% |
| 10 | 9 | 27 | 1.10% |

In this table number of aggressor UEs represents the UEs that are simultaneous scheduled in NTN UL within 1 sqkm. Correspondingly, the number of victim UEs represent the number of TN UEs that are simultaneously scheduled in TN DL.

It can be seen that the interference probability in this evaluation is linearly dependent on the number of aggressor UEs. This can be explained so that number of aggressor UEs define the area within which the interference may occur. Given the uniformly random UE positions, the propability turns out to correspond to ratio between the “interference area” and total area.

Therefore, one relevant question could be how many aggressors there would be in a given area. TR 38.863 gives on example of LEO600 antenna pattern, was has 4.4 degree 3 dB beamwidth. Assuming the 3 dB beamwidth sets the cell size, this cell would have 46km cell radius and cover 6648 sqkm of area. For a 20 MHz carrier there are 106 resource blocks available. Rel-19 enhancements on UL capacity consider scheduling 4 UEs in a single resource block. This would mean that 4*106 = 424 UEs could theoretically be scheduled at the same time. The resulting UE density of transmitting UEs is 0.0638 UEs per sqkm. If the interference probabilities for Table 2 are scaled to this UE density, interference probability becomes only 0.008%

This analysis assumes that UE locations are uniformly distributed over the area being analyzed. The deterministic worst case where UEs are right next to each other is does take place in some of the drops. However, the analysis does not cover cases where only a single active UE in both networks and these UEs are right next to each other, or that larger number of UEs would be concentrated in hotspots.

Observation 2: Assuming UE locations are uniformly distributed within an area, the likelihood that active NTN UL transmission causes interference to TN DL is very low given the satellite beam serves a very large area.

## 3	Conclusion

In this contribution co-existence between new NTN S-band and adjacent 3GPP bands was discussed. Following observations were made.

Observation 1: If UE-to-UE coexistence limit is based purely on distance, moving the assumption from 1m to 10m would mean 20 dB relaxation to the emission limit assuming free space propagation between UEs. Moving from 1m to 20m would mean 26 dB relaxation.

Observation 2: Assuming UE locations are uniformly distributed within an area, the likelihood that active NTN UL transmission causes interference to TN DL is very low given the satellite beam serves a very large area.

## 4	References

- RP-241885, Revised WID on Introduction of another NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), Qualcomm Incorporated

- R4-080710, TS36.101: TP for UE Spurious emission limits, Motorola