3GPP TSG-RAN WG4 Meeting #112bis	R4-2416463

# Hefei, China, 14 – 18 August 2024

Agenda item:		6.26.2.2

Source: 		Qualcomm Inc.

Title: 		NR NTN UL capacity enhancements

Document for:		Approval

## 1	Introduction

Work item for the third phase of NR NTN has been approved in [1]. The work item includes RAN4 for multiple objectives, including UL capacity/cell throughput enhancement for FR1-NTN, with a sub-objective to update RF requirements accordingly, if needed. In this contribution RAN4 RF impact is discussion

## 2	Discussion

RAN1 has already been working on the work item for a few meetings and reached initial agreements on the scope of the capacity enhancements. Relevant agreements from RAN1#117 have been reproduced below:

Agreement

For the normative phase, at least one of the OCC techniques will be specified:

- Inter-slot time-domain OCC with PUSCH repetition Type A with OCC length 2 or 4

- Inter-symbol(s) time domain OCC with OCC length 2 or 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 or 4

- FFS Combination of OCC techniques including multiplexing of 8 UEs

- FFS Use of OCC techniques with TBoMS

- FFS Backward compatibility with non-Rel-19 UEs

RAN1 made further agreements in RAN1#118

Agreement

At least one of the OCC techniques when PUSCH repetitions are used will be specified:

- Inter-slot time-domain OCC with OCC length 2

- Inter-slot time-domain OCC with OCC length 2 and 4

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2

- Intra-symbol pre-DFT-s OCC (comb-like structure as in PUCCH format 4) with OCC length 2 and 4

- Note: combination of techniques is not precluded

- PUSCH repetition Type B is not considered

As such inter-symbol OCC is no longer considered and RAN4 agreements reached in previous meeting are no longer meaningful.

Observation 1: Inter-symbol OCC is no longer considered in RAN1

In previous meeting one company provided input that UE would need to maintain a fixed CFO during OCC transmission for the inter-slot scheme to guarantee performance [2]. Firstly, the performance loss for the inter-slot OCC scheme has been recognized in RAN1, while similar performance loss is not present with the intra-symbol pre-DFT-s OCC. It is not yet clear if RAN1 will end specifying a scheme with performance loss when it seems better options are available.

Fixing UE CFO error for the OCC transmission would be extremely difficult. UE LO frequency is fundamentally dependent on the crystal oscillator (XO) frequency and the stability of the XO. The issue is that XO frequency is not stable. Multiple datasheets can be found showing frequency accuracies ranging e.g. from +/- 20ppm to +/- 40ppm, one example is [3]. The variability within the specified range is dependent e.g. on temperature. This means that e.g. active UL transmissions impacting the UE internal temperature will impact the LO frequency.

UE does frequent frequency correction updates to ensure the output frequency stays within the allowed +/-0.1 ppm frequency error. The timescales between consecutive corrections may occur between few milliseconds in some conditions. That means that on top of drift, there are “jumps” in the frequency error. On top of this, for NTN applications UE needs to do frequency pre-compensation. This adds on the uncertainty of the frequency error. Pausing the LO frequency corrections could lead to UE violating the frequency error specification.

Observation 2: Fixed frequency error within the OCC transmission is very difficult to guarantee. Pausing frequency corrections could lead to UE failing the +/- 0.1ppm frequency error requirement, while on-going frequency corrections can cause an abrupt jump in frequency error.

Observation 3: It cannot be assumed that frequency error at different time instants is the same.

Even if a theoretical perfect UE would be able to fix the frequency error for a short time period, gNB would need to be able to estimate the frequency errors and apply the estimation on UL scheduling, making sure that UEs with similar frequency error are paired together. Currently, there is no information in RAN4 how accurate the frequency error estimation is at gNB, how long it takes, and how quickly gNB can apply the estimation results into pairing UEs in UL scheduling.

Observation 4: More information is needed on gNB capabilities to estimate the frequency error and the delay on applying the estimation to UE pairing.

Based on the challenges and the lack of information of gNB capabilities, it is suggested to wait for further progress in RAN1 before further work on inter-slot OCC.

Proposal 1: Wait for further progress in RAN1 before further work on inter-slot OCC, if any.

## 3	Conclusion

In this contribution NR NTN UL capacity enhancements were discussed. Following observations and proposals were made

Observation 1: Inter-symbol OCC is no longer considered in RAN1

Observation 2: Fixed frequency error within the OCC transmission is very difficult to guarantee. Pausing frequency corrections could lead to UE failing the +/- 0.1ppm frequency error requirement, while on-going frequency corrections can cause an abrupt jump in frequency error.

Observation 3: It cannot be assumed that frequency error at different time instants is the same.

Observation 4: More information is needed on gNB capabilities to estimate the frequency error and the delay on applying the estimation to UE pairing.

Proposal 1: Wait for further progress in RAN1 before further work on inter-slot OCC, if any.

## 4	References

[1]  RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

[2]  R4-2412981, Other NTN UE RF impact, Ericsson

[3]  Specification of a crustal unit, Murata, available https://www.murata.com/products/productdata/8801079590942/SPEC-XRCGB32M000F3M00R0.pdf?1541043011000