3GPP TSG-RAN WG4 Meeting #112bis	R4-2416464

# Hefei, China, 14 – 18 August 2024

Agenda item:		6.27.2

Source: 		Qualcomm Inc.

Title: 		IoT NTN UL capacity enhancements

Document for:		Approval

## 1	Introduction

Work item for the third phase of NTN IoT has been revised in RAN#105 in [1]. The work item includes RAN4 for the objective to support capacity enhancements for uplink, with a sub-objective to update RF requirements accordingly, if needed. In this contribution RAN4 RF impact is discussion

## 2	Discussion

When the work item description is examined in detail, it can be observed that UE RF specification is not included in the list of impacted specifications. Therefore, it is not fully clear whether the discussion should be fully focused on potential impact on SAN, or also UE RF is considered. SAN RF specification 36.108 is listed in the WI, but only for performance part. In this contribution we have considered UE RF.

Observation 1: UE RF specification 36.102 is not listed as an impacted specification in the WID, neither for core nor performance part

RAN1 has already been working on the work item for a few meetings and reached initial agreements on the scope of the capacity enhancements. Relevant agreements from RAN1#117 have been reproduced below:

Agreement

For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

In RAN1#118, the following was agreed:

Agreement

RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity

·         UL gaps for synchronization (from Rel-13)

·         Gaps around NPRACH occasions

·         UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

·         TDM DMRS that are muted

·         Guard periods for 3.75kHz UL transmissions

Agreement

The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1:

·         Option 1: OCC2, Symbol-level, TDM DMRS

·         Option 2: OCC2, Symbol-level, CDM DMRS with new pattern

·         Option 3: OCC2, Slot-level, TDM DMRS

·         Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

·         Option 6: OCC4, Symbol-level, CDM DMRS with new pattern

The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1:

·         Option 1: OCC2, Symbol-level, TDM DMRS

·         Option 3: OCC2, Slot-level, TDM DMRS

·         Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

·         Option 5: OCC4, Symbol-level, TDM DMRS

·         Option 7: OCC4, Slot -level, TDM DMRS

·         Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern

Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS.

Note 2: Companies to report DMRS sequence applied.

Agreement

For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

Observation 2: Both inter-symbol and inter-slot OCC are being studied in RAN1

In previous meeting one company provided input that UE would need to maintain a fixed CFO during OCC transmission for the inter-slot scheme to guarantee performance [2]. Firstly, the performance loss for the inter-slot OCC scheme has been recognized in RAN1, while similar performance loss is not present with the intra-symbol OCC. It is not yet clear if RAN1 will end specifying a scheme with performance loss when it seems better options are available.

Fixing UE CFO error for the OCC transmission would be extremely difficult. UE LO frequency is fundamentally dependent on the crystal oscillator (XO) frequency and the stability of the XO. The issue is that XO frequency is not stable. Multiple datasheets can be found showing frequency accuracies ranging e.g. from +/- 20ppm to +/- 40ppm, one example is [3]. The variability within the specified range is dependent e.g. on temperature. This means that e.g. active UL transmissions impacting the UE internal temperature will impact the LO frequency.

UE does frequent frequency correction updates to ensure the output frequency stays within the allowed +/-0.1 ppm frequency error. The timescales between consecutive corrections may occur between few milliseconds in some conditions. That means that on top of drift, there are “jumps” in the frequency error. On top of this, for NTN applications UE needs to do frequency pre-compensation. This adds on the uncertainty of the frequency error. Pausing the LO frequency corrections could lead to UE violating the frequency error specification.

Observation 3: Fixed frequency error within the OCC transmission is very difficult to guarantee. Pausing frequency corrections could lead to UE failing the +/- 0.1ppm frequency error requirement, while on-going frequency corrections can cause an abrupt jump in frequency error.

Observation 4: It cannot be assumed that frequency error at different time instants is the same.

Even if a theoretical perfect UE would be able to fix the frequency error for a short time period, gNB would need to be able to estimate the frequency errors and apply the estimation on UL scheduling, making sure that UEs with similar frequency error are paired together. Currently, there is no information in RAN4 how accurate the frequency error estimation is at gNB, how long it takes, and how quickly gNB can apply the estimation results into pairing UEs in UL scheduling.

Observation 5: More information is needed on gNB capabilities to estimate the frequency error and the delay on applying the estimation to UE pairing.

Based on the challenges and the lack of information of gNB capabilities, it is suggested to wait for further progress in RAN1 before further work on inter-slot OCC.

Proposal 1: Wait for further progress in RAN1 before further work on inter-slot OCC, if any.

## 3	Conclusion

In this contribution NTN IoT UL capacity enhancements were discussed. Following observations and proposals were made

Observation 1: UE RF specification 36.102 is not listed as an impacted specification in the WID, neither for core nor performance part

Observation 2: Both inter-symbol and inter-slot OCC are being studied in RAN1

Observation 3: Fixed frequency error within the OCC transmission is very difficult to guarantee. Pausing frequency corrections could lead to UE failing the +/- 0.1ppm frequency error requirement, while on-going frequency corrections can cause an abrupt jump in frequency error.

Observation 4: It cannot be assumed that frequency error at different time instants is the same.

Observation 5: More information is needed on gNB capabilities to estimate the frequency error and the delay on applying the estimation to UE pairing.

Proposal 1: Wait for further progress in RAN1 before further work on inter-slot OCC, if any.

## 4	References

[1]  	RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)

[2]	R4-2412992, IoT NTN UE RF impact, Ericsson